Roof Report on MSFD Common Indicators for Bulgaria and Romania (18. February 2017)

Disclaimer

This draft of a Bulgarian and Romanian joint "Roof Report" is based on the draft outline presented in the Support Project´s Inception Report as well as on a revised outline discussed at the CBE1-CBE4-workshops in Varna and Constanta; it has evolved as a result of insights from an in-depth assessment of reporting requirements and technical information, ongoing work in other Regional Sea Conventions (OSPAR; HELCOM, UNEP/MAP) and current developments in BG and RO.

The present version takes into account the state of MSFD implementation and current work on the 2018 update in Bulgaria and Romania.

A "mature" Roof Report will be delivered at the end of the project (planned for February 2017). To meet the reporting requirements, it will not need to be finalised by Bulgaria and Romania until July 2018, but it has been prepared in a way to guarantee maximum possible usability by Romania and Bulgaria to fulfil their reporting (Article 17 MSFD) and public participation and data provision (Article 19 MSFD) requirements (i.e. data format used and its integration into WISE-Marine and the information exchange platform).

# Content

Content	2

Introduction and Overview	4

Aims and Objectives of the Roof Report	7

Progress achieved since the first implementation cycle and how COM Art. 12 recommendations have been considered	8

Assessment of the first MSFD implementation cycle and Art. 12 recommendations by the COM	8

Actions taken by BG and RO based on COM Art. 12 regional and technical assessment reports	9

Latest developments	11

The Black Sea Marine Region 	13

Human activities, predominant pressures and impacts on the marine environment	17

Physical restructuring of rivers, coastline or seabed (water management)	21

Canalisation and other watercourse modifications	21

Coastal defense and flood protection	22

Extraction of non-living resources	24

Extraction of oil and gas, including infrastructure	24

Extraction of living resources	25

Fish and shellfish harvesting (professional, recreational)	25

Fish and shellfish processing	27

Cultivation of living resources	28

Marine Aquaculture	28

Agriculture	30

Forestry	31

Transport	32

Shipping	32

Ports	33

Urban and industrial uses	35

Urban uses and Industrial uses	35

Waste treatment and disposal	37

Inflow from Rivers	39

Tourism and leisure	41

Tourism and leisure infrastructure	42

Tourism and leisure activities	42

Military operations	43

Assessment Areas and Common Indicators	44

Assessment areas	45

Descriptor 2: Non-indigenous Species (NIS)	46

Descriptor 3: Commercially exploited fish and shellfish	48

Descriptor 5: Eutrophication	50

Descriptor 6: Seafloor integrity (physical loss and disturbance)	53

Descriptor 7: Hydrographical changes	53

Descriptor 8: Contaminants	54

Descriptor 9: Contaminants in seafood	56

Descriptor 10: Marine litter	56

Descriptor 11: Underwater noise	58

Descriptors 1, 4, 6: Biodiversity and ecosystems	59

Article 8.1c and the analysis of the Cost of Degradation	64

Key conclusions and outlook 	65

Competent authorities	66

Bulgaria	66

Romania	67

Annex I: Indicator Factsheets	72

# Introduction and Overview

The Marine Strategy Framework Directive (2008/56/EC) (MSFD) is implemented in six-year cycles. The forthcoming reporting requirements in 2018 require Member States (MS) to update the initial assessments of their marine waters. The Commission (COM) has highlighted the need for increased regional cooperation, coordination and harmonisation in the implementation of the MSFD (Art. 6). In response, regional "roof reports" are being prepared that provide information on pressures, impacts and state of the marine environment at a regional level. These can be used by Member States to fulfil some of the reporting requirements for MSFD, complemented by national assessments where needed.

For the Black Sea marine region, Bulgaria (BG) and Romania (RO) have agreed to prepare this regional Roof Report that provides coordinated assessments of commonly-agreed indicators between both Member States in a harmonized way ("Part A" of the reporting). Aspects not addressed in the Roof Report will be prepared and reported nationally ("Part B" of the reporting), which will cover indicators that are not (yet) harmonised between the two countries.

The current document has been prepared under Phase III of the Support Contract ENV.D2/FRA/2012/0017 (project “Technical and administrative support for the joint implementation of the Marine Strategy Framework Directive (MSFD) by the EU Black Sea Member States - Phase III”). This follows and builds on Phases I and II that were implemented in previous years: Phase I, which focused on monitoring programmes (finalised in January 2015) and Phase II, which aimed to develop coordinated or joint Programmes of Measures (finalised February 2016).

The process to revise Annex III of the MSFD and the Commission Decision on criteria and methodological standards on good environmental status (GES) (2010/477/EU) resulted in versions by the MSFD Regulatory Committee on 10th November 2016. These documents are considered draft as they still need to go through public consultation and the Council and Parliament. These approved draft versions of Annex III and the Commission Decision are used as the basis for the criteria, activities, pressures and impacts in this Roof Report.

The following table provides an overview, per criterion, of the aspects addressed in the Roof Report, and the aspects to be addressed at national level.

Table 1: Overview of aspects addressed in the Roof Report, and what will remain to be addressed at national level.

|  | Addressed in its entirety in the Roof Report |  |  | Addressed partially in Roof Report (i.e. a common indicator is included, but it does not address all aspects of the criterion) |  |  | Addressed at national level |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Pressure Descriptors |  |  |  |  |  |  |  |  |  |
| D2C1 |  |  |  |  |  |  |  |  |  |
| D2C2 |  |  |  |  |  |  |  |  |  |
| D2C3 |  |  |  |  |  |  |  |  |  |
| D3C1 |  |  |  |  |  |  |  |  |  |
| D3C2 |  |  |  |  |  |  |  |  |  |
| D3C3 |  |  |  |  |  |  |  |  |  |
| D5C1 |  |  |  |  |  |  |  |  |  |
| D5C2 |  |  |  |  |  |  |  |  |  |
| D5C3 |  |  |  |  |  |  |  |  |  |
| D5C4 |  |  |  |  |  |  |  |  |  |
| D5C5 | Not yet clarified whether D5C8 will be used instead in Bulgaria and Romania |  |  |  |  |  |  |  |  |
| D5C6 |  |  |  |  |  |  |  |  |  |
| D5C7 |  |  |  |  |  |  |  |  |  |
| D5C8 | Not yet clarified whether D5C8 will be used instead of C5 in Bulgaria and Romania |  |  |  |  |  |  |  |  |
| D6C1 |  |  |  |  |  |  |  |  |  |
| D6C2 |  |  |  |  |  |  |  |  |  |
| D6C3 |  |  |  |  |  |  |  |  |  |
| D7C1 |  |  |  |  |  |  |  |  |  |
| D7C2 |  |  |  |  |  |  |  |  |  |
| D8C1 |  |  |  |  |  |  |  |  |  |
| D8C2 |  |  |  |  |  |  |  |  |  |
| D8C3 |  |  |  |  |  |  |  |  |  |
| D8C4 |  |  |  |  |  |  |  |  |  |
| D9C1 |  |  |  |  |  |  |  |  |  |
| D10C1 |  |  |  |  |  |  |  |  |  |
| D10C2 |  |  |  |  |  |  |  |  |  |
| D10C3 |  |  |  |  |  |  |  |  |  |
| D10C4 |  |  |  |  |  |  |  |  |  |
| D11C1 |  |  |  |  |  |  |  |  |  |
| D11C2 |  |  |  |  |  |  |  |  |  |
| Environmental Status Descriptors |  |  |  |  |  |  |  |  |  |
|  | M | B | F | M | B | F | M | B | F |
| D1C1 |  |  |  |  |  |  |  |  |  |
| D1C2 |  |  |  |  |  |  |  |  |  |
| D1C3 |  |  |  |  |  |  |  |  |  |
| D1C4 |  |  |  |  |  |  |  |  |  |
| D1C5 |  |  |  |  |  |  |  |  |  |
| D1C6 |  |  |  |  |  |  |  |  |  |
| D6C4 |  |  |  |  |  |  |  |  |  |
| D6C5 |  |  |  |  |  |  |  |  |  |
| D4C1 |  |  |  |  |  |  |  |  |  |
| D4C2 |  |  |  |  |  |  |  |  |  |
| D4C3 |  |  |  |  |  |  |  |  |  |
| D4C4 |  |  |  |  |  |  |  |  |  |

M= mammals, B= Birds, F= Fish

The Roof Report uses an "indicator-based" approach. The focus is on common indicators that are harmonised between BG and RO; assessments of individual common indicators are provided in "Indicator Factsheets" (see Annex I), which have been prepared in accordance with Guidance Document No 13. The overall status of individual descriptors is not assessed, but comment is provided on what the available indicators show.

It has been possible to complete draft status assessments on a number of common indicators as of February/March 2017 (D3, D5, D8 and D9). The assessment of the remaining common indicators will need to be completed by Bulgaria and Romania by July 2018.

The Roof Report is open for other Black Sea countries to contribute. Although its main focus is the presentation of Romanian and Bulgarian common indicators to fulfill the 2018 MSFD reporting requirements, it should relate to the work planned and ongoing within the Black Sea Commission (e.g. BSIMAP, Black Sea SAP and planned Marine Litter Regional Action Plan).

Hence, other BSC Member States are invited to present own national methodologies towards defining indicators, targets/objectives and monitoring approaches for protecting their marine waters.

# Aims and Objectives of the Roof Report

This Roof Report has a number of general aims and specific objectives. It will:

- contribute to the timely and adequate compliance with the reporting obligations of Article 17 of the MSFD in both countries, i.e. the updating of MSFD reports on Articles 8, 9 and 10. The challenge is hereby to ensure that the reporting of Bulgaria and Romania due in July 2018 is undertaken in a coordinated, regionally coherent and consistent manner, thus meeting the requirements of Article 5(2) of the MSFD;

- facilitate the public consultation obligations of Article 19 MSFD;

- demonstrate the harmonisation of approaches (such as GES definitions, indicator assessments for Art. 8 etc.) and timelines between Bulgarian and Romanian implementation of the MSFD on the one hand, and the related work planned and ongoing within the Black Sea Commission on the other hand (e.g. Black Sea Strategic Action Plan (SAP) and planned Marine Litter Regional Action Plan);

- foster cooperation with third countries (Art. 6 MSFD), as envisaged by the Marine Directors in May 2015 in Riga; and

- have the potential to serve as an important platform to foster the work on coordinated activities in the frame of the National Action Plan of Bulgaria for overcoming gaps and shortcomings identified in reporting on Article 8, 9 and 10 (section of bilateral coordination within the Black Sea marine region) and the official letter sent by Romania to the COM on 25.02.2015, which can be considered the National Action Plan of Romania (section on regional cooperation).

In order to achieve these aims, the Roof Report has the following specific objectives:

- To focus on harmonised (i.e. coordinated, regionally coherent and consistent) elements of MSFD assessments, and therefore of the reporting.

- To provide an overview of progress achieved since the first implementation cycle of the MSFD and how the Article 12 recommendations have been addressed.

- To invite other Black Sea Commission Member States to also present own national methodologies towards defining indicators, targets/objectives and monitoring approaches for protecting their marine waters.

- To compile information on human activities, pressures and impacts in the two countries’ marine waters.

- To present assessments of common indicators agreed between Bulgaria and Romania. The indicators not covered by this Roof Report are not yet defined or harmonised between the two countries, and will be covered in the national part of the 2018 reporting.

## Progress achieved since the first implementation cycle and how COM Art. 12 recommendations have been considered

The present section is a short summary of how the content of the MSFD Articles 8, 9 and 10 evolved from 2012 to 2016 in Bulgaria and Romania. It briefly documents changes between the information reported in 2012 and the status quo in 2016, including the definitions of GES and targets used in the Art. 13 Programmes of Measures (PoM) reports and a short overview of Art. 11 monitoring programmes. Also, results from Phases I and II of the EC support project are considered.

### Assessment of the first MSFD implementation cycle and Art. 12 recommendations by the COM

In 2014, the Commission issued its Art. 12 report on the first phase of MSFD implementation, which considered the MS’ achievements and shortcomings towards MSFD obligations. The report urged the MS to significantly improve the quality and coherence of their Initial Assessments and determinations of GES and environmental targets as soon as possible and by 2018 at the latest, in order to ensure that the second round of implementation yields greater benefits. The overall level of coherence between GES, targets and indicators in BG and RO was rated as very low (only in the case of Descriptor 5 on Eutrophication the coherence was assessed as moderate). Moreover, it was observed that both countries had made very limited reference to the work of the Black Sea Commission and to relevant standards from the Black Sea region. In general, both BG and RO did not consider their work within the Bucharest Convention as being relevant for the achievement of their obligations under MSFD but MS informed BSC and Contracting Parties about the progress in the implementation of MSFD.

As a result of the above-mentioned assessments, the COM identified key shortcomings and issued recommendations for improvements to Romania and Bulgaria on national as well as regional levels. It specifically asked Bulgaria to address knowledge gaps identified in the Initial Assessment, i.e. through the MSFD monitoring programme and research programmes, focusing on those descriptors considered as inadequate or partially adequate. Romania was requested to: (a) further develop its approaches to assessing (quantifying) impacts from the main pressures to lead to improved and more conclusive assessment results for 2018; (b) identify knowledge and information gaps and address these, i.e. through the MSFD monitoring programme and research programmes, focusing on those descriptors considered as inadequate or partially adequate; and (c) address the scope of marine waters, as defined in the Directive, through exclusion of Water Framework Directive (WFD) Transitional Waters (which was not possible, as Transitional Waters are defined as Marine Waters). Several of the identified shortcomings applied equally to both countries and resulted in similar general recommendations, such as to strengthen the GES definition of the biodiversity descriptors, to ensure that targets are SMART and cover all relevant pressures, to improve the consistency between the criteria used in GES, the assessment of the impact and the proposed targets etc. Clear and comprehensive recommendations were also issued regarding increased regional cooperation of the Commission and Member States with Contracting Parties to the Bucharest Convention and other Regional Sea Conventions.

### Actions taken by BG and RO based on COM Art. 12 regional and technical assessment reports

During a regional meeting between the COM and RO and BG on 26-27th March 2014, Bulgaria and Romania committed themselves to revising and together developing the specific definitions of GES, environmental targets and related indicators for their marine waters, taking into account the assessment and the recommendations of the COM. In this context, they committed to jointly prepare a respective Action Plan. In practice, however, the countries prepared individual Action Plans.

In the National Action Plan of Bulgaria activities are grouped in short-term (done in 2014), mid-term (2015-2017) and long-term activities (2017-2018). Completed and mid-term activities mainly include results from externally funded projects, whereas long-term activities shall mainly focus on ensuring sustainable national and international funding and enhancing public participation in the MSFD implementation process. Also, joint activities between Bulgaria and Romania are presented, which are however so far limited to the Phases I and II of the EU support project. Finally, some additional actions in the framework of the BSC are also listed.

The Action Plan of Romania is more general, but also contains some information on progress achieved in 2013-2014. The most important points are:

- setting GES values for missing descriptors thanks to the introduction of new data sets;

- setting environmental targets for Descriptors 2, 6, 7, 9, 10, 11; and

- improvements in the next cycle of MSFD monitoring

The document also emphasises improvements in the cooperation between RO and BG achieved due to several projects (e.g. MISIS, EU support project Phases I and II etc.).

In the scope of the project “Technical and administrative support for the joint implementation of the Marine Strategy Framework Directive (MSFD) by the EU Black Sea Member States - Phase I, GES definitions and targets were revised in both countries, however with the reservation that the definitions could not be considered as final and that there was a need to establish/revise GES and target definitions as soon as more data become available through future monitoring programmes.

For the drafting of the monitoring programs and PoMs, Romania and Bulgaria agreed to use the common GES and target definitions, as defined in the respective phase of the support project and as reported to the COM.

In Phase II of the EC support project, GES definitions and targets have not been revised. The project was dealing with development of transboundary joint or coordinated measures between Bulgaria and Romania. There were 17 designated measures planned as a part of national PoM of both countries, which even then were not with official character. Finally:

- For Bulgaria: all 17 preliminary discussed and agreed as common, transboundary measures became a part of the BG national PoM. They have been further developed by the Black sea Basin Directorate (BSBD) and forwarded to the Ministry of Environment and Water. During the Public Consultations, some additional information has been added.

- For Romania:

++Bulgarian and Romanian Art. 11 reports on the establishment and implementation of coordinated monitoring programmes were assessed, coming to the conclusion that some changes regarding GES, targets and indicators were reported by both countries. Regarding the monitoring programmes and future work (on GES and targets), both countries estimated that the majority of their monitoring programmes will adequately cover the monitoring needs for the assessment of progress towards their GES definitions and targets by 2018. As for the coherence of BG and RO monitoring programmes and monitored parameters, they were assessed as being medium to high depending on the descriptor in question.

### Latest developments

The core objective and outcome of the comparative analysis of Bulgaria and Romania Article 8, 9 and 10 reports from 2012, undertaken by the Phase III support project, was the clarification of status quo definitions of GES, targets and indicators, i.e. taking into account the current work in Bulgaria and Romania. As a result, a detailed overview sheet was produced on GES, targets and (common) indicators and/or parameters in Bulgaria and Romania plus smaller, more user-friendly Descriptor-specific sheets. The outcomes depict both the development of GES, target and indicator definitions from 2012 to 2016 and the definitions updated to the present status (based on the "old" COM Decision 477/2010). Additionally, for reasons of better comparability, an updated summary of GES, target and indicator definitions was developed at the beginning of Phase III in the same format as applied in Phases I and II (Table 2 below).

Table 2: Analysis of GES and targets in BG and RO under the MSFD support project (beginning of Phase III in spring 2016, i.e. based on the "old" GES Decision 477/2010/EU)

|  | Bulgaria |  | Romania |  | Convergence Analysis |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | GES defined | Targets defined | GES defined | Targets defined | GES | Targets |
| D1-Fish | Yes | Yes | Yes | Yes | No | No |
| D1-Mammals | Yes | Yes | Yes | Yes | Yes | Yes |
| D1-Birds | Yes | Yes | Yes | Yes | Yes | Yes |
| D1Seabed Habitats | Yes | Yes | Yes | Yes | Yes | Yes |
| D1-Water Column Habitats | Yes | Yes | Yes | Yes | Yes | Yes |
| D2 | No | Yes | Yes | Yes | No | No |
| D3 | Yes | Yes | Yes | Yes | Yes | Yes |
| D4 | Yes | Yes | No | Yes | No | No |
| D5 | Yes | Yes | Yes | Yes | Yes | Yes |
| D6 | Yes | Yes | No | Yes | No | No |
| D7 | Yes | Yes | Yes | Yes | No | No |
| D8 | Yes | Yes | Yes | Yes | Yes | Yes |
| D9 | Yes | Yes | Yes | Yes | Yes | Yes |
| D10 | Yes | Yes | Yes | Yes | No | Yes |
| D11 | No | No | Yes | Yes | No | No |

Note: orange colour means that definitions of GES and targets were preliminary/work in progress.

In November 2016, the MSFD Regulatory Committee presented approved draft versions of a revised Annex III of the MSFD and the Commission Decision on criteria and methodological standards on good environmental status (GES) (2010/477/EU), after a long revision process. Both document significantly change the "order" of how important MSFD elements, e.g. criteria (and indicators), activities, pressures and impacts are structured. Hence GES, targets and indicators developed before the approved draft versions of Annex III and the Commission Decision were partly obsolete, and needed restructuring or entirely new definitions.

In the Roof Report, some of these elements are described, following an "indicator-based approach", i.e. by defining and describing common, harmonized indicators used in both countries, and associated threshold values (e.g. for GES). Where monitoring data was available, conclusions on status were derived.

Recently (28th December 2016), the Marine Strategy of Bulgaria, including the PoM, was adopted by the Council of Ministers; the Romanian PoM will probably be finalized by spring 2017. Also, several research projects (both funded by international donors, as well as nationally) to complement the monitoring programmes, define better (and quantified) GES definitions and updated Article 8 and status assessments are set to deliver results in early 2017.

# The Black Sea Marine Region

The Black Sea is a semi-enclosed marine water body. Its only connection to the (Atlantic) ocean is via the Mediterranean Sea — through the Bosporus Strait (31 km in length), the Sea of Marmara (internal sea) and the Dardanelles Strait (64 km in length). The Black Sea is bordered by six coastal states: Ukraine to the north, the Russian Federation to the north-east, Georgia to the east, Turkey to the south, and Bulgaria and Romania to the west (see Map 1 below). Currently prevailing socio-economic framework conditions in the region are marked by considerable differences in historic starting positions as well as different political and economic courses the Black Sea countries have pursued in recent years.

Map 1: Physiographic and political maps of the Black Sea region

The Black Sea is one of the largest regional seas in the world, with a surface area of 432,000 km2, total water volume of 547,000 km3 and an average depth of 1,315 meters. However, its catchment area of 2,000,000 km2 is about six times larger than its surface area, collecting the water from almost all the European countries, except the westernmost ones. The annual river inflow to the Black Sea is approximately 340.6 km3 and its salinity varies between 18–22‰.

Table 3: The Black Sea in figures

| Geographical Coordinates | 46°33' - 40°56 N and 27°27' - 41°42' E |
| --- | --- |
| Surface area | 432,000 km2 |
| Drainage/catchment area | 2,000,000 km2 (about six times larger than its surface area) |
| Total water volume | 547,000 km3 |
| Total shoreline (without Sea of Azov shoreline) | 4,340 km |
| Total annual river inflow | 340.6 km3 |
| Average/maximal depth | 1,315/2,212 m |
| Salinity | 18 - 22 pro mil |
| Average fresh water balance | 3.7 - 441 km3 |
| Total water renewal period | Over 100 hundred years |
| Anoxia & hydrogen sulfide (H2S) ratio | Anoxic to nearly 87%/Hydrogen sulfide (H2S) below 150m. |

For Bulgaria and Romania the following key figures are important

Table 4: Key figures for Bulgaria and Romania

|  | Bulgaria (BG) | Romania (RO) |
| --- | --- | --- |
| Coastline length | 378 km | 245 km |
| Territorial Waters | 6,358 km² | XXX |
| Marine waters | 29,052 km2 | 22,486 km2 |
| MPA* | 31 sites | 9 sites |
| MPA coverage | 13.8 km2 | 6,057.2 km2 |

* For Bulgaria, the number of MPAs includes 17 protected areas (PAs) under Habitats Directive and 14 PAs under the Birds Directive that have land area and marine aquatory (designated under Bulgarian Biodiversity Act and Natura 2000);  out of this MPAs number, but so far with a status of different type of protection along the coast, are 50 protected localities under Bulgarian Protected Areas Act (1 of them is with only marine aquatory), 19 natural landmarks and 2 natural parks.

The leading regional intergovernmental actor for the protection of the Black Sea from pollution and other environmental problems is the Commission on the Protection of the Black Sea Against Pollution (Black Sea Commission or BSC). The BSC is the governing and implementing body of the Convention on the Protection of the Black Sea Against Pollution (Bucharest Convention), adopted by the six Black Sea states in 1992. The Convention entered into force on 15th January 1994, but the Commission Secretariat became officially operational only in October 2000, which seriously hampered the functioning of the BSC in different areas. The Bucharest Convention, together with its satellite documents (four Protocols, Declarations and the Black Sea Strategic Action Plan (BS SAP 2009) form the legal basis for the work of the BSC. Moreover, in 2009, a revised and updated version of the 1992 LBS Protocol was adopted, which however has not since been ratified by the necessary minimum of Contracting Parties.

In the BSC, only Bulgaria and Romania are EU Member States with the obligation to implement the MSFD.

The MSFD was adopted shortly before the BS SAP 2009 was approved. The underlying philosophies of the MSFD and the BS SAP are different but complementary. The BS SAP is based on targeting environmental priority problems for the Black Sea; its management targets do not directly state what the environmental status should be as a result of the activities undertaken under the BS SAP. In the framework of the Final Diagnostic Report 2010 (produced by the BSC Permanent Secretariat with the financial support of the European Environment Agency (EEA)), a summary of the suitability of Black Sea data for the calculation of BSC and EEA indicators and MSFD descriptors was prepared. As a result, some indicators were identified for almost all MSFD descriptors (except Descriptor 10). In 2015, the BSC revised and approved regional reporting indicators, which are reported annually to the BSC by the respective Advisory Groups supporting the BSC. The indicators are grouped in six tables, according to the thematic focus of Advisory Groups (e.g. Biodiversity, Land-based pollution (LBP) etc.). Some of the agreed indicators are also quite relevant for the MSFD implementation process.

The BSC approved the revised Black Sea Integrated Monitoring and Assessment Programme 2017-2022 (BSIMAP) in October 2016. BSIMAP was built in the light of MSFD implementation, taking into account descriptors, GES and targets. After the approval of BSIMAP by the BSC in October, these regional reporting indicators became a part of it. Its adoption is a positive step, as the indicators contribute to the harmonisation of the reporting format across Black Sea countries and could provide the basis for comparing general environmental trends of the Black Sea marine environment. However, more efforts are needed towards harmonisation of methodological approaches in determining GES by descriptors, criteria and/or indicators at the regional level, in order to better align the MSFD and BS SAP 2009 implementation processes in the future.

As a result, at the present moment, Bulgaria and Romania do not consider that the BSC regional reporting indicators could provide a fully adequate basis for MSFD monitoring and assessment and are therefore progressing with the identification of common indicators under the MSFD. Both Member States have the willingness to share the data and knowledge achieved during the implementation MSFD with the other Black Sea countries to support the integration process between MSFD and the regional BSIMAP as far as possible.

# Human activities, predominant pressures and impacts on the marine environment

With regard to pressures from human activities, the Black Sea is often considered to be one of the most heavily impacted seas in the world. A combination of certain features renders its ecosystem very vulnerable to anthropogenic pressures. These are, in particular: a large catchment-to-surface area ratio, a high number of users, large population centers surrounding the sea, a poor drainage to the oceans (residence time of water over 100 years) with a respectively high accumulation potential for pollutants, stratified waters, and the saturation of seawater layers below 150m with hydrogen sulfide (H2S).

The Black Sea Transboundary Diagnostics Analysis 2007, which is the scientific foundation of the BS SAP adopted in 2009, identifies four main environmental problems of transboundary importance:

- eutrophication/nutrient enrichment;

- changes in marine living resources;

- chemical pollution (including oil); and

- biodiversity/habitat changes, including alien species introduction.

However, the MSFD requires the identification of the predominant pressures and human activities according to Annex III and in line with the eleven descriptors of GES. Annex III to Directive 2008/56/EC lays down the indicative lists of characteristics, pressures and impacts which are referred to in Articles 8(1), 9(1), 9(3), 10(1), 11(1) and 24 of that Directive and indicates the human activities relevant for the analyses according to Art. 8.1c.

In 2012, as part of the first cycle of implementation of their marine strategies, Member States reported under Articles 9(2) and 10(2) of Directive 2008/56/EC on the initial assessment of their marine waters, the determination of good environmental status and their environmental targets. The Commission’s assessment of these Member State’s reports, undertaken in accordance with Article 12 of that Directive, highlighted that more efforts were urgently needed if Member States and the Union are to reach good environmental status by 2020. To ensure that the second cycle of implementation contributes to the achievement of the Directive’s objectives and yields more consistent determinations of good environmental status, the Commission therefore recommended in its report on the first phase of implementation that, at Union level, the Commission services and Member States collaborate to "revise, strengthen and improve Decision 2010/477/EU by 2015, aiming at a clearer, simpler, more concise, more coherent and comparable set of good environmental status criteria and methodological standards" and "review Annex III of the Marine Strategy Framework Directive, and if necessary revise, and develop specific guidance to ensure a more coherent and consistent approach for assessments in the next implementation cycle".

The new Annex III to Directive 2008/56/EC provides "elements for assessment (Article 8(1) of that Directive) with regard to good environmental status (Article 9(1) of that Directive), provide elements for monitoring (Article 11(1) of that Directive), which are complimentary to assessment (e.g. temperature, salinity), and provide elements for consideration when setting targets (Article 10(1) of the Directive)" (preamble of updated Annex III). It is important to ensure that the elements in Annex III are clearly related to the qualitative descriptors of Annex I and to Decision 2010/477/EU, as well as to their application in relation to Articles 8, 9, 10 and 11 of Directive 2008/56/EC.

For the Romanian and Bulgarian marine waters, the following relationships have been identified between human activities, the pressures they exert on the environment and the consequent state of the environment, taking account of the impacts (adverse effects) from the pressures. In the following figure, each aspect is illustrated with indicative examples. The links to the three parts of Article 8(1) and the associated tables in Annex III of the MSFD are also shown.

Figure 1: Schematic relation between human activities, the pressures they exert on the environment and the consequent state of the environment (according to the technical background paper "Review of the GES Decision 2010/477/EU and MSFD Annex III – cross-cutting issues")

The table below provides the relationship between activity, predominant pressures and impact as well as state, as relevant to different MSFD descriptors in RO and BG. This table is based on the assessment of finished and ongoing research projects in the Black Sea region, and expert discussion in various meetings.

Table 5: Activities, predominant pressures and impacts and state, as relevant to different MSFD descriptors in RO and BG marine waters

| Descriptor | Activity | Pressure (at source) | Pressure at sea | Environmental impact (effect) | Environmental status (D1, D4) |
| --- | --- | --- | --- | --- | --- |
| D2 | Shipping | Introductions of non-indigenous species via ship hulls and ballast water | Populations of NIS established in marine waters | Changed composition of native marine communities, displacement of native species | Altered condition of pelagic and benthic communities, and bird, mammal and fish communities. |
| D3 | Fishing | Removal of commercial fish and other (non-commercial) species |  | Mortality of fish and other species | Reduced population size, altered age/size structure of population |
| D5 | Agriculture, Forestry Aquaculture – freshwater, Transport | Introduction of nutrients through rivers or directly from land | Raised nutrient levels (enrichment) in sea | Increased algal productivity, oxygen depletion, benthic mortality, fish mortality | Altered condition of plankton and benthic communities, hypoxia/anoxia |
| D6 | Fishing (demersal/ benthic) | Disturbance of seabed |  | Changes in sediment structure, injury and mortality of species | Altered benthic community and habitat condition |
| D6 | Canalisation and other watercourse modification; Coastal defence and flood protection | Change in seabed substrate (e.g. to concrete, metal) |  | Loss of natural habitat, Changes in species composition | Habitat loss, altered habitat condition (hydrological conditions and community) |
| D7 | Extraction of oil and gas, including infrastructure | Permanent alteration of hydrographical conditions |  | Loss of natural habitat, altered hydrological conditions (D7) | Habitat loss, altered habitat condition (hydrological conditions and community) |
| D8 | Industry, Transport. Urban Waste Water | Contaminants | Contaminants in water, sediment and biota | Effects of contaminants on life history aspects of species; | Altered condition of species (e.g. reproductive ability) |
| D9 | Agriculture, Aquaculture – freshwater, Urban Waste Water | Contaminants | Contaminants in biota | Accumulation of contaminants in seafood and human health effects | Altered condition of species (e.g. reproductive ability) |
| D10 | Tourism, Shipping. Urban Waste Water | Input of litter – discarded on beach | Litter on seabed | Smothering of benthic habitats, injury to animals | Altered habitat condition, affected condition of species |
| D11 | Extraction of oil and gas, including infrastructure, Shipping, Tourism | Noise from piling | Noise level in sea | Disturbs cetaceans, moving away from noise | Altered species distribution |

In the following section information summarised in the table above is described in more detail, including maps of human activities and pressures, where available. The human activities listed below are those which have been identified by RO and BG experts and research activities as the main causes for the predominate pressures and impacts identified.

## Physical restructuring of rivers, coastline or seabed (water management)

Physical restructuring of rivers, coastline or seabed is based on various human activities as listed below.

### Canalisation and other watercourse modifications

For Bulgaria hydromorphological alterations including canalisation and other watercourse modifications of surface water bodies along the coast have not been assessed as a significant type of pressure on the marine environment.

In Romania two main canals exist, naimly i) Danube – Black Sea channel with 2 branches: Poarta Alba – Midia and Poarta Alba – Agigea, and ii) the Sulina channel - central Danube branch/arm modified for navigation purpose.

Map 1: Channels into the Black Sea

### Coastal defense and flood protection

The natural forces and processes which shape the coastline give rise to two hazards which can affect coastal property: erosion and flooding. Coastal erosion is the process that forms predominantly cliff coastlines. 70.8% of the Bulgarian coastline is eroding and in Romania it is noticed especially during the last decades, as it has become almost a general phenomenon on the Romanian seaside, leading to the decrease of the beach surfaces. In order to prevent against such events physical modifications of the coast have been established. These take a variety of forms including the use of concrete seawalls, rock armour.

Flood and coastal defenses cause direct pressures on habitats (water column and benthic habitats) through habitat loss and further significant indirect impacts may occur as a result of coastal squeeze.

Map 2: Coastal defence works

Coastal defence works take a length of 70 km in Romania and X km in Bulgaria

### Extraction of non-living resources

The extraction of non-living resources refers normally to marine mining (sand, gravel, rock), dredging and desalination/water abstraction. While in Bulgaria no such activities are currently taking place in Romania beach nourishment can be found.

[Map will come from RO- is there Information on amount?]

### Extraction of oil and gas, including infrastructure

Oil and gas production overall is rather limited in both countries. Romania runs 4 offshore platforms for offshore HC exploitation. Bulgaria only has three installations for exploration so far in its EEZ.

In Bulgaria an infrastructure related to oil and gas abstraction is available in front of Varna Bay (close to Cape Galata), a pipeline from 12 nm area.

in Romania infrastructure related to oil and gas are 2 pipelines ( each about 63 km in front of Portita (Sinoe lake) and one oil terminal near Midia Cape.

Figure 2: Gas pipeline in BG and RO

| Bulgaria: pipeline 12nm in front of Varna Bay (close to Cape Galata) | Romania: 2 pipelines ( each about 63 km in front of Portita (Sinoe lake) and one oil terminal near Midia Cape. |
| --- | --- |

The impacts of these activities lead to pressures on the hydrographical conditions of the natural coastline and the seabed (D7, D6) leading to the loss of natural habitats.

These activities might also have negative impacts on the water quality as this might release contaminates (in particular from oil and gas abstractions but also from contaminated sediments) (D8). The contaminants might accumulate in seafood leading to human health effects (D9), but also to chronic pollution effects in the marine habitats. These activities also cause underwater noise, especially during construction  (D11), that might impact cetaceans by disturbing them and changing their natural habitats by, moving away due to the generated noise.

## Extraction of living resources

### Fish and shellfish harvesting (professional, recreational)

According to the 2014 Romanian Fleet Report, Romania has 158 registered vessels, of which 123 are active. In 2012, the number of fishing enterprises in the Romanian fleet totalled 91, with the vast majority (79 %) owning a single vessel. Only 18 % of enterprises owned between two and five fishing vessels. Total employment in 2012 was estimated at 471 jobs. Employment levels dropped between 2008 and 2012, by 48 %.

According to the Bulgarian EMFF Operational Programme (OP), the fishing fleet consists of 2 005 fishing vessels (2 043 in 2013), of which 1 104 are active. In 2012, there were 184 fishing enterprises: the majority (68 %) owned a single vessel, and 30 % owned between two and five fishing vessels. Total employment in 2012 was estimated at 5 638 jobs.

Data on landings from the Black Sea by Bulgaria and Romania from 2005–2014 (latest 10 years of data available) from FAO FishStat are provided in Table 6 and demonstrate that 12 species make up 98% of the combined volume of landings of the two countries.

Table 6: Bulgaria and Romania combined landings from the Black Sea, 2005–2014 (tonnes)

| - Species | - Landings 2005–2014 (tonnes) | - Cumulative % of landings |
| --- | --- | --- |
| Sea snails | 38,064 | 45% |
| European sprat | 36,903 | 88% |
| Mediterranean horse mackerel | 2,028 | 90% |
| Bluefish | 1,178 | 92% |
| European anchovy | 1,139 | 93% |
| Red mullet | 1,036 | 94% |
| Turbot | 844 | 95% |
| Gobies not elsewhere identified | 814 | 96% |
| Pontic shad | 634 | 97% |
| Thornback ray | 437 | 98% |
| Picked dogfish | 383 | 98% |
| Whiting | 358 | 98% |
| Other | 1,279 | - 2% |
| - TOTAL | 85,097 | - 100% |

- Source: FAO FishStat, GFCM regional dataset, 1970–2014.

The set quotas for fishing in both countries are as follows:

|  | 2016 |  | 2017 |  |
| --- | --- | --- | --- | --- |
|  | BG | RO | BG | RO |
| Turbot | 43.2t | 43.2t | 43.2t | 43.2t |
| Sprat | 8032.5t | 3442.5t | 8032.5t | 3442.5t |

In Bulgaria ,a 3 miles bans for fishing by trawling exists. In Romania, trawling is forbidden in all MPAs.

[RO/BG to provide map on fishing areas? if available? How important is recreational fishing?]

Map 3Zones with high, medium and low pressure from abrasion of seabed due to fishery activities, 2011; high – red, medium – yellow, low - green.

Recreational fishing was not assessed under the 2012 Initial Assessment of marine environment and still there is no information available.

For Romania:

### Fish and shellfish processing

For Bulgaria there are few fish ports that have special areas for fish and shellfish processing. Still these activities are not so widely presented. Most of catch is transported directly to urban areas (fresh products to fish and other markets) and small fish factories for further processing.

[RO to provide information if available?]

## Cultivation of living resources

Cultivation of living resources refers manly to aquaculture, agriculture and forestry. The impacts of aquaculture, agriculture and forestry lead to increased nutrients levels (D5) and contaminated waters (D9) from pesticides. Increase nutrient levels are a main driver for Increased algal productivity, oxygen depletion, benthic mortality and fish mortality, while pesticides might enter the human food chain but have also chronic toxic effects on the habitats.

### Marine Aquaculture

While aquaculture can be found in Romania on one marine site, Bulgaria has 41 mussel farms along the coast.

Map 4: Aquaculture (mussels farms)

In the Black Sea marine region, Turkey is the main producer, harvesting 6199 tons of Rapana whelk, followed by Bulgaria with 4732 tons, Romania with 1953 tons in 2014 (STECF, 2015).

### Agriculture

According to the latest EEA statistics the % of agriculturally used land per coastal region, is ranging between 46% and 62%. As shown in the table below the area in Bulgaria increased over the years, while the area in Romania slightly decreased.

Table 7: Agricultural areas in the coastal zone

| NUTS_ID | AREA | Nuts_name | 2009 | 2010 | 2011 | 2012 | % in 2012 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| BG34 | 1980293,02 | Yugoiztochen | 898,6 | 905,48 | 909,33 | 918,804 | 46,3973761 |
| BG33 | 1464666,08 | Severoiztochen | 869,1 | 880,24 | 878,977 | 886,789 | 60,5454726 |
| RO22 | 3574926,95 | Sud-Est | 2319,1 | 2363,865 | 2345,612 | 2229,603 | 62,367792 |

[BG to provide information based on CORINE landcover]

Map 5: Agricultural and forestry areas in RO (Data source: Corine land Cover)

### Forestry

[BG to provide information based on CORINE landcover]

## Transport

### Shipping

According to the latest available information in Eurostat (2014) the main shipping activities are related to freight transport. In 2014 the number of vessels in the main ports (based on inwards declarations) was in RO 3,354 and in BG 4,320.

Passenger transport is very limited. The main ferry routes are shown below.

Map 6: Ferry routes

The environmental impact of shipping includes oil and acoustic pollution (D8, D9, D11), pollution from marine litter (D10), the introduction if non-indigenous species from ballast water (D2) and there is a risk of wildlife collisions (D1).

### Ports

According to the latest available information in Eurostat (2014), RO has four main ports (Constanta, Mangalia, ………) which handled a total cargo of 43.8 million tonnes (- 5.6% compared to 2013). BG has two main industrial ports (Varna and Burgas), which handled 27.4 million tonnes (+ 0.4% compared to 2013). Related to Bulgaria, Map 4 below includes public transport ports of national and regional importance, special purpose ports, fish ports, marinas.

Map 7: Ports in RO and BG

Ports can be considered as physical restructuring of rivers, coastline or seabed and the impacts are as mentioned above, such facilities are also often a main source for contaminated (D9) and marine litter (D10), leading to accumulation of contaminates and litter leads to smothering of benthic habitats, injury to animals. Shipping might Introduce non-indigenous species via ship hulls and ballast water leading to a changed composition of native marine communities and the displacement of native species (D2).

## Urban and industrial uses

One of the most damaging ways in which cities and industrial areas pollute coastal and marine waters is the discharge of wastewater and sewage. Many coastal cities discharge sewage, industrial effluent and other wastewater directly into their surrounding seas. Other sources are runoff from rainfalls and litter that is not systematically collected. The impacts of these urban and industrial activities are similar to those from activities falling under the cultivation of living resources.

### Urban uses and Industrial uses

[BG to provide information based on CORINE landcover]

Map 8: Urban and Industrial uses in RO (Data source: Corine land Cover)

### Waste treatment and disposal

In RO 10 waste water treatment plants with direct discharge into Black Sea exist, and in BG 9 water treatment plants exist as shown in the map below. They treatment capacity in RO is X inhabitant equivalent. In BG these WWTPs serve coastal municipalities mostly above 10 000 PE. The level of treatment can be summarised as follows for these plants:

For Bulgaria: 9 coastal WWTPs discharge directly or not treated wastewaters into coastal waters. Untreated wastewaters of 5 coastal municipalities are discharging into coastal waters (sewerages).

The level of treatment of municipal wastewaters as increasing in the last 6 years by building new and reconstructing and modernising existing WWTPs, by providing biological treatment and removing TN & TP (secondary and tertiary stage). 4 pipelines for deep discharges from coastal WWTPs up to 1nm were constructed. The population connected to public sewage network system (PSNS) is 71,5 %. Map 5 below also shows some small resort WWTPs which are situated around up to 2 km from the coast (source: second RBMP of Black sea river basin for water management 2016 – 2021, http://www.bsbd.org/bg/index_bg_5493788.html)

For Romania:

[RO to provide information]

Map 9: Urban waste water plants discharging into the WFD coastal waters in RO

## Inflow from Rivers

There are 28 rivers in the Black Sea Basin on the territory of Bulgaria (those, which directly or indirectly inflow into the Black Sea). They bring X tons of nutrients and several chemical substances into the Black Sea. Along the Romanian coast, the Black Sea coastal waters quality depends on the quality of the Danube River having in mind that most of the pollutants stemming to the BS are carried out by the Danube River (99,53% nutrients, 99% Nitrogen, 91,83% Phosphorus – as estimated for Romania). The dominant direction of Black Sea currents favors the fast distribution of nutrients (D5) and hazardous substances (D9) along the Romanian coast. What about marine litter?

Map 10: Rivers in Bulgaria and Romania flowing into the Black Sea

## Tourism and leisure

The main concerns surrounding tourism and recreation activities on the marine environment include boat anchoring on sensitive habitats such as seagrass beds, trampling of sensitive intertidal habitats, organic enrichment from boats and the introduction of litter. These pressures tend to occur at small spatial scales but may result in significant impacts on sensitive habitats at a local level, particularly as there may be few regulatory controls to manage the activities other than voluntary best practice agreements.

### Tourism and leisure infrastructure

According to Eurostat, in 2009, European regions with a Black Sea coastline had around 2 100 tourist accommodation facilities, 77.0 % of which were hotels (or similar establishments). The number of beds per km² in these regions is on average approximately 10 beds per km² as opposed to an average of 16 beds per km² in the Mediterranean's coastal regions.

For Bulgaria (2015): touristic accommodation capacities (number of places ) are 1,252. Number of tourist arrivals (number of tourists): 2.537.131.

Number of accommodation beds: 217.111.

### Tourism and leisure activities

For Bulgaria (2015): Number of nights: 13.664.046.

| ROMANIA County | Local administrative units | Night of tourists (2016) |
| --- | --- | --- |
| Constanta | CONSTANTA | 1666702 |
| Constanta | MANGALIA | 1603691 |
| Constanta | MEDGIDIA | 5115 |
| Constanta | EFORIE | 723927 |
| Constanta | NAVODARI | 169912 |
| Constanta | OVIDIU | 10591 |
| Constanta | TECHIRGHIOL | 141400 |
| Constanta | 23 AUGUST | 4076 |
| Constanta | AGIGEA | 63 |
| Constanta | COSTINESTI | 165022 |
| Constanta | LIMANU | 39944 |
| Constanta | MIHAIL KOGALNICEANU | 3296 |
| Constanta | TUZLA | 797 |
| Tulcea | TULCEA | 79429 |
| Tulcea | SULINA | 542 |
| Tulcea | JURILOVCA | 20976 |
| Tulcea | MAHMUDIA | 7303 |
| Tulcea | MURIGHIOL | 5267 |
| Tulcea | SFINTU GHEORGHE | 22134 |

Source: Data source: INSSE Romania

Other infrastructure is related to marinas: in Bulgaria, there are 14 marinas, which are shown on Map 11 “Ports in RO and BG”. In Romania: Y.

Bathing waters: According to the information reported to the EEA (European bathing water quality report for 2015), the number of coastal bathing waters in Bulgaria are 90 of which 3 have been assessed in poor quality. The number of "Blue Flag" beaches in Bulgaria for 2014 is 10. In Romania, the number of coastal bathing is 49 of which 1 is in poor status.

Map 12: State of bathing waters

[RO please update if you have any newer information and complete if possible)]

## Military operations

Both countries have designated areas for military operation but the exact dimension and locations are confidential.

# Assessment Areas and Common Indicators

This part of the Roof Report addresses the harmonised aspects of MSFD Art. 8, 9 and 10, focusing on common indicators, which are described per Descriptor and criterion (according to the approved draft versions of Annex III and the Commission Decision). Common threshold values are described as well, if agreed between Romania and Bulgaria. More detailed information on the common indicators and information on monitoring and status assessment is provided in the "Indicator Factsheets" annexed to this report, which are filled with the information available in early 2017 (some will have to be finalised until 2018).

The section also provides an overall description of each Descriptor, highlighting the status information available in the project´s lifetime (i.e. until the beginning of 2017).

Common GES definitions as described below are mostly based on the definitions provided by the two countries at the beginning of the project (early 2016), i.e. they do not take the approved draft versions of Annex III and the Commission Decision into account. In the scope of the national project “Investigations on the state of the marine environment and improving monitoring programs developed under the MSFD” (ISMEIMP), Bulgaria has revised national GES definitions, targets and indicators (and has defined quantitative threshold values where possible) following revised COM Decision and Annex III MSFD 2008/56/EC.

Indicators that are not commonly agreed will be reported separately in the national reporting in 2018.

The approved draft version of the Commission Decision requires EU MS to define quantitative thresholds, but only from 2018 onwards. In many cases, such thresholds are not possible to define in neither Bulgaria nor Romania presently (specific explanations are provided below).

This chapter is open for other Black Sea countries to contribute with descriptions of their own GES and targets/indicators definitions, if available; this will happen in a different structure (probably as separate, individual boxes).

## Assessment areas

Romania assessment areas under MSFD and WFD and national monitoring network

[ BG add information and map with other areas]

Map 13: Coastal Water Bodies under the Water Framework Directive and monitoring stations

## Descriptor 2: Non-indigenous Species (NIS)

For Descriptor 2, the indicator for the primary criterion C1 is narrowly defined in the approved draft version of the Commission Decision ("The number of non-indigenous species which are newly introduced via human activity into the wild"), and was proposed/is confirmed as a common indicator in both countries; however, a common threshold value has not yet been defined.

For the secondary criteria C2 and C3, only one species (Mnemiopsis leidyi) was selected as an indicator species (biomass for C2, with a common threshold defined, and the Biopollution Index for C3, with a proposed common threshold). Other species to be covered and associated threshold values have not yet been defined.

[Information/conclusions on status of D2 (overall) or the assessed common indicators will be included here until 2018]

An assessment of the status of D2 is not possible at the moment, due to missing data/information regarding other species than Mnemiopsis leidyi.

| Descriptor 2 – Non-indigenous species introduced by human activities are at levels that do not adversely alter the ecosystems. |  |  |
| --- | --- | --- |
| Criteria | Indicator in common (RO+BG) | Common assessment and threshold (RO+BG) |
| D2C1 | PROPOSAL: the number of NIS which are newly introduced via human activity into the wild, per assessment period (6 years), measured from the reference year as reported for the initial assessment under Article 8(1) | To be defined according to the revised GES Decision. |
| D2C2 | Biomass of Mnemiopsis leidyi | Mean biomass ≤ 4 g/m3 or 120 g/m2 (Vinogradov et al., 2005) Others to be defined according to the revised GES Decision. |
| D2C3 | Biopollution Index for Mnemiopsis leidyi | PROPOSAL: BPLI ≤ 2; ADR: C Impact on community: C1 – C2 Impact on habitats: H1 – H2 Impact on ecosystems: E1 – E2 |
|  | Ratio between non-indigenous and indigenous species | Others to be defined according to the revised GES Decision. |

GES: No common definition.

BG: Bulgaria has defined GES in terms of NIS on general Descriptor level, also for D2C1, D2C2 and D2C3. The monitoring programme from 2014 partially addressed the indicators related to D2. Revised programme was improved considerably and now provides adequate GES definitions. At this stage the definition of GES on Descriptor 2 is defined only in terms of planktonic communities: numbers and biomass of invasive alien species below the thresholds for GES and NIS do not have measurable effects on native species, communities and habitats and ecosystem functions.

RO: Good Environmental Status is achieved when the abundance and distribution of non-indigenous species do not increase and there are no further introductions of non-indigenous species. Non-indigenous species should not cause negative influence on population of indigenous species and natural habitats.

Targets: There are no agreed common environmental targets for D2 according to the conceptual use of targets as expressed in the Common Understanding document (i.e. as "operational tools", such as necessary levels of reduction in each pressure at its sources).

Status assessment if possible (link to Art. 8 and maps): An assessment of the status of Descriptor 2 is not possible, as several indicators and/or threshold definitions are missing. Information/conclusions on status of D2 (overall) or the assessed common indicators will be included here until 2018.

In Annex I, Indicator Factsheets for the following common indicators for D2 are included:

- D2C1 Number of NIS which are newly introduced via human activity into the wild, per assessment period (6 years), measured from the reference year as reported for the initial assessment under Article 8(1)

- D2C2 Biomass of Mnemiopsis leidyi

- D2C3 Biopollution Index for Mnemiopsis leidyi

- D2C3 Ratio between non-indigenous and indigenous species

## Descriptor 3: Commercially exploited fish and shellfish

For Descriptor 3, criteria C1 and C2 (both primary, as all criteria are primary for D3) are commonly defined in BG and RO, and closely follow the approved draft version of the Commission Decision (C1: "fishing mortality rate of populations of commercially-exploited species" and C2: "Spawning Stock Biomass of populations of commercially-exploited species"). For C1, a common threshold was also defined, but not for C2.

D3C3 has not yet been commonly defined.

[Information/conclusions on status of D3 (overall) or the assessed common indicators will be included here until 2018].

| Descriptor 3 – The population of commercial fish species is healthy |  |  |
| --- | --- | --- |
| Criteria | Indicator in common (RO+BG) | Common assessment and threshold (RO+BG) |
| D3C1 | Fishing mortality of sprat (Sprattus sprattus sulinus), whiting (Merlangius merlangus euxinus), turbot (Psetta maxima maeotica), horse mackerel (Trachurus mediterraneus ponticus), anchovy (Engraulis encrasicolus ponticus), dogfish (Squalus acanthias), red mullet (Mullus barbatus ponticus) | F <= FMSY = 0.64 (sprat) F≤ FMSY =0.4 (whiting) F ≤ FMSY = 0.15 (turbot) F≤ FMSY =0.54 (anchovy) F≤ FMSY =0.18 (dogfish) F≤ FMSY =0.46 (red mullet) Horse mackerel: no threshold value |
|  | Catch/biomass ratio of sprat and turbot (%) | Catch-biomass ratio <= 0.082 (sprat) Catch-biomass ratio <= 0.033 (turbot) |
| D3C2 | Population size (spawning stock biomass - SSB) of sprat, whiting, turbot, horse mackerel, anchovy, dogfish, red mullet (thousand tons) | To be defined according to the revised GES Decision. |
| D3C3 | To be defined according to the revised GES Decision. | To be defined according to the revised GES Decision. |

GES: The definitions of GES for D3 are not the same between RO and BG, although both address fishing mortality and spawning stock biomass.

BG: Populations of all commercially exploited fish and shellfish species are within safe biological limits, exhibiting a population age and size distribution that is indicative of a healthy stock. Fishing activities are conducted in a manner and at scale that does not surpass the maximum sustainable yield, do not lead to systematic decline of the exploited populations and their breeding capacity, do not harm their habitats (particularly harm to the seabed habitats by trawling gear), and do not reduce the potential for fish stock uses from the future generations.

RO: The level of fishing pressure and reproduction capabilities are at those levels set out in the environmental targets to sustain MSY for a long-term period of some relevant key fish species (sprat, turbot, whiting, horse mackerel, anchovy, dogfish, red mullet).

Targets: There are no agreed common environmental targets for D3 according to the conceptual use of targets as expressed in the Common Understanding document (i.e. as "operational tools", such as necessary levels of reduction in each pressure at its sources).

Status assessment if possible (link to Art. 8 and maps): The Indicator Factsheet on D3 (see Annex I) presents initial results for a preliminary status assessment.

In Annex I, Indicator Factsheets for the following indicators for D3 are included:

- D3: Status of commercial fish-stock incorporating assessment of D3C1 and D3C2 population size (spawning stock biomass - SSB) of sprat, whiting, turbot, horse mackerel, anchovy, dogfish, red mullet (thousand tonnes)

- D3C1 Catch/biomass ratio of sprat and turbot (%)

## Descriptor 5: Eutrophication

[revise text and split between WFD and MSFD areas when data has been submitted by RO/BG]

For Descriptor 5, common indicators are defined in Bulgaria and Romania for the primary criteria C1 (no threshold values defined yet) and C2 (no threshold values defined yet), as well as for the secondary criteria C4 (no threshold values defined yet), and C5 and C7, with partly defined threshold values.

For the primary criterion C5, discussions are ongoing with regard to the question of possibly using the secondary criterion C8 as a substitute (as proposed by the approved draft version of the Commission Decision).

For the secondary criterion C3, no common indicator has been defined yet.

[Information/conclusions on status of D5 (overall) or the assessed common indicators will be included here until 2018].

| Descriptor 5 – Eutrophication is minimised |  |  |
| --- | --- | --- |
| Criteria | Indicator in common (RO+BG) | Common assessment and threshold (RO+BG) |
| D5C1 | DIP and DIN for all marine waters except Coastal Waters, which are treated under WFD (N-NO3, N-NO2, N-NH4 are the parameters measured in both countries) | Both MS have threshold for DIP and DIN in all waters but the assessment methods (RO: 75 Percentile of all data, BF: seasonal average and one out all out) behind do not allow a common assessment. BG only data for pilot areas in both countries for open waters |
| D5C2 | Chl a in the water column | Both MS have threshold for Chla in all waters but the assessment methods (RO: 75 Percentile of all data, BF: seasonal average for spring and summer and one out all out) behind do not allow a common assessment. BG only data for pilot areas in both countries |
| D5C3 | To be defined according to the revised GES Decision. | To be defined according to the revised GES Decision. |
| D5C4 | Secchi depth (transparency) | To be defined according to the revised GES Decision. |
| D5C5 | Dissolved oxygen in the bottom of the water column (in Romania: D5C8 is considered to be used instead - under expert´s discussion) | Different thresholds are applied |
| D5C6 | Ecological Index (EI) (Dencheva. K., Doncheva V., 2014) | EI: >6 (up to 3m depth) |
|  | Wet biomass of opportunistic MA species For the criterion D5C6, the proportion of perennial and opportunistic species will be applied (the wet weight of perennial species must be > 60 % of the total WW, to be considered in GES). | Wet Weight of the opportunistic species <22% of the total WW of the samples (Dencheva. K., Doncheva V., 2014; Berov D., 2015) |
| D5C7 | EI (Dencheva. K., Doncheva V., 2014; Berov D., 2015) | EI: >6 (up to 3m depth) |
|  | Average WW of the ESGI macroalgae biomass (Dencheva. K., Doncheva V., 2014; Berov D., 2015) | >60% of the total WW of the samples (Dencheva. K., Doncheva V., 2014; Berov D., 2015) |
|  | WW of Cystoseira barbata The EI Index is an integrated index, developed based on average fresh biomass for the perennial and opportunsitic species. So, the index comprises the Average WW of the ESGI macroalgae biomass, the Wet Weight of the opportunistic species and the WW of Cystoseira barbata (these are not distinct indicators). For this crierion (D5C7), for Romania, the EI will be applied (the EI must be > 6, to be considered in GES). | >60% of the total WW of the samples (Dencheva. K., Doncheva V., 2014; Berov D., 2015) |
| D5C8 | To be defined according to the revised GES Decision. PROPOSAL/under discussion: M-AMBI*(n) | To be defined according to the revised GES Decision. |

GES: There is an overall GES definition for D5 as a whole only in Bulgaria; Romania defined GES according the three criteria listed in the "old" COM Decision 477/2010.

BG: Good Environmental Status in relation to this Descriptor 5 will be achieved when the human-induced eutrophication is minimised, especially the adverse effect thereof, such as losses in biodiversity, ecosystem degradation, harmful algal blooms and oxygen deficiency in bottom waters.

Also there are GES definition on criteria D5С2, D5С3, D5С4, D5С5, D5С6, D5С7 and D5С8.

RO: various definitions for D5.1, D5.1.1, D5.2 and D5.3.

Targets: There are no agreed common environmental targets for D5 according to the conceptual use of targets as expressed in the Common Understanding document (i.e. as "operational tools", such as necessary levels of reduction in each pressure at its sources).

Status assessment if possible (link to Art. 8 and maps): Information/conclusions on status of D5 (overall) or the assessed common indicators will be included here until 2018.

In Annex I, Indicator Factsheets on the following indicators for D5 are included:

- D5 on coastal waters under the WFD incooperating  D5-coastal waters incorporating assessments for D5C1 Nutrient concentrations, D5C2 - Chlorophyll a in the water column D5C4: Secchi depth (transparency)  D5C5- Dissolved oxygen in the bottom of the water column, D5C6 Opportunistic macroalgae of benthic habitats, D5C7 Macrophyte communities of benthic habitats and D5C8 Macrofaunal communities of benthic habitatsand D5 open waters incooperating D5-open waters incorporating assessments for D5C1 Nutrient concentrations, D5C2 - Chlorophyll a in the water column and D5C5- Dissolved oxygen in the bottom of the water column based on the BEAST approach.

- D5C6 for open waters Wet biomass of opportunistic MA species .

- D5C7 for open waters Average wet weight of the ESGI macroalgae biomass.

- D5C7 for open waters Wet weight of Cystoseira barbata.

- D5C8 and C5 for open waters M-AMBI*(n).

## Descriptor 6: Seafloor integrity (physical loss and disturbance)

For Descriptor 6, no pressure-related common indicators (i.e. for the primary criteria C1, C2 and C3) have been defined between Romania and Bulgaria.

In both countries, there is a lack of monitoring data to define and assess indicators and thresholds.

[Information/conclusions on status of D6 (overall) or the assessed common indicators will be included here until 2018].

| Descriptor 6 – The sea floor integrity ensures functioning of the ecosystem |  |  |
| --- | --- | --- |
| Criteria | Indicator in common | Common assessment and threshold (RO+BG) |
| D6C1 | To be defined according to the revised GES Decision. | To be defined according to the revised GES Decision. |
| D6C2 | To be defined according to the revised GES Decision. | To be defined according to the revised GES Decision. |
| D6C3 | To be defined according to the revised GES Decision. | To be defined according to the revised GES Decision. |
| For D6C4 and C5: see below (D1, 6) |  |  |

GES: There is no common GES definition for D6.

BG: There are a number of GES definitions for D6 revised/new defined in 2016.

RO: There is no GES definition for D6.

Targets: There are no agreed common environmental targets for D6 according to the conceptual use of targets as expressed in the Common Understanding document (i.e. as "operational tools", such as necessary levels of reduction in each pressure at its sources).

## Descriptor 7: Hydrographical changes

For Descriptor 7, no common indicators have been defined between Romania and Bulgaria.

BG is already following the approved draft version of the Commission Decision (November 2016), there are defined indicators, which support GES assessment under D1,6 (benthic habitats). RO still applies the 2010 Decision.

[Information/conclusions on status of D7 (overall) or the assessed common indicators will be included here until 2018].

| Descriptor 7 – Permanent alteration of hydrographical conditions does not adversely affect the ecosystem |  |  |
| --- | --- | --- |
| Criteria | Indicator in common | Common assessment and threshold (RO+BG) |
| D7C1 | To be defined according to the revised GES Decision. | MS are not required to set thresholds according to GES decisions 2016 |
| D7C2 | To be defined according to the revised GES Decision. | To be defined according to the revised GES Decision. |

GES: No common definition.

BG: Permanent alteration of hydrographical conditions resulting from human activities in the marine environment (individual and cumulative) does not have a significant adverse impact on the biotic and abiotic structure of the broad types of benthic habitats and their functions.

RO (proposal in May 2016): To avoid the negative effects of the ecosystem changes that may result in permanent hydrographical conditions. GES will be achieved when: the nature and the extent of any long-term changes in prevailing hydrographic conditions resulting from human activities (individual and cumulative) in the marine environment do not lead to a significant negative impact on the species, population or ecosystem, with the exception of heavily modified water bodies (including, but not limited to the sea salinity, sea temperature, pH and hydrodynamics).

Targets: There are no agreed common environmental targets for D7 according to the conceptual use of targets as expressed in the Common Understanding document (i.e. as "operational tools", such as necessary levels of reduction in each pressure at its sources).

## Descriptor 8: Contaminants

For Descriptor 8, common indicators and thresholds have been defined for the primary criterion C1 (concentrations of contaminants according to the WFD, EQS Directive and the BSIMAP).

No common indicators have been defined yet for criteria C2, C3 and C4.

[Information/conclusions on status of D8 (overall) or the assessed common indicators will be included here until 2018].

| Descriptor 8 – Concentrations of contaminants give no effects |  |  |
| --- | --- | --- |
| Criteria | Indicator in common (RO+BG) | Common assessment and threshold (RO+BG) |
| D8C1 | Concentrations of contaminants in accordance with WFD (RBSP), EQS Directive (Priority Substances) and BSIMAP | Concentrations of contaminants in accordance with the EQS Directive (Priority Substances) |
| D8C2 | To be defined according to the revised GES Decision. | To be defined according to the revised GES Decision. |
| D8C3 | To be defined according to the revised GES Decision. | To be defined according to the revised GES Decision. |
| D8C4 | To be defined according to the revised GES Decision. | To be defined according to the revised GES Decision. |

GES: The definitions of GES for D8 are not the same between RO and BG, but similar in their meaning.

BG: The concentrations of substances in the water and sediments are below their maximum levels as defined in European legislation (water and biota). Acute pollution events are prevented and are extremely rare.

RO: The contaminants concentrations measured in relevant matrices do not pose any risk of occurrence of pollution effects, which are shown by high levels of hazardous substances.

Targets: There are no agreed common environmental targets for D8 according to the conceptual use of targets as expressed in the Common Understanding document (i.e. as "operational tools", such as necessary levels of reduction in each pressure at its sources).

Status assessment if possible (link to Art. 8 and maps): The Indicator Factsheet on D8 (see Annex I) presents initial results for a preliminary status assessment.

In Annex I, an Indicator Factsheet on the following indicator for D8 is included:

- Concentrations of contaminants in accordance with WFD (RBSP), EQS Directive (Priority Substances) and BSIMAP.

## Descriptor 9: Contaminants in seafood

For Descriptor 9, common indicators and thresholds are defined for the sole criterion C1, according to Regulation (EC) No 1881/2006 and the Maximum Allowable Concentrations (MAC) values stated there.

[Information/conclusions on status of D9 (overall) or the assessed common indicators will be included here until 2018].

| Descriptor 9 – Contaminants in seafood are below safe levels |  |  |
| --- | --- | --- |
| Criteria | Indicator in common (RO+BG) | Common assessment and threshold (RO+BG) |
| D9C1 | Contaminants listed in Regulation (EC) No 1881/2006. | MAC from Regulation (EC) No 1881/2006. |

GES: The definitions of GES for D9 are not the same between RO and BG, but very similar in their meaning.

BG: The actual levels of contaminants are below the threshold values indicative of GES (no contaminants exceeding the maximum allowable levels for safety of human consumption) and do not increase in the long term.

RO: The level of contaminants (heavy metals, OCPs, PCBs and PAHs) in fish and shellfish caught from the wild do not affect the safety of human consumption (this definition is followed by references to Regulation (EC) No 1881/2006).

Targets: A common environmental target has been proposed in the frame of the EC support project - to phase out the use of contaminants and reach MAC levels below Regulation (EC) No 1881/2006.

Status assessment if possible (link to Art. 8 and maps): The Indicator Factsheet on D9 (see Annex I) presents initial results for a preliminary status assessment.

In Annex I, an Indicator Factsheet on the following indicator for D9 is included:

- Contaminants listed in Regulation (EC) No 1881/2006.

## Descriptor 10: Marine litter

For Descriptor 10, no common indicators have been defined between Romania and Bulgaria.

Bulgaria is already following the approved draft version of the Commission Decision (November 2016). There are defined several indicators for macro and microliter in different matrixes following the requirement of revised COM Decision and Marine litter Guidance, 2013. Still there is a lack of monitoring data to define basic and threshold values and to assess indicators.

In Romania, there is a lack of monitoring data to define and assess indicators and thresholds.

[Information/conclusions on status of D10 (overall) or the assessed common indicators will be included here until 2018].

| Descriptor 10 –Marine litter does not cause harm to the marine environment |  |  |
| --- | --- | --- |
| Criteria | Indicator in common (RO+BG) | Common assessment and threshold (RO+BG) |
| D10C1 | To be defined according to the revised GES Decision. | To be defined according to the revised GES Decision. |
| D10C2 | To be defined according to the revised GES Decision. | To be defined according to the revised GES Decision. |
| D10C3 | To be defined according to the revised GES Decision. | To be defined according to the revised GES Decision. |
| D10C4 | To be defined according to the revised GES Decision. | To be defined according to the revised GES Decision. |

GES: No common GES definitions between Bulgaria and Romania regarding D10.

BG: Absence or negligible quantities of marine litter deposited on the beaches/coastlines, floating on the marine surface and deposited on the seafloor close to coastal "hot spots" as sources of waste - estuaries, resorts, urbanized areas, public beaches and seaports and in the open marine water. Absence of registered cases of found entangled, injured and / or dead marine mammals, seabirds and fish, as a result of introduced marine litter in the environment.

RO: Good environmental status is achieved when the impact of marine litter and their decomposition products is reduced, ceases to cause harm to the marine and coastal environment.

Targets: There are no agreed common environmental targets for D10 according to the conceptual use of targets as expressed in the Common Understanding document (i.e. as "operational tools", such as necessary levels of reduction in each pressure at its sources).

## Descriptor 11: Underwater noise

For Descriptor 11, no common indicators have been defined between Romania and Bulgaria.

Bulgaria has defined two indicators, whose definitions are very similar to both criteria D11C1 and D11C2. Not possible to define specific targets, also basic and threshold values due to lack of data on underwater noise level and effects on marine animals.

In Romania, there is a lack of monitoring data to define and assess indicators and thresholds.

[Information/conclusions on status of D11 (overall) or the assessed common indicators will be included here until 2018].

| Descriptor 11 – Introduction of energy (including underwater noise) does not adversely affect the ecosystem |  |  |
| --- | --- | --- |
| Criteria | Indicator in common (RO+BG) | Common assessment and threshold (RO+BG) |
| D11C1 | To be defined according to the revised GES Decision. | To be defined according to the revised GES Decision. |
| D11C2 | To be defined according to the revised GES Decision. | To be defined according to the revised GES Decision. |

GES:

BG: GES definition on criterion D11C1, indicator 1 (impulsive noise): Introduction of impulsive sounds by human activities in the marine environment is regulated so as not to have significant adverse effects at the population level or on vulnerable species and key functional groups.

GES definition on criterion D11C2, indicator 1 (continuous noise): The introduction of anthropogenic continuous low frequency sounds in the marine environment does not pose a significant risk to marine living resources at the population level or no significant impact on vulnerable species and key functional groups, such as marine mammals and fish by masking of biologically important sounds and behavior.

In Romania, a proposal was discussed in May 2016: "Adverse effects of underwater noise on the main groups of the sea species are minimized as far as possible".

Targets: There are no agreed common environmental targets for D11 according to the conceptual use of targets as expressed in the Common Understanding document (i.e. as "operational tools", such as necessary levels of reduction in each pressure at its sources).

## Descriptors 1, 4, 6: Biodiversity and ecosystems

The new GES decision required to look at a broad range of species groups covering birds, mammals, reptiles fish and cephalopods. Not all of them appear in the Black Sea marine region as shown in the table below.

Table 8: Species groups that can be found in the Black Sea

| Ecosystem component | Species groups | Appears in the Black Sea |
| --- | --- | --- |
| Birds | Grazing birds |  |
|  | Wading birds |  |
|  | Surface-feeding birds |  |
|  | Pelagic-feeding birds |  |
|  | Benthic-feeding birds |  |
| Mammals | Small toothed cetaceans |  |
|  | Deep-diving toothed cetaceans |  |
|  | Baleen whales |  |
|  | Seals |  |
| Reptiles | Turtles |  |
| Fish | Coastal fish |  |
|  | Pelagic shelf fish |  |
|  | Demersal shelf fish |  |
|  | Deep-sea fish |  |
| Cephalopods | Coastal/shelf cephalopods |  |
|  | Deep-sea cephalopods |  |

For the biodiversity descriptors 1,4,6, several common indicators have been defined for different themes and species groups.

Marine reptiles and cephalopods do not occur in the Black Sea, and are therefore excluded from the following list and table.

Ecosystem components (mammals, birds, fish) (relating to D1):

- Marine mammals: the incidental by-catch, the population abundance and the distributional range of the Harbour Porpoise, Common Dolphin, Bottlenose Dolphin are used as common indicators for D1C1, D1C2 and D1C4. No common indicators for marine mammals for D1C3 and D1C5 have been defined yet.

- Birds: common indicators have not been defined yet.

- Fish: the mortality rate per species from incidental by-catch is used as common indicator for D1C1, and one single coastal species group (Gobiidae) is used for C2, C3 and C4. No common indicator for D1C5-Fish has been defined yet.

Ecosystem component: Pelagic Habitats (relating to Descriptor 1): a common indicator has been defined for the condition of the habitat type (D1C6) (phytoplankton biomass for summer season in pelagic habitats), but without an accompanying commonly agreed threshold value.

Ecosystem component: Benthic Habitats (relating to Descriptors 1 and 6): for the two primary "state-related" criteria (D6C4: "extent of loss of the habitat type"; D6C5: "extent of adverse effects from anthropogenic pressures"), common indicators have been defined, but also without accompanying threshold values.

Theme Ecosystems, including food webs (relating to Descriptors 1 and 4): no common indicators have yet been defined between Romania and Bulgaria.

No common threshold values have been defined yet for all themes and ecosystem components.

[Information/conclusions on status of D1 (overall/by theme) or the assessed common indicators will be included here until 2018].

Theme Species groups of birds, mammals, reptiles, fish and cephalopods (relating to D1)

| Criteria | Indicator in common (RO+BG) | Common assessment and threshold (RO+BG) |
| --- | --- | --- |
| D1C1 | Mammals: Incidental by-catch of Harbour Porpoise, Common Dolphin, Bottlenose Dolphin Birds: To be defined according to the revised GES Decision. Fish: mortality rate per species from incidental by-catch | To be defined according to the revised GES Decision. |
| D1C2 | Mammals: Harbour Porpoise, Common Dolphin, Bottlenose Dolphin Birds: To be defined according to the revised GES Decision. Fish: Abundance of Gobiidae (coastal fish) |  |
| D1C3 | Mammals: To be defined according to the revised GES Decision. Birds: To be defined according to the revised GES Decision. Fish: Average length and 95 percentile of Gobiidae species (coastal fish) |  |
| D1C4 | Mammals: Harbour Porpoise, Common Dolphin, Bottlenose Dolphin Birds: To be defined according to the revised GES Decision. Fish: Area and distribution of Gobiidae (coastal fish) |  |
| D1C5 | Mammals: To be defined according to the revised GES Decision. Birds: To be defined according to the revised GES Decision. Fish: To be defined according to the revised GES Decision. |  |

Theme Pelagic habitats (relating to Descriptor 1)

| Criteria | Indicator in common (RO+BG) | Threshold in common (RO+BG) |
| --- | --- | --- |
| D1C6 | Phytoplankton biomass for summer season in pelagic habitats – coastal, shelf and offshore | Thresholds are different (RO: boundaries; BG: ranges) |

Theme Benthic habitats (relating to Descriptors 1 and 6)

| Criteria | Indicator in common (RO+BG) | Threshold in common (RO+BG) |
| --- | --- | --- |
| For D6C1 to C3: see above (D6) |  |  |
| D6C4 | Area (km²) and % loss of the broad habitat types due to permanent change of the natural seabed | Thresholds are different in BG and RO |
| D6C5 | M-AMBI*(n) (Sigovini et al. 2013, Todorova et al., 2015, Todorova, 2016) for broad habitat types |  |

Theme Ecosystems, including food webs (relating to Descriptors 1 and 4)

| Criteria | Indicator in common (RO+BG) | Threshold in common (RO+BG) |
| --- | --- | --- |
| D4C1 | To be defined according to the revised GES Decision. | To be defined according to the revised GES Decision. |
| D4C2 | To be defined according to the revised GES Decision. | To be defined according to the revised GES Decision. |
| D4C3 | To be defined according to the revised GES Decision. | To be defined according to the revised GES Decision. |
| D4C4 | To be defined according to the revised GES Decision. | To be defined according to the revised GES Decision. |

GES: Both countries use different approaches for defining GES for the biodiversity Descriptors.

BG: Bulgaria has GES definitions (according to the "old" COM Decision 477/2010) for mammals (frequency and accidental death due to human activities of three dolphin species), birds (population characteristics - species composition, distribution, population, health status and dynamics, habitat status of two protected bird species and other migratory species), fish (stable or increasing distribution, population size and relative abundance/biomass of non-commercial fish species) and habitats (distribution, extent and condition), plus criterion-level definitions specifying the general GES definitions.

RO: Romania has "overall" definitions only for mammals - the definition is the same as in Bulgaria. Other GES definitions are at other levels of criteria as in Bulgaria, although the general content is the same or similar: the same three dolphin species and their abundance and mortality, one of the two protected bird species (Mediterranean Shearwater), population abundance and biomass of one family of non-commercial populations (Fam. Gobiidae), and the distribution and distributional range of habitats.

Targets: There are no agreed common environmental targets for D1,4,6 according to the conceptual use of targets as expressed in the Common Understanding document (i.e. as "operational tools", such as necessary levels of reduction in each pressure at its sources).

Status assessment for mammals (D1) (link to Art. 8 and maps): The Indicator Factsheet on D1 Mammals (see Annex I) presents initial results for a preliminary status assessment.

In Annex I, Indicator Factsheets on the following indicators for D1,4,6 are included:

- D1- Mammals

- D1C1 Fish: Mortality rate per species from incidental by-catch

- D1C2 Fish: Abundance of Gobiidae (coastal fish)

- D1C3 Fish: Average length and 95 percentile of Gobiidae species (coastal fish)

- D1C4 Fish: Area and distribution of Gobiidae (coastal fish)

- D1C6 Phytoplankton biomass for summer season in pelagic habitats – coastal, shelf and offshore

- D6C4 Area (km²) and % loss of the broad habitat types due to permanent change of the natural seabed

- D6C4 M-AMBI*(n) for broad habitat types

# Article 8.1c and the analysis of the Cost of Degradation

[This section will be completed until 2018, as there are no concrete plans or ongoing projects at the moment in neither Bulgaria not Romania regarding an update of Art. 8.1c].

# Key conclusions and outlook

[To be finalized in 2018 when further elements of the Roof Report are ready and more status assessments have been made, GES and targets are aligned with the new GES decision].

# Competent authorities

## Bulgaria

The Marine Strategy Framework Directive 2008/56/EC (MSFD) is transposed to the Bulgarian national legislation by the Regulation on the protection of the environment in marine waters, transposed MSFD 2008/56/EO, adopted by the Government Decree № 273 from 23.11.2010 (prom. SG № 94 dated 30.11.2010, in force from 30.11.2010).

Responsible authorities for MSFD implementation according to the Regulation on the protection of the environment in marine waters (NOOSMV)

1. Council of Ministers of Republic of Bulgaria

2. Ministry of Environment and Waters (MoEW)

3. Black Sea Basin Directorate

Supporting institutions

4. Ministry of Transport, information technology and communications

- Executive Agency "Maritime administration" (EAMA) and Directorates Varna and Burgas

- Bulgarian Ports Infrastructure Company (BPI Co.) and its territorial divisions in Varna and Burgas.

5. Ministry of Health and its regional structures as Regional Health Inspectorates

6. Ministry of agriculture and food:

- Executive Agency for Fisheries and Aquaculture

- Bulgarian Food Safety Agency (BFSA) and Regional Directorates for Food Safety (ODBH) Dobrich, Varna and Burgas

7. Ministry of Foreign Affairs

8. Ministry of Tourism

9. Ministry of Energy

10. Ministry of Economy

11. Ministry of Regional Development and Public Works

12. Chairman of Bulgarian Academy of Science or authorized by them persons

Another national body for implementation of MSFD is Consultative and Coordinative Council for protection of Black Sea environment in Bulgarian sector is established under the Regulation on the protection of the environment in marine waters, transposed MSFD 2008/56/EO. Its role is to coordinate all responsible authorities and other relevant stakeholders involved in the process of development, discussion and adoption of Marine Strategy and its Program of Measures.

Other responsible authorities for MSFD implementation of some measures from the Bulgarian Program of measures (PoMs) are the following Black sea coastal Regional administrations and Municipalities:

- Dobrich regional administration

- Varna regional administration

- Burgas regional administration

- Municipality Shabla

- Municipality Kavarna

- Municipality Balchik

- Municipality Aksakovo

- Municipality Varna

- Municipality Byala

- Municipality Dolni chiflik

- Municipality Avren

- Municipality Nessebar

- Municipality Pomorie

- Municipality Burgas

- Municipality Sozopol

- Municipality Primorsko

- Municipality Tsarevo

## Romania

Competent authorities

(according to art.7, paragraph (1))

Table no. 1

| Crt. no. | The competent authority | Address |  | Legal status | Responsibilities |
| --- | --- | --- | --- | --- | --- |
| 1. | Ministry of Environment and Forests | Libertăţii Boulevard 12, sect. 5, Bucharest |  | G.D no. 1635/2009, art. 1 (3), art. 5 | Transposition and implementation of the Directive. |
| 2. | Ministry of Agriculture and Rural Development | Carol I Boulevard no.17, sect. 2, Bucharest |  | G.D no. 8/2009 art. 1 (3) | Protection of living resources. Fishing-barring in the marine protected areas. The fishing control. Discarding the population’s aggressive fishing tackle. Sustainable agricultural practices. |
| 3. | Ministry of Administration and Interior / General Inspectorate of the Border Police | Răzoare Street no. 5, Sector 6, Bucharest |  | GEO no. 104/2001 regarding the organization and functioning of the Romanian Border Police including subsequent amendments and additions. | Intervention and control in case of accident, living resources protection and combating poaching, monitoring of ships in territorial waters, securing the perimeter with ships in case of pollution. |
| 4. | Ministry of Transportation and Infrastructure | Bulevardul Dinicu Golescu Boulevard no. 38, sect.1, Bucharest |  | GD no. 76/2009 regarding the organization and functioning of the Ministry of Transport and Infrastructure including subsequent amendments and additions. | "Elaborate specific norms and regulations in transportation, on environmental protection and approve management plans for ship-generated waste and cargo residues’’. |
| Members |  |  |  |  |  |
| 1. | National Institute for Marine Research and Development “Grigore Antipa” (NIMRD) |  | Mamaia Boulevard 300, Constanta | G. D. no. 253/2015 - I.N.C.D.M. "Antipa" Constanta become again an independently institute under the Ministry of Education and Research. | Research and Monitoring of ecological status of marine ecosystems. Technical operator of monitoring network for national physical chemical, biological wastewater, as well coastal and marine surveillance erosion. NIMRD is empowered to propose the Ministry of Environment and Forests regulations field. |
| 2. | National Administration of Romanian Waters |  | Edgar Quinet Street 6, sect. 1, Bucharest | OU no.107 / 2002 including subsequent amendments and additions. | The competent authority to implement Directive. Harmonization of Marine Strategy Framework Directive requirements for marin environment with WFD requirements for achieving environmental objectives for coastline. Monitoring and water quality control for transitional and marine waters (inland sea, the territorial sea), as well as other activities arising from legislation. |
| 3. | Danube Delta Biosphere Reserve Authority |  | Portului Street 34A, 820243 – Tulcea | Law no. 82/1993 regarding foundation of Danube Delta Reserve | Protection of marine protected areas related to the Danube Delta Reserve. |
| 4. | National Agency for Environmental Protection |  | Splaiul Independenţei Boulevard 294, sect. 6, Bucharest | National Environmental Protection Agency is organized and operates in accordance with the Environmental Protection Law no. 137/1995, republished, with all amendments and completions, the G.D. no. 408/2004 on the organization and functioning of the Ministry of Environment and Water Management and compliance with the Civil Service Regulations and other applicable laws. | Emission control, environmental approvals. At local level performs the tasks and responsibilities of the Central Environmental Authority under competences established by the authority in accordance with the provisions of the Environmental Protection Law no. 137/1995, republished, including subsequent amendments and additions. |
| 5. | National Environmental Guard |  | Unirii Boulevard, no. 78, bl. J2, sect. 3, Bucharest | G.D. nr.112/2009 | Control and penalties for pollution and destruction of marine ecosystem. |
| 6. | National Agency for Fisheries and Aquaculture |  | Agricultorilor Street 37-39, sect. 2, Bucharest | G.D. nr. 1194/2008 amended and supplemented. | Management of living resources from marine protected areas (art. 1 (2) management authority of the Operational Programme for Fisheries, art. 8 lit. a and b. |
| 7. | National Agency for Mineral Resources |  | Mendeelev Street 36-38, sect.1, Bucharest | Law no. 85/2003 and Law no. 134/1995 amended and supplemented, GD. no. 756/2003 regarding founding and operating of NAMR | Reducing and eliminating pollution from marine offshore oil platforms. According to the letter f), follows the implementation of the established measures for environmental protection during and after the petroleum operations and mining activities; |
| 8. | Naval Romanian Authority |  | Constanta Port Site no.1, Constanta county. | G.D. no. 1133/2002 regarding organization and functioning of the Romanian Naval Authority, including subsequent amendments and additions. | - controls management and delivery of all waste generated on board ships and residues of transported goods; -investigates the navigation events and accidents, including accidental pollution with oil and other harmful substances, produced by vessels in territorial waters; - coordinate the sea actions intervention, in case of marine pollution. |
| 9. | National Company "Maritime Port Administration "- S. A. Constanta |  | Gara Maritimă Street, gate 1, Constanta | H. G. no. 517/1998 regarding the establishment of the National Company "Maritime Ports Administration Constanta" S.A., including subsequent amendments and additions. | Follow and coordinates shipping activities in harbors, conducted by authorized economic operators, manages the takeover waste activity generated by ship exploitation and cargo residue. |
| 10. | Constanta County Department of Public Health |  | M. Eminescu Street no. 2, Constanta | Order MS. No. 127/10 February 2009 regarding the approving of the organization and functioning regulation and organizational structure of public health county departments and Bucharest county. | Bathing water monitoring in jurisdiction areas. |
| 11. | Tulcea County Department of Public Health |  | Viitorului Street no. 50, Tulcea | Order MS. No. 127/10 February 2009 regarding the approving of the organization and functioning regulation and organizational structure of public health county departments and Bucharest county. | Bathing water monitoring in jurisdiction areas. |
| 12. | Local authorities bordering the Black Sea, namely the counties of Tulcea and Constanta |  |  | Law no. 51/2006 public utilities services; Law no. 241/2006 water supply and sanitation service, Article 9, Article 11. | Ensure adequate quality of the discharged water into the sea. |

Table no. 2

| Regional and subregional coordination |  |  |  |
| --- | --- | --- | --- |
| Black Sea Commission | Dolmabahce Sarayi 2 Hareket Kosku 34353 Besiktas Istanbul, Turkey | Convention for the Protection of the Black Sea Against Pollution (Bucharest Convention, 1992) | Coordination of pollution monitoring and control. Assessing the state of biodiversity in the marine region - Black Sea. |
| Romanian-Bulgarian Commission | - | Agreement between the Romanian Ministry of Environment and Water Management and the Bulgarian Ministry of Environment and Water regarding the Cooperation in Water Management, signed in Bucharest, 2004. | Coordination of activity in the common area in order to implement DCSM |
| Danube Commission | Vienna international Centre, D0412, Viena, Austria | Convention of Danube River Protection | Coordinating the activity of monitoring and control of water quality of the Danube River. Ensuring a river proper quality water will improve the quality of the marine water. |

# Annex I: Indicator Factsheets

See http://projects.eionet.europa.eu/black-sea-marine-region-documents/library/phase-iii/final-report/draft-final-version-06022017/draft-indicator-factsheets

European Commission. 2016. Guidance on a structure for MSFD-related indicator assessment reports. DG Environment, Brussels. pp12. (MSFD Guidance Document number 13).

COM(2014) 97 final: The first phase of implementation of the Marine Strategy Framework Directive (2008/56/EC); The European Commission's assessment and guidance and the CSWD accompanying this report (SWD(2014) 49 final. Milieu and the COM also assessed the coherence in the Black Sea region (Bulgaria and Romania) in the CSWD Regional Report and Milieu´s "Article 12 Technical Assessment of the MSFD 2012 obligations - Black Sea".

Specific, measurable, achievable, relevant and time-bound.

The National Action of Bulgaria contains activities that should be implemented in a coordinated way with Romanian authorities and is a good basis for common and coordinated activities for implementing MSFD requirements. Romania sent an official letter to the COM on 25.02.2015, which contains a document with conducted and anticipated actions in response to Art. 12 COM recommendations and can in fact be considered to be the National Action Plan of Romania.

Such as: Participation in the revision and implementation of BSIMAP, the presentation of Phase I and II results regarding GES/targets/indicators to the BSC (as a possible basis for the "regional monitoring programme", probably BSIMAP); and joint measures and the preparation of a regional document on marine litter to be prepared by BG in 2015.

MSFD Guiding Improvements in the Black Sea Integrated Monitoring System.

See the "Commission Report assessing Member States' monitoring programmes under the Marine Strategy Framework Directive" and the accompanying Commission Staff Working Document (http://ec.europa.eu/environment/marine/eu-coast-and-marine-policy/implementation/reports_en.htm).

See Annex 3 to the Interim Report of July 2016.

A new convergence analysis was not conducted. Instead, the results of Phases I and II (ARCADIS) were used.

GES on Descriptor 2 is defined only in terms of planktonic communities: numbers and biomass of invasive alien species below the thresholds for GES without measurable effects on native species, communities and habitats and ecosystem functions; this is considered inadequate and requires improvement.

These documents are considered draft as they still need to go through public consultation and the Council and Parliament. These approved draft versions of Annex III and the Commission Decision are used as the basis for the criteria, activities, pressures and impacts in this Roof Report.

Zaitsev Yu. and Mamaev V.O., Biological diversity in the Black Sea: A study of change and decline, Black Sea Environmental Series, Vol. 3, United Nations Publishing, New York, 1997, p. 5; European Environment Agency (EEA), Europe's biodiversity - biogeographical regions and seas: The Black Sea - an oxygen-poor sea; EEA Report No 1/2002, p. 4.

Sources: European Environment Agency (EEA) at: http://www.eea.europa.eu/data-and-maps/figures/physiography-of-black-sea-biographical-region and Wikimedia Common at: https://commons.wikimedia.org/wiki/File:Black_Sea_map.png.

Sources: Website of the Black Sea Commission (BSC) at: http://www.blacksea-commission.org/_geography.asp; BSC, 2002. State of the Environment of the Black Sea: Pressures and Trends 1996-2000 at: http://www.blacksea-commission.org/_publ-SOE2002-eng.asp#_Toc39914629; Overview of the Black Sea region on the UNEP RSP website at: http://www.unep.org/regionalseas/programmes/nonunep/blacksea/instruments/r_profile_bs.pdf and European Environment Agency (EEA), Europe's biodiversity - biogeographical regions and seas: The Black Sea - an oxygen-poor sea; EEA Report No 1/2002.

Sources: Website of the Black Sea Commission (BSC) at: http://www.blacksea-commission.org/_geography.asp; BSC, 2002. State of the Environment of the Black Sea: Pressures and Trends 1996-2000 at: http://www.blacksea-commission.org/_publ-SOE2002-eng.asp#_Toc39914629; Overview of the Black Sea region on the UNEP RSP website at: http://www.unep.org/regionalseas/programmes/nonunep/blacksea/instruments/r_profile_bs.pdf and European Environment Agency (EEA), Europe's biodiversity - biogeographical regions and seas: The Black Sea - an oxygen-poor sea; EEA Report No 1/2002.

Protocol on the Protection of the Marine Environment of the Black Sea from Land-based Sources (1992 LBS Protocol), Protocol on Co-operation in Combating Pollution of the Black Sea marine Environment by Oil and other harmful Substances in Emergency Situations (1992 Emergency Protocol), Protocol on the Protection of the Black Sea marine Environment against Pollution by Dumping (1992 Dumping Protocol) and the Black Sea Biodiversity and Landscape Conservation Protocol (2002 CBD Protocol).

Protocol on the Protection of the Marine Environment of the Black Sea from Land-Based Sources and Activities (2009 LBSA Protocol).

BSC, 2007. Black Sea Transboundary Diagnostics Analysis; (in the following also: BS TDA 2007).

Id., p. 50. Notably, its predecessor, the first Black Sea Transboundary Diagnostic Analysis 1996, identified seven environmental problems, which were later narrowed down in order to tackle the environmental problems of the Black Sea more effectively (Compare BSC, 1996. Black Sea Transboundary Diagnostics Analysis on UNEP GRID website at: http://www.grid.unep.ch/bsein/tda/main.htm, accessed August 2016).

Report from the Commission to the Council and the European Parliament - The first phase of implementation of the Marine Strategy Framework Directive (2008/56/EC) - The European Commission's assessment and guidance (COM(2014)097 final, 20.2.2014)

For more specific information and information on "local" (i.e. only national) activities, pressures and impacts, please see the national reporting.

Keremedchiev and Stancheva (2006), in: Stancheva, M., 2013. Bulgaria. In: Pranzini and Williams (eds), Coastal erosion and protection in Europe, Routledge, Oxon, pp. 378-395.

http://ec.europa.eu/maritimeaffairs/atlas/maritime_atlas/#lang=EN;p=w;pos=30.058:43.921:8;bkgd=20:0.49;gra=0;mode=1;theme=73:0.8:1:0,21:1:1:0 ;

https://ec.europa.eu/fisheries/sites/fisheries/files/docs/body/op-romania-fact-sheet_en.pdf

https://ec.europa.eu/fisheries/sites/fisheries/files/docs/body/op-bulgaria-fact-sheet_en.pdf

http://www.eea.europa.eu/data-and-maps/figures/agricultural-land-use-within-coastal

http://ec.europa.eu/eurostat/statistics-explained/index.php/Maritime_ports_freight_and_passenger_statistics

http://ec.europa.eu/eurostat/statistics-explained/index.php/Maritime_ports_freight_and_passenger_statistics

http://ec.europa.eu/eurostat/statistics-explained/index.php/Archive:Mediterranean_and_Black_Sea_coastal_region_statistics

http://www.eea.europa.eu/themes/water/status-and-monitoring/state-of-bathing-water

Environmental Quality Standards (2013/39/EU).

Coastal fish and habitats are not confined to coastal waters, but are ecologically defined.