Explanatory Documentation of the 
EAGLE Concept

-  Version 3.2  -

Prepared by:

Stephan Arnold, Barbara Kosztra, Gebhard Banko, 
Pavel Milenov, Geoff Smith, Gerard Hazeu

Revised by:

Stephan Arnold, Geoff Smith, Michael Bock, Gerard Hazeu, 
Barbara Kosztra, Christoph Perger, Mario Caetano

Date:

30. March 2023

Document History:

- Version 2.3 was finalized in December 2014.

- Version 2.3.1 contains minor correction of errata (typos, chapter numbers), published in December 2016.

- Version 2.3.2 continued to correct other minor errata (typos, chapter numbers), without substantial change of content, published in May 2020.
[Tree height threshold was set from 8 to 5 m.]

- Version 3.1 contains substantial changes, mainly in the LCH block. New elements have been added, others replaced or partly renamed. Grouping of matrix elements into modules and segments. The order of appearance in the text chapters follow the structures of the Land Cover Components and Land Use Attributes they relate to; firstly Built-up Characteristics, secondly Vegetation Characteristics, followed by Water Characteristics. It continues with Agricultural Land Management, Forestry Land Management and Mining Details, just as these themes also appear under the LUA block. Afterwards, all other Characteristics follow, which may apply to any LCC or LUA.

- Version 3.1.1 contains some minor corrections and enhancements of version 3.1. The Crop Type list has been added in the documentation, which was before only represented by a placeholder heading, without explicitly listing all crop types. In the matrix however, no change was made, as the Crop Type entries have been part of it already.

- Version 3.1.2 contains corrections of the numbering of listed elements corresponding to the hierarchical numbering in matrix, that occurred as typo errors during formatting of documentation. “Back to top” hyperlinks have been added under each paragraph.

Land Use Attributes have been extended on 3rd level under Secondary production sector; subtypes under forestry land use and mining land use have been removed, because they are better placed and further described under III. LCH matrix block. Some more LCHs have been added.

New EAGLE matrix block “IV. EAGLE Metadata” has been added, metadata characteristics have been moved there from III. LCH block.

Version 3.2 received a review of the crop types, eventual cultivation purposes have been removed from crop types and moved to own segment about cultivation purpose. Besides, some more LCH topics have been added/extended: Groundwater, Grazing characters, Forest species types, Parameter value fields.

It is important to state that the EAGLE concept with the data model, the matrix and this explanatory documentation are living documents and do not claim to be completely finalized. They are in the state of constant work in progress. Furthermore, any suggestion for improvement or modification is welcome and will be taken into consideration for future enhancement of the concept.

Legal notice

The content of this publication does not necessarily reflect the official opinions of the European Commission or other institutions of the European Union. Neither the European Environment Agency nor any person or company acting on behalf of the Agency is responsible for the use that may be made of the information contained in this report.

Copyright notice

© EEA, Copenhagen, 2023, Reproduction is authorized, provided the source is acknowledged, save where otherwise stated.

Explanatory Documentation of the EAGLE Concept

CONTENT [overview; detailed view see following pages]

Part A: Introduction to the EAGLE concept	20

Part B: Thematic Content and Definitions of EAGLE Model Elements	32

I. Matrix block: LAND COVER COMPONENTS (LCC)	33

1	Abiotic / Non-Vegetated Surfaces and Objects	33

2	Biotic / Vegetated Surfaces	37

3	Water Surfaces	40

II. Matrix block: LAND USE ATTRIBUTES (LUA)	43

1_Primary Production Sector	43

2_Secondary Production Sector	45

3_Tertiary Service Sector	49

4_Transport Networks, Logistics And Utilities	55

5_Residential Use	58

6_Other Uses	59

III. Matrix block: LAND CHARACTERISTICS (LCH)	60

1	Built-up Characteristics	60

2	Physical Characteristics	68

3	Biotic / Vegetation Characteristics	68

4	Water Characteristics	75

5	Land Management	81

6	Status / Condition	114

7	Geographical Characteristics	120

8	Spatial Characteristics	126

9	Object Characteristics	129

IV. Matrix block: EAGLE METADATA (EMD)	131

1	Feature life cycle	131

2	Database edit dates	131

3	Data Acquisition Date	131

4	Data Source Type	132

5	Geometric specifications	132

References	133

CONTENT [in detail]

Part A: Introduction to the EAGLE concept	20

Scope of EAGLE Concept	20

Background and motivation	20

Given situation	20

Criteria collection	21

Guiding questions	21

Definition of terms	22

Content of EAGLE concept	23

a.	Structuring of the EAGLE Matrix	23

b.	Structuring of the EAGLE Data Model	25

How to use the matrix – The bar coding method	26

Examples for bar code values for CLC classes:	27

Part B: Thematic Content and Definitions of EAGLE Model Elements	32

Structuring of Land Cover Components:	32

Structuring of Land Use Attributes:	32

Structuring of Characteristics:	32

I. Matrix block: LAND COVER COMPONENTS (LCC)	33

1	Abiotic / Non-Vegetated Surfaces and Objects	33

1.1	Artificial Surfaces and Constructions	33

1.1.1	Sealed Artificial Surfaces and Constructions	33

1.1.1.1	Buildings	33

1.1.1.1.1	conventional buildings	33

1.1.1.1.2	Specific buildings	34

1.1.1.2	specific structures and facilities	34

1.1.1.3	open sealed surfaces	34

1.1.2	Non-Sealed Artificial Surfaces	34

1.1.2.1	Open Non-Sealed Artificial Surfaces	34

1.1.2.2	waste materials	35

1.2	Natural Material Surfaces	35

1.2.1	Consolidated Surfaces	35

1.2.1.1	bare rock	35

1.2.1.2	hard pan	35

1.2.2	Un-Consolidated Surfaces	36

1.2.2.1	Mineral Fragments	36

1.2.2.1.1	boulders, stones	36

1.2.2.1.2	pebbles, gravel	36

1.2.2.1.3	sand, grit	36

1.2.2.1.4	clay, silt	36

1.2.2.1.5	mixed unsorted material (moraines, till)	37

1.2.2.2	bare soils	37

1.2.2.3	Natural Deposits	37

1.2.2.3.1	inorganic deposits	37

1.2.2.3.2	organic deposits (peat)	37

2	Biotic / Vegetated Surfaces	37

2.1	Woody Vegetation	37

2.1.1	trees	37

2.1.2	Bushes, Shrubs	38

2.1.2.1	regular bushes	38

2.1.2.2	dwarf shrubs	38

2.2	Herbaceous Vegetation (grass-like, forbs, ferns)	38

2.2.1	Graminoids (grass-like)	38

2.2.1.1	grasses, sedges, rushes, cereals	39

2.2.1.2	reeds, bamboos, canes	39

2.2.2	Non-graminoids (forbs, ferns)	39

2.3	Succulents, Cacti	39

2.4	Lichens, Mosses, Algae	40

2.4.1	lichens	40

2.4.2	mosses	40

2.4.3	Algae	40

2.4.3.1	macro algae (seaweed, kelp)	40

2.4.3.2	micro algae (plankton)	40

3	Water Surfaces	40

3.1	Liquid Water Bodies	40

3.1.1	Inland Water Bodies	41

3.1.1.1	water course	41

3.1.1.2	standing water	41

3.1.2	marine waters	41

3.2	Solid waters	41

3.2.1	snow	41

3.2.2	ice, glaciers	42

II. Matrix block: LAND USE ATTRIBUTES (LUA)	43

1_Primary Production Sector	43

1_1_Agriculture	43

1_1_1_Commercial Agricultural Production	43

1_1_2_Farming Infrastructure (buildings, facilities)	43

1_1_2_1_Animal Husbandry	43

1_1_2_2_Farming Storage	43

1_1_2_3_Other farming infrastructure	44

1_1_3_Agricultural Production For Own Consumption	44

1_2_Forestry	44

1_3_Mining And Quarrying	44

1_4_Aquaculture And Fishing	44

1_4_1_Aquaculture	44

1_4_2_Professional Fishing	44

1_5_Other Primary Production	44

1_5_1_Hunting	45

1_5_2_Management Of Migratory Animals	45

1_5_3_Picking Of Natural Products	45

1_5_4_Apiculture (Bee hives)	45

2_Secondary Production Sector	45

2_1_Manufacturing / producing industry	45

2_1_1_Raw Industry	45

2_1_1_1_Manufacturing Of Textile Products	46

2_1_1_2_Manufacturing Of Wood And Wood Based Products	46

2_1_1_3_Manufacturing Of Pulp, Paper And Paper Products	46

2_1_1_4_Manufacturing Of Coke, Refined Petroleum Products And Nuclear Fuel	46

2_1_1_5_Manufacturing Of Chemicals, Chemical Products, Man-Made Fibers	46

2_1_1_6_Manufacturing Of Basic Metals And Fabricated Metals	46

2_1_1_7_Manufacturing Of Non-Metallic Mineral Products	46

2_1_1_8_Manufacturing Of Rubber Plastic Products	46

2_1_1_9_Manufacturing Of Other Raw Materials	47

2_1_2_Heavy End Product Industry	47

2_1_2_1_Manufacturing Of Machinery	47

2_1_2_2_Manufacturing Of Vehicles And Transport Equipment	47

2_1_2_3_Manufacturing Of Other Heavy End Products	47

2_1_3_Light End Product Industry	47

2_1_3_1_Manufacturing Of Food Beverages And Tobacco Products	47

2_1_3_2_Manufacturing Of Clothes And Leather	47

2_1_3_3_Publishing And Printing	47

2_1_3_4_Manufacturing Of Electrical And Optical Equipment	48

2_1_3_5_Manufacturing Of Other Light End Products	48

2_2_Energy Production	48

2_2_1_Nuclear Based Energy Production	48

2_2_2_Fossil Fuel Based Energy Production	48

2_2_2_1_Coal Lignite Based	48

2_2_2_2_Mineral Oil Based	48

2_2_2_3_Mineral Gas Based	48

2_2_2_4_Peat And Other Fossil Fuel Based	48

2_2_3_Biomass Based Energy Production	48

2_2_4_Renewable Energy	49

2_2_4_1_Water Energy	49

2_2_4_2_Solar Energy	49

2_2_4_3_Wind Energy	49

2_2_4_4_Geo-Thermal Energy	49

2_2_4_5_Tidal Wave Energy	49

2_2_5_Waste Combustion Energy	49

2_5_Other Industry	49

3_Tertiary Service Sector	49

3_1_Commercial Services	50

3_1_1_Wholesale And Retail Trade, Repair Of Vehicles And Personal And Household Goods	50

3_1_2_Real Estate Services	50

3_1_3_Accommodation And Food Services	50

3_1_4_Other Commercial Services	50

3_2_Financial Professional And Information Services	50

3_2_1_FinancialAndInsuranceServices	50

3_2_2_Professional Technical And Scientific Services	50

3_2_3_Information And Communication Services	51

3_2_4_Administrative And Support Services	51

3_2_5_Other Financial Professional And Information Services	51

3_3_Community Services	51

3_3_1_Public Administration Defense And Social Security Services	51

3_3_2_Educational Services	51

3_3_3_Health And Social Services	51

3_3_4_Religious Services	52

3_3_4_1_House Of Worship	52

3_3_4_2_Monastery	52

3_3_4_3_Cemetery	52

3_3_5_Other Community Services	52

3_4_Cultural Entertainment And Recreational Services	52

3_4_1_Cultural Services	52

3_4_1_1_Indoor Cultural Services	52

3_4_1_2_Outdoor Cultural Services	52

3_4_2_Entertainment Services	53

3_4_3_Sports Infrastructure	53

3_4_3_1_Golf Course	53

3_4_3_2_Ski Piste	53

3_4_3_3_Outdoor Racing Course	53

3_4_3_4_Sport Hall	53

3_4_3_5_Stadium	53

3_4_3_6_Swiming Pool	53

3_4_3_7_Sports Ground	54

3_4_3_8_Indoor sports or fitness facilities	54

3_4_3_9_Yacht harbor, sport boat marina	54

3_4_4_Open Air Recreational Areas	54

3_4_4_1_Urban Greenery And Parks	54

3_4_4_2_Semi Natural Areas And National Parks	54

3_4_5_Other Recreational Services	54

3_4_5_1_Allotment Garden	54

3_4_5_2_Amateur Fishing	55

3_5_Other Services	55

4_Transport Networks, Logistics And Utilities	55

4_1_Transport Networks	55

4_1_1_Road Transport	55

4_1_2_Railway Transport	55

4_1_3_Air Transport	55

4_1_4_Water Transport	55

4_1_5_Other Transport Network	55

4_2_Logistical And Storage Services	56

4_3_Utilities	56

4_3_1_Power Distribution Services	56

4_3_1_1_Electricity Distribution	56

4_3_1_2_Gas Distribution	56

4_3_1_3_Thermal Energy Distribution	56

4_3_2_Water And Sewage Infrastructure	56

4_3_2_1_Drinking Water Facility	56

4_3_2_2_Sewage Water And Treatment Facility	56

4_3_2_3_Water Run-Off Retention Basin	57

4_3_2_4_Artificial Snow Water Pond	57

4_3_2_5_Irrigation Or Fire Fighting Water Pond	57

4_3_3_Waste Treatment	57

4_3_3_1_Dump Site	57

4_3_3_1_1_hazardous waste	57

4_3_3_1_2_inert (non-hazardous) waste	57

4_3_3_2_Recycling Facility	57

4_3_4_Other Utilities	58

5_Residential Use	58

5_1_Permanent Residential Use	58

5_2_Residential Use With Other Compatible Uses	58

5_3_Other Residential Use	58

5_3_1_Temporary Residential, Permanent Structure	58

5_3_2_Temporary Dwelling, Non-permanent Structure	58

5_3_3_Informal Dwelling	59

6_Other Uses	59

6_1_Transitional Areas	59

6_3_Natural Areas Not In Other Economic Use	59

6_3_1_Land Areas Not In Other Economic Use	59

6_3_2_Water Areas Not In Other Economic Use	59

6_4_Flood Protection	59

III. Matrix block: LAND CHARACTERISTICS (LCH)	60

1	Built-up Characteristics	60

1.1	Soil sealing degree	60

1.2	Built-up Pattern	60

1.2.1	scattered single houses, discontinuous	60

1.2.2	single blocks, discontinuous	60

1.2.3	suburban row houses/terraced/semi-detached houses	60

1.2.4	city street blocks, closed front	61

1.2.5	large complex buildings, big halls	61

1.3	Building Nature Type	61

1.3.1	Building Nature Type Value	61

1.4	Other Construction Nature Type	61

1.4.1	Other Construction Nature Type Value	61

1.5	Artificial Surface Material Type	62

1.5.1	Mineral Compound	62

1.5.1.1	glass	62

1.5.1.2	ceramic (clay tiles, bricks)	62

1.5.1.3	concrete	62

1.5.1.4	asbestos fibre cement	62

1.5.1.5	non-asbestos fibre cement	62

1.5.2	Solar Panels (silicium based)	62

1.5.3	Metal	62

1.5.3.1	Copper	62

1.5.3.2	Zinc	63

1.5.3.3	Aluminium	63

1.5.3.4	Steel	63

1.5.3.5	Iron	63

1.5.3.6	Lead	63

1.5.4	Hydrocarbon Compound (Crude oil based)	63

1.5.4.1	Tar	63

1.5.4.2	Asphalt	63

1.5.4.3	Bitumen	63

1.5.5	Polymer, Plastic, Synthetic Fiber	63

1.5.5.1	hard rubber	63

1.5.5.2	hard plastic	63

1.5.5.3	plastic foil	63

1.5.5.4	synthetic fiber	63

1.5.6	Mineral Material	64

1.5.6.1	natural stone	64

1.5.6.2	slate	64

1.5.6.3	pebbles, gravel	64

1.5.6.4	sand	64

1.5.6.5	ashes, slag	64

1.5.7	Organic	64

1.5.7.1	Wood	64

1.5.7.2	reed/straw/leaves	64

1.6	Artificial Surface Material Arrangement	64

1.6.1	seamless sealing	64

1.6.2	closed pavement	64

1.6.3	permeable paving, grass pavers	65

1.6.4	tiles	65

1.7	Building Greening Type	65

1.7.1	Greened Roof	65

1.7.1.1	Low Growing Roof Vegetation	65

1.7.1.2	High Growing Roof Vegetation	65

1.8	Transportation Network Characteristics	65

1.8.1	Road Network Type	66

1.8.1.1	Fast Transit Road, Highway	66

1.8.1.2	Pedestrian Zone	66

1.8.1.3	Associated Walk- Or Cycleway	66

1.8.1.4	Walk- and Cycleway	66

1.8.1.5	Cycleway	66

1.8.1.6	Walkway	66

1.8.1.7	Open Square	66

1.8.2	placeholder>	66

1.8.3	<placeholder>	67

1.8.4	Harbour Type	67

1.8.4.1	Cargo Port	67

1.8.4.2	Passenger Port	67

1.8.4.3	Fishing Port	67

1.8.4.4	Naval Port	67

1.8.4.5	Marina	67

1.8.4.6	Local Multifunctional Harbour	67

1.8.4.7	Shipyard	67

1.8.4.8	Jetty	68

2	Physical Characteristics	68

2.1	Permafrost zone	68

3	Biotic / Vegetation Characteristics	68

3.1	Leaf form	68

3.1.1	needle leaved	68

3.1.2	broad leaved	68

3.1.3	palm leaved	68

3.1.4	non-leafy	68

3.2	Foliage persistence	68

3.2.1	evergreen	69

3.2.2	deciduous	69

3.2.2.1	winter deciduous	69

3.2.2.2	summer deciduous	69

3.3	Leaf anatomy	69

3.3.1	sclerophyllous	69

3.4	Phenology (Plant life span)	69

3.4.1	annual plant	69

3.4.2	biennial plant	70

3.4.3	perennial plant	70

3.4.4	ephemeral plant	70

3.5	Plant Location	70

3.5.1	terrestrial plant	70

3.5.2	epiphyte plant	70

3.5.3	aquatic submerged plant	71

3.5.4	aquatic emergent plant	71

3.6	Growth Form/Habit	71

3.6.1	erect growth (single stem)	71

3.6.2	open/spreading/dense growth (multi-stem)	71

3.6.3	prostrate (creaping)	71

3.6.4	clump-forming	71

3.6.5	cushion-/mat-forming	71

3.6.6	climbing	71

3.7	Raunkiaer Life Form	71

3.7.1	phanerophyte	72

3.7.2	epiphyte	72

3.7.3	chamaephyte	72

3.7.4	hemicryptophyte	72

3.7.5	cryptophyte	72

3.7.6	therophyte	72

3.7.7	aerophyte	72

3.8	Vegetation cover transition	72

3.8.1	natural succession	72

3.8.2	vegetation degradation	72

3.9	Plant Species Origin	73

3.9.1	native plant	73

3.9.2	non-native plant	73

3.9.3	endemic plant	73

3.9.4	invasive plant	73

3.9.5	migratory plant	73

3.10	Species Type	74

3.10.1	Species	74

3.11	Plant Community Type	74

3.11.1	Plant Community	74

3.12	Growing Season	74

3.12.1	growing season start	74

3.12.2	growing season end	74

3.13	Crown Cover Density	75

4	Water Characteristics	75

4.1	Water Body Formation	75

4.1.1	Natural water body	75

4.1.2	Controlled / regulated / heavily modified water body	75

4.1.3	man-made water body	75

4.2	Water Regime (Hydrological Persistence)	75

4.2.1	dry water regime	76

4.2.2	ephemeral (episodic) water regime	76

4.2.3	intermittent (periodic/seasonal) water regime	76

4.2.4	perennial water regime	76

4.3	Water dynamics	76

4.3.1	running water	76

4.3.2	almost standing water	77

4.3.3	standing water	77

4.4	Moisture regime (Wetness)	77

4.4.1	Aquatic( water at surface)	77

4.4.2	waterlogged/saturated ground	77

4.4.3	wet	77

4.4.4	seasonally wet	78

4.4.5	mesic	78

4.4.6	dry	78

4.4.7	very dry	78

4.4.8	xeric	78

4.4.9	semi-desert	78

4.4.10	desert	78

4.5	Wetness source	78

4.5.1	rainfed	78

4.5.2	surface water	78

4.5.3	subterranean/groundwater	78

4.6	Salinity (Water or Soil)	79

4.6.1	brine	79

4.6.2	saline	79

4.6.3	brackish	79

4.6.4	fresh	79

4.6.5	ultra fresh (rain water)	80

4.7	pH value	80

4.8	Tidal Phenomena	80

4.8.1	tidal influence	80

4.8.2	tidal range	80

4.9	<placeholder>	80

4.10	Snow Characteristics	80

4.10.1	Snow Height	81

4.11	Ice Characteristics	81

5	Land Management	81

5.1	Agricultural Land Management	81

5.1.1	Agricultural Cultivation Form	81

5.1.1.1	cropland	81

5.1.1.1.1	arable crop land	81

5.1.1.1.2	permanent crop land	82

5.1.1.2	managed permanent grassland	82

5.1.2	Cultivation Practice	82

5.1.2.1	crop rotation	82

5.1.2.2	no crop rotation	82

5.1.2.3	plantation	83

5.1.2.4	extensive cultivation practice	83

5.1.2.5	agroforestry	83

5.1.2.6	shifting cultivation (slash&burn)	83

5.1.2.7	intercropping	83

5.1.2.8	kitchen garden	84

5.1.2.9	paddy field cultivation (rice fields)	84

5.1.3	Cultivation Installation	84

5.1.3.1	greenhouse (under glass/ foil)	84

5.1.3.2	under foil (on ground)	84

5.1.3.3	plantation protection net	84

5.1.3.4	espalier / trellis / lattice	85

5.1.3.5	hydroculture	85

5.1.3.6	vertical farming	85

5.1.4	Cultivation Measure	85

5.1.4.1	Ploughing and tillage	85

5.1.4.1.1	no ploughing	85

5.1.4.1.2	ploughing applied, direction unknown	86

5.1.4.1.3	across the slope (parallel to contour lines)	86

5.1.4.1.4	oblique direction (diagonal to slope)	86

5.1.4.1.5	in direction of slope (perpendicular to contour lines)	86

5.1.4.2	Fertilization	86

5.1.4.2.1	no fertilizing	86

5.1.4.2.2	fertilization applied, type unknown	86

5.1.4.2.3	organic fertilizer	86

5.1.4.2.3.1	animal manure	86

5.1.4.2.3.2	green manure	87

5.1.4.2.4	synthetic fertilizer	87

5.1.4.3	Weed Control	87

5.1.4.3.1	no weed control	87

5.1.4.3.2	weed control applied, method unknown	87

5.1.4.3.3	mechanical / biological weed control	87

5.1.4.3.4	chemical weed control	87

5.1.4.4	Pest Control	88

5.1.4.4.1	no pest control	88

5.1.4.4.2	pest control applied, method unknown	88

5.1.4.4.3	mechanical / biological pest control	88

5.1.4.4.4	chemical pest control	88

5.1.4.5	Irrigation	88

5.1.4.5.1	no irrigation	88

5.1.4.5.2	irrigation applied, method unknown	89

5.1.4.5.3	surface irrigation (flooding, channels)	89

5.1.4.5.4	sprinkler irrigation (periodic, mobil)	89

5.1.4.5.5	irrigation carousel	89

5.1.4.5.6	drip irrigation	89

5.1.4.5.7	subsurface drip irrigation	89

5.1.4.5.8	manual irrigation	89

5.1.4.6	Irrigation Water Source	89

5.1.4.6.1	groundwater	90

5.1.4.6.2	reservoir	90

5.1.4.6.3	water course	90

5.1.4.7	Drainage	90

5.1.4.7.1	no drainage	90

5.1.4.7.2	drainage applied, method unknown	90

5.1.4.7.3	open ditch drainage	90

5.1.4.7.4	tube drainage (under surface)	90

5.1.4.7.5	filled ditch drainage	91

5.1.4.8	Mowing	91

5.1.4.8.1	no mowing	91

5.1.4.8.2	mowing applied, frequency unknown	91

5.1.4.8.3	extensive mowing (1 x per year)	91

5.1.4.8.4	medium intensity mowing (2 x per year)	91

5.1.4.8.5	intensive mowing (> 2 x per year)	91

5.1.4.8.6	mowing frequency	92

5.1.4.9	Grazing	92

5.1.4.9.1	no grazing	92

5.1.4.9.2	grazing present, intensity unknown	92

5.1.4.9.3	extensive/ freerange grazing	92

5.1.4.9.4	intensive grazing	92

5.1.4.9.5	grazing intensity	92

5.1.4.10	shrub clearance	92

5.1.4.11	biomass burning	93

5.1.4.12	liming	93

5.1.4.13	pruning	93

5.1.4.14	set aside	93

5.1.5	Cultivation Purpose	93

5.1.5.1	alimentary crop production	94

5.1.5.2	fodder crop production	94

5.1.5.3	industrial crop production	94

5.1.5.4	energy crop production	94

5.1.5.5	ornamental/ functional plants production	94

5.1.5.6	multiple purpose	94

5.1.6	Crop Type	94

5.1.6.1	Arable crops	95

5.1.6.1.1	Cereals for the production of grain	95

5.1.6.1.1.1	common wheat and spelt	95

5.1.6.1.1.2	durum wheat	95

5.1.6.1.1.3	rye	95

5.1.6.1.1.4	barley	95

5.1.6.1.1.5	oats	95

5.1.6.1.1.6	grain maize	95

5.1.6.1.1.7	rice	95

5.1.6.1.1.8	other cereals for the production of grain	95

5.1.6.1.2	Dried pulses and protein crops for the production of grain	95

5.1.6.1.2.1	peas, field beans and sweet lupins	95

5.1.6.1.2.2	other dry pulses	95

5.1.6.1.3	potatoes	95

5.1.6.1.4	sugar beet	95

5.1.6.1.5	fodder roots and brassicas	96

5.1.6.1.6	Industrial crops	96

5.1.6.1.6.1	tobacco	96

5.1.6.1.6.2	hops	96

5.1.6.1.6.3	cotton	96

5.1.6.1.6.4	rape and turnip rape	96

5.1.6.1.6.5	sunflower	96

5.1.6.1.6.6	soya	96

5.1.6.1.6.7	linseed (oil flax)	96

5.1.6.1.6.8	other oil seed crops	96

5.1.6.1.6.9	flax	96

5.1.6.1.6.10	hemp	96

5.1.6.1.6.11	other fibre crops	96

5.1.6.1.6.12	aromatic plants, medicinal and culinary plants	96

5.1.6.1.6.13	other industrial crops not mentioned elsewhere	96

5.1.6.1.7	Fresh vegetables, melons and strawberries	96

5.1.6.1.8	Flowers and ornamental plants	97

5.1.6.1.9	Plants harvested green	97

5.1.6.1.9.1	Temporary grasses and grazings	97

5.1.6.1.9.2	leguminous plants	97

5.1.6.1.9.3	green maize	97

5.1.6.1.9.4	cereals harvested green (excl. maize)	97

5.1.6.1.9.5	other plants harvested green	97

5.1.6.1.10	arable land seed and seedlings	97

5.1.6.1.11	other arable land crops	97

5.1.6.1.12	fallow land (not crop)	97

5.1.6.1.13	kitchen gardens	97

5.1.6.2	Pasture / meadow	97

5.1.6.3	Permanent crops	97

5.1.6.3.1	Fruit and berry plantations	97

5.1.6.3.1.1	Fruit of temperate climate zones	97

5.1.6.3.1.2	Fruit of subtropical climate zones	98

5.1.6.3.1.3	Berry species	98

5.1.6.3.1.4	Nuts	98

5.1.6.3.2	Citrus plantations	98

5.1.6.3.3	Olive plantations	98

5.1.6.3.3.1	Normally producing table olives	98

5.1.6.3.3.2	Normally producing olives for oil production	98

5.1.6.3.4	Vineyards	98

5.1.6.3.5	Nurseries	98

5.1.6.3.6	Other permanent crops	98

5.1.6.4	Mushrooms, energy crops and genetically modified crops	98

5.1.6.4.1	Mushrooms	98

5.1.6.4.2	energy crops	98

5.1.6.4.3	genetically modified crops	98

5.1.6.5	Aquatic crops	98

5.1.6.5.1	Algae crops	99

5.1.6.6	Unknown crop	99

5.1.6.7	crop residues/ harvested field	99

5.1.7	Plant growth stage	99

5.1.7.1	germination (sprouting)	99

5.1.7.2	vegetative phase (leaf growth, shooting, tillering)	99

5.1.7.3	reproductive phase (booting, heading, flowering)	99

5.1.7.4	ripening phase (fruit development, maturation, senescence)	99

5.2	Forest Land Management	99

5.2.1	Forest Age Structure	100

5.2.1.1	even-aged stands	100

5.2.1.2	uneven-aged stands	100

5.2.1.3	all-aged stands (old-growth forest)	100

5.2.2	Forest Age Stage	100

5.2.2.1	seedlings	100

5.2.2.2	young / mid-aged trees	101

5.2.2.3	grown-up / mature trees	101

5.2.3	Forest Composition	101

5.2.3.1	mixed forest stands	101

5.2.3.2	unmixed forest stands	101

5.2.4	Forestry Practice	101

5.2.4.1	coppice / Low forest	101

5.2.4.2	coppice-standard combination	102

5.2.4.3	high forest	102

5.2.5	Forestry Rotation System	102

5.2.5.1	short rotation coppice (SRC) plantation	102

5.2.5.2	short rotation forestry (SRF) plantation	102

5.2.5.3	intermediate / long rotation forestry	102

5.2.5.4	continuous cover forestry (CCF)	103

5.2.6	Forest Rejuvenation	103

5.2.6.1	seeded / planted	103

5.2.6.2	natural forest succession	103

5.2.6.2.1	self-seeded succession	103

5.2.6.2.2	re-sprouting	103

5.2.7	Silvicultural Measure	103

5.2.7.1	Forest cleaning / weeding	104

5.2.7.2	Forest thinning	104

5.2.7.3	Forest pruning	104

5.2.7.4	ring-barking / girdling	104

5.2.7.5	interplanting	104

5.2.7.6	dead wood removal	105

5.2.7.7	forest irrigation	105

5.2.7.8	forest drainage	105

5.2.7.9	forest liming	105

5.2.7.10	forest firebreak	105

5.2.8	Silvicultural Harvest and Regeneration Method	105

5.2.8.1	clearcutting	105

5.2.8.2	shelterwood cutting	105

5.2.8.3	femel cutting	106

5.2.8.4	strip cutting	106

5.2.8.5	selection cutting	106

5.2.8.6	coppicing	107

5.2.8.7	pollarding	107

5.2.9	Forest Product Type	107

5.2.9.1	Wood Product	107

5.2.9.1.1	timber wood	107

5.2.9.1.2	energy wood	107

5.2.9.1.3	wood fibre	108

5.2.9.2	Non-wood Product	108

5.2.9.2.1	cork	108

5.2.9.2.2	resin	108

5.2.10	Forest History Type	108

5.2.10.1	primary / virgin forest	108

5.2.10.2	naturally regenerated forest (secondary forest)	109

5.2.10.3	reforestation	109

5.2.10.4	afforestation	109

5.3	Industrial Land Management	109

5.3.1	Mining Technique	109

5.3.1.1	Surface Open Pit Mining	109

5.3.1.2	Underground Mining	109

5.3.1.3	Under Water Mining	110

5.3.1.4	Saline Extraction Site	110

5.3.1.5	Leaching/Solution Mining	110

5.3.1.6	Fracking	110

5.3.2	Mining Product Type	110

5.3.2.1	Fossil Hydrocarbon Fuels	110

5.3.2.1.1	Coal Lignite	110

5.3.2.1.2	Crude Oil	110

5.3.2.1.3	Oil Sands	110

5.3.2.1.4	Mineral Gas	110

5.3.2.1.5	Peat	110

5.3.2.2	Metal Ores	110

5.3.2.2.1	Iron Ores	111

5.3.2.2.2	Nonferrous Metal Ores	111

5.3.2.3	Stones Earths	111

5.3.2.4	Chemical Minerals	111

5.3.2.5	Salt	111

5.3.2.6	Other Mining Products	111

5.3.3	End Use Potential Type	111

5.3.3.1	End Use Potential Type Value	111

5.3.4	Fossil Fuel Type	111

5.3.4.1	Fossil Fuel Type Value	111

5.4	Terrain Modification and Land Engineering Measure	111

5.4.1	terraced terrain	111

5.4.2	artificial terrain modification	112

5.4.3	artificial snow preparation	112

5.4.4	avalanche prevention or protection installation	112

5.5	Administrative regulations / Use constraints	112

5.5.1	Access restriction	112

5.5.1.1	civil / open	112

5.5.1.2	civil and military	112

5.5.1.3	temporary restricted	112

5.5.1.4	military only / restricted	113

5.5.2	Protected area type	113

5.5.2.1	nature conservation area	113

5.5.2.2	landscape conservation area	113

5.5.2.3	Natura2000	113

5.5.2.4	Birds retrieval area	113

5.5.2.5	Biosphere reserve	113

5.5.2.6	Geopark	114

5.5.2.7	National Park	114

5.5.2.8	other protected area types	114

6	Status / Condition	114

6.1	Land use status	114

6.1.1	under construction	114

6.1.2	not in use (never been)	115

6.1.3	out of use (temporary)	115

6.1.4	abandoned	115

6.1.5	clear cut	115

6.1.6	collapsed / destroyed / damaged	115

6.1.7	managed nature restoration	115

6.2	Damage Reason	115

6.2.1	Geological or Hydrological	115

6.2.1.1	tsunami	116

6.2.1.2	volcanic eruption	116

6.2.1.3	earthquake	116

6.2.1.4	subsidence and collapse	116

6.2.1.5	landslide	116

6.2.1.6	snow avalanche	116

6.2.1.7	flood	117

6.2.2	Meteorological or Climatological	117

6.2.2.1	drought	117

6.2.2.2	extreme temperature (heat / frost)	117

6.2.2.3	storm damage (Tornados, Hurricanes, strong Winds)	117

6.2.2.4	lightning	117

6.2.2.5	storm surge	117

6.2.3	Fire	118

6.2.3.1	forest fire, wildfire	118

6.2.3.2	underground fire	118

6.2.4	Biological	118

6.2.4.1	insect infestation	118

6.2.4.2	mould infestation	118

6.2.4.3	epidemic	118

6.2.4.4	allergens	118

6.2.4.5	animal browsing / overgrazing	118

6.2.4.6	animal trampling	119

6.2.5	Cosmic	119

6.2.5.1	meteorite impact	119

6.2.5.2	magnetic disruption	119

6.2.5.3	solar and cosmic radiation	119

6.2.6	Contamination or pollution	119

6.2.6.1	radioactive	119

6.2.6.2	chemical / toxic	119

6.2.6.3	soil salinization	119

6.2.6.4	oil contaminated	119

6.2.6.5	explosives (military ammunition / industrial)	120

6.2.6.6	plastic (micro & macro)	120

6.2.7	Intentional / Accidental Structural Damage	120

6.2.7.1	structural decay	120

6.2.7.2	demolition, explosion	120

6.3	Degenerative process	120

6.3.1	deforestation	120

6.3.2	soil erosion	120

6.3.3	desertification	120

6.3.4	glacier melting	120

7	Geographical Characteristics	120

7.1	Habitat Context	120

7.1.1	Marine Habitats	121

7.1.2	Coastal Habitats	121

7.1.3	Inland Surface Waters	121

7.1.4	Mires, Bogs and Fens	121

7.1.5	Grasslands and Lands Dominated By Forbs, Mosses Or Lichens	121

7.1.6	Heathland, Scrub and Tundra	121

7.1.7	Woodland, Forest and Other Wooded Land	121

7.1.8	Inland Unvegetated Or Sparsely Vegetated Habitats	121

7.1.9	Regularly Or Recently Cultivated Agricultural, Horticultural and Domestic Habitats	121

7.1.10	Constructed, Industrial and Other Artificial Habitats	121

7.1.11	Habitat Complexes	122

7.1.12	Lagoon	122

7.1.13	Estuary	122

7.1.14	Marine Inlets, Fjords	122

7.1.15	Coastal Salt Marsh	122

7.1.16	Intertidal Flat	122

7.2	Height Zone	122

7.2.1	planar	122

7.2.2	collin	122

7.2.3	submontane	122

7.2.4	montane	122

7.2.5	high montane	123

7.2.6	subalpine	123

7.2.7	alpine	123

7.2.8	nivel	123

7.3	Geographical Context	123

7.3.1	inland	123

7.3.2	coastal	123

7.3.3	island	123

7.3.4	oceanic	124

7.3.5	urban	124

7.3.6	rural	124

7.3.7	riparian	124

7.3.8	mountain	124

7.4	Geomorphological landform type	124

7.4.1	Geomorphological landform	125

7.5	Littoral Zone	125

7.5.1	Epilittoral	125

7.5.2	Supralittoral	125

7.5.3	Eulittoral/Intertidal	125

7.5.4	Infralittoral/Sublittoral	126

8	Spatial Characteristics	126

8.1	Spatial Distribution Patterns	126

8.1.1	Homogenous (evenly textured)	126

8.1.2	Heterogenous / mixed (unevenly textured)	126

8.1.3	Mosaic (clearly distinct patches)	127

8.1.4	Scattered (sporadic texture)	127

8.2	Linear feature patterns	127

8.2.1	hedge rows	127

8.2.2	rows of trees	127

8.2.3	stone walls	128

8.3	Linear (technical) Networks	128

8.3.1	roads / railways	128

8.3.2	pipelines	128

8.4	Vertical position	128

8.4.1	on ground surface	128

8.4.2	suspended / elevated	128

8.4.3	under ground	128

9	Object Characteristics	129

9.1	Geometric parameters	129

9.1.1	area size	129

9.1.2	length	129

9.1.3	width	129

9.1.4	height	129

9.1.5	area coverage	129

9.2	Occurrence Type	129

9.2.1	Percentage occurrence	129

9.2.2	Countable occurrence	129

9.2.3	Present occurrence	130

9.3	Temporal Parameters	130

9.3.1	Instant event date	130

9.3.2	duration	130

9.3.3	period	130

9.3.4	recurring frequency	130

IV. Matrix block: EAGLE METADATA (EMD)	131

1	Feature life cycle	131

1.1	BeginLifespan	131

1.2	EndLifespan	131

2	Database edit dates	131

2.1	Entered in DB	131

2.2	Last Modified in DB	131

2.3	Deleted from DB	131

3	Data Acquisition Date	131

4	Data Source Type	132

4.1	Satellite imagery	132

4.2	Air-borne Sensor	132

4.3	In-Situ data collection	132

5	Geometric specifications	132

5.1	Minimum Mapping Unit (MMU)	132

5.2	Minimum Mapping Width (MMW)	132

References	133

# Part A: Introduction to the EAGLE concept

## Scope of EAGLE Concept

The EAGLE concept embodies the EAGLE matrix and the EAGLE data model as the two outcomes of the EAGLE groups work, as well as the conceptual idea of its application. To help the user understanding the concept, this explanatory documentation has been written.

This documentation consists of two main parts:

- introduction to the topic with scope and background, brief explanation of how to use the matrix and the relation of the EAGLE concept to other existing standards and nomenclatures.

- hierarchically structured explanations and definitions of the Land Cover Components, the Land Use Attributes and the Land Characteristics.

## Background and motivation

According to the broad field of applications of land cover (LC) and land use (LU) information there are many existing classification systems and nomenclatures. Each of them is addressing the themes LC and LU according to their purposes with partly different definitions and thematic focus. Land cover and land use are strongly interconnected and influence each other, and for many applications both land cover and land use information is needed. Therefore most of the existing classification systems contain to some extent a mixture of land cover and land use class definitions. An overall tendency shows on the one hand a focus on land cover information e.g. in the vegetation area, and on the other hand another focus on land use information in the anthropogenic areas, all occurring within the one and same classification system. So far, it has always been difficult to clearly separate strictly LC from LU information among the existing approaches.

For recent and future land monitoring activities and initiatives it is important to be able to have both a pure land cover and pure land use perspective on landscape separately from each other, as well as being able to re-combine those two themes.

## Given situation

The concept and nomenclature of CORINE Land Cover (CLC) has established itself as the quasi-standard for LCLU mapping in Europe since 1990. Over time however, the technical circumstances including quality and revisit intervals of satellite imagery, data storage capacities, computing power, and analytical methodologies, as well as thematic requirements and political reporting obligations have evolved and changed. Consequently, the need for a revision of the CLC concept had become evident. In the case of CLC, the most evident  shortcomings are:

- Mixed land cover and land use information;

- Ambiguous description and semantic gaps or overlaps provoke inconsistencies in class definitions;

- Only selective incorporation of temporal aspect;

- Lack of thematic content details (e.g. differentiation of grasslands and wetlands, cultivation practices, parameterized information like imperviousness or crown cover density);

- Missing option for the attribution of spatial objects;

- No flexibility to react on new appearing landscape phenomena (e.g. energy crop plantations, artificial snowmaking or habitat restoration);

- Unequal representation of landscape types from different bio-geographical regions.

## Criteria collection

In response to existent and upcoming monitoring requirements, driven by European legislation and monitoring activities and user needs on European and national level, a list of criteria for a suitable data model has been collated. From the conceptual point of view, the data model for a future European land monitoring system should:

- Be object-oriented and describe landscape by its elementary properties instead of classifying,

- Separate land cover from land use information,

- Describe land cover in a mutually exclusive and comprehensive way,

- Be scale-independent,

- Allow semantic translation between classification systems,

- Be INSPIRE compliant,

- take into account COPERNICUS products,

- Support the bottom-up approach as well as top-down initiatives,

- Store parameterized data such as counts and numerical values,

- Be capable of handling spatial multi-scale dimensions and temporal aspects,

- support backwards compatibility between past time and recent  datasets,

- Be flexible enough to allow the insertion of new elements in the model.

## Guiding questions

A fundamental criterion for the coherent description of landscape is the separation of LC and LU. The development of the EAGLE concept was based on the seed questions:

- What is the ideal way to model the landscape from a separated and re-combinable perspective of LC and LU?

- What kind of land cover information can be captured from bird’s perspective above landscape, mainly with remote sensing data and methods?

- How to integrate further landscape information that is important for land monitoring purposes from other sources of information besides remote sensing to understand the consequences of human interaction with the environment?

- How can an object-oriented modeling approach be applied in the field of land monitoring?

- How to make the data model open for various applications and information sources as well as independent from scale?

- How to maintain backward compatibility to existing historical data sets (e.g. CLC time series)?

To outline the targeted application purpose of the EAGLE concept, three key messages shall be mentioned here:

- The EAGLE matrix aims at being a tool for analytic decomposition of class definitions and for semantic translation between recent or future nomenclatures.

- The EAGLE model offers a conceptual basis for a future harmonized European land monitoring system and is open to be implemented as an object-oriented guideline for mapping and monitoring initiatives.

It is important to state that the EAGLE concept with the data model, the matrix and this explanatory documentation are living documents and do not claim to be completely finalized. They are in the state of constant work in progress. Furthermore, any suggestion for improvement or modification is welcome and will be taken into consideration for future enhancement of the concept.

The EAGLE concept does not represent another classification system but instead a descriptive vehicle for harmonization of LC/LU information supporting both centralized top-down and decentralized bottom-up approaches.

## Definition of terms

For a better understanding of this documentation, at least the terms ‘land cover’ and ‘land use’ shall be briefly defined, how they are understood in the context of the EAGLE concept.

Land cover is seen as the “physical and biological cover of the Earth's surface including artificial surfaces, agricultural areas, forests, (semi-)natural areas, wetlands, water bodies” in the INSPIRE Directive. It is an abstraction of reality as the Earth´s surface is actually populated with landscape elements.

The landscape elements are physical features like buildings, roads, trees, plants, water bodies etc. Inside a unit of land, the combination of these landscape elements together with their (bio-) physical characteristics form the land cover type of that particular unit. However, mapping and describing land cover within a certain classification system is usually different from the mapping of the individual landscape elements and it is concerned with the portrayal of a continuous surface and not with the individual elements that comprise this surface. In this sense, classified land cover types are understood already as an abstraction of the surface.

In terms of the EAGLE concept, the abstracted representations of the real world landscape elements, that are relevant for land cover modeling, are called ‘land cover components’ (LCC). These land cover components appear to have their specific distribution or mixture and spatial extent. In conventional classification systems, typical spatial constellation of LCCs, that seem common in the area of appearance, are given a name by embracing them in land cover classes, which then are mapped according to mapping guidelines.

Land use is defined as the “territory characterized according to its current and future planned functional dimension or socio–economic purpose (e.g. residential, industrial, commercial, agricultural, forestry, recreational)” in the INSPIRE Directive.

Land cover and land use are, however, related and often combined in practical applications. Data sets combining land use and land cover often emphasize land use aspects in intensively used areas (e.g. settlements, croplands) and land cover aspects in extensively used areas (e.g. forest, natural vegetation).

Land characteristics are used as a third important term in the context of the here explained concept. They may contain further property information on a particular land unit and specify it in more detail with some other information that cannot be stored neither under “land cover” nor under “land use”. Further explanation on that is given in the following chapter.

### Content of EAGLE concept

The two representations of the EAGLE concept, matrix and model, contain the same information and are based on the same considerations and model elements. According to the application purpose the users can decide to either choose to work with the matrix or with the Unified Modeling Language (UML) model. In this regard, please be aware of the different versions that so far has been released, when comparing matrix with UML model.

### Structuring of the EAGLE Matrix

In the EAGLE concept, the basis for the description of landscape is the land cover components. The list of land cover components is intended to be exhaustive and mutually exclusive.

Land cover, land use and other characteristic descriptors can - and mostly must - be used in flexible combination with each other. These combinations attached to a certain class from one classification system can then be compared with the componential description of a class from another classification system.

The EAGLE matrix itself is presented in the form of an Excel cross table and is subdivided into three content-related main blocks. The matrix elements (arranged in columns) represent atomic landscape descriptors of

- LAND COVER Components – LCC, (e.g. trees, bushes, bare soils)

- LAND USE Attributes – LUA, (e.g. agriculture, forestry, mining, residential areas)

- LAND CHARACTERISTICS - LCH (e.g. land management type, spatial pattern, (bio)-physical characteristics, geographical context, geometric or temporal parameters, ecosystems types, status).

- EAGLE Metadata - EMD

The subdividing of the matrix into those three content blocks gives room for flexibility to add / take out / modify some elements of a matrix block (e.g. restructure the “cultivation measures” part under “land management” in the LCH block, without the need to adapt other parts of the matrix (e.g. re-arrange also agricultural land use).

The forth block is there to collect some metadata about the codified datasets or objects. The elements in this matrix block are not predefined but only placeholders for the specific metadata information that come with the data of origin.

Figure 1: Schematic illustration of the EAGLE matrix with the Land Cover Components (LCC), the Land Use Attributes (LUA) and the Characteristics (LCH) as columns and the classes of a source nomenclature (here e.g. CLC) to be expressed in EAGLE terminology as lines.

Additionally to the content-related Matrix block, a forth matrix block called “IV. EAGLE Metadata” has been inserted.

The elements placed in that block shall help to store some fundamental metadata about a given dataset, like when an object has been created or deleted in the dataset, and when it has been uploaded in a target database, like the CLC+ CORE infrastructure. Also, the source of information can be named, or the minimum mapping unit which has been applied while mapping an area by creating a certain dataset.

In theory, the structure of the matrix and the “architecture” of the UML model allow the LCC to be combined with any kind of LUA or LCH. In practice however, the user has to be aware of the fact that not all combinations of matrix elements make sense. The architecture of the UML model takes this into account in a way that the various LCH to choose from are arranged as attributive attachments with their code lists and code lists to the LCC on different hierarchical levels of the LCC or LC Unit (see below).

To find a common language when speaking about the EAGLE matrix, we suggest to use distinct expressions to address particular parts of the matrix. This help mostly when speaking about the Land Characteristic (LCH) block.

    - Matrix block: the three main blocks of the matrix are the Land Cover Components, the Land Use Attributes and the Land Characteristics. These are the first entry points into the content of the matrix structure

    - Matrix module: a collection of matrix elements that refer to a certain topic or phenomenon (e.g. Status), a main Land Cover Component (e.g. Biotic / Vegetation Characteristics) or a certain Land Use Type (e.g. Agricultural Land Management).

    - Matrix segment: a section of a matrix module that contains a coherent group of elements under a thematic sub-heading (e.g. Water Regime under the module Water Characteristics). The segments may be subdivided into sub-segments (sub-headings).

    - Matrix element: the lowest but most important level of the matrix, where the single values are listed to store the characteristic information of an object in particular. All entries (LCCs, LUAs, LCHs) taken together form the entire matrix (and model respectively).

Depending on the complexity of the matrix modules, they are subdivided into sometimes many, sometimes few segments, sub-segments and elements. Therefore the information carrier elements will not always be placed on the same hierarchical level throughout the entire matrix, but the “depth” of bottom level may vary from segment to segment.

In Block I. (LCC) and Block II: (LUA), all elements follow a hierarchical order, and are subdivided into more details subtypes. No headings exist, every element can be chosen on every level to describe a land unit regarding existing LCCs  (e.g. level 1 LCC vegetation, level 2 LCC woody vegetation, level 3 LCC trees) and LUAs (level 1 LUA industries, level 2 LUA mining, level 3 LUA under water mining). In the LCH block it is different, as the higher leveled matrix entries are basically headings (modules), followed by sub-heading (segments), which can´t be chosen to characterize a Land Unit, but firstly to give structure to the matrix elements. Therefore some intermediate level entry does not make sense to consider as information carrier (e.g. “spatial pattern”), but only the particular type of spatial pattern one level further down  called e.g. “mosaic pattern”). In exceptional cases – e.g. in the segment of crop types, also elements above the bottom level can be used (e.g. arable crops as a group of cereals) to further characterize a given LCC or LUA .

### Structuring of the EAGLE Data Model

The EAGLE data model is available as a UML (Unified Modeling Language) class diagram that is following the ISO standard 19109 (Geographic information - Rules for application schema).

The object-oriented data model is designed to be plugged in the data model as specified in the INSPIRE Directive Annex II - Land Cover data specification where a land cover data set consists of a collection of land cover units. At each land cover unit, the land cover has been observed on one or more observation dates. The extension of the EAGLE data model starts at the land cover unit (LCU) level where each unit contains one to many land cover components (LCCs) (simplified situation).

The land cover unit is described as a discrete geometric feature whereas the land cover component is described as a parametric observation; also several LCCs can occur in combination with each other inside the containing land cover unit. Coming from the extension of the INSPIRE Land Cover specification the parametric observation is meant to store either the presence of a particular land cover type, a countable parameter (i.e. number of trees) or a percentage value indicating the covered area within the enclosed geometry.

The data model is expressed in UML and is visualized in a UML chart. It makes it possible to mirror the Land Cover Components from the matrix as UML elements into three main branches of UML-classes “ABIOTIC”, “VEGETATION/BIOTIC” and “WATER”. Any description of landscape unit or decomposing of a given land cover class (as part of a nomenclature) must start with the selection of one or more Land Cover Components

Figure 2: Simplified graphic on the EAGLE UML data model

The Land Use attributes are strongly related to the existing proposal of the Hierarchical INSPIRE Land Use Classes (HILUCS) and are not included explicitly in the EAGLE UML chart portrayal, but only mentioned through a linked relation. The LUA are attached to the Land Cover Unit.

The Characteristics, which can be chosen to describe further details of the Land Cover Components, are arranged around the Land Cover Components as additional boxes.

Depending on their applicability, they are only valid for specific part of the Land Cover Components and therefore are linked respectively. E.g. the attribute “soil sealing degree” can be applied for any abiotic spatial unit ( - even any biotic in theory), while e.g. the status attribute “clear cut” only relates to vegetation or abiotic natural surfaces materials. Other characteristics like “spatial landscape patterns” could be used to describe a spatial pattern composed by one element or a mosaic of several landscape components in larger scale.

The complete documentation of the UML data model can be found for downloading on the EAGLE web page in a separate document (EAGLE_datamodel_documentation_v2.3.pdf), the data model itself is provided in an Enterprise Architect file (EAGLE_model_v2.3.eap).

## How to use the matrix – The bar coding method

While using the matrix as a decomposing tool and to make up a certain land cove class or land surface unit, it starts with selecting one or more land cover components from the LCC block, continues with attaching some land use information from the LUA block, and is finalized (if needed) by adding further characteristic information on landscape. Targeting at forming a complete and mutually exclusive list, land cover components can describe 100% cover of any land surface unit at any scale, making up a standalone descriptive system.

The matrix can be used in various ways, focusing first on land cover components (LCCs) only:

  - The LCCs can be used simply as a kind of nomenclature, attaching one single LCC to a certain land cover unit, this way categorizing the land cover of that unit in a data model.

  - LCCs can be used in a descriptive way, which gives room for attaching more than one LCC to a certain land cover unit, expressing that on a particular location in landscape where exists more than one land cover component. This application of the matrix goes in-line with the idea of the descriptive approach of the ISO standard 19144-2 (LCML).

  - Use the matrix in a descriptive way like mentioned in b), but more elaborated by not only mentioning more than one LCC to be attached to a certain land cover unit, but also entering a percentage value, indicating the relative fraction and spatial composition of the considered LCCs inside the area extent of a definite land cover unit. This third method of using the matrix is also in line with the concept of the Copernicus Land Monitoring Service (CLMS) raster products High Resolution Layers (HRL).

For both methods a) and b) the encoding of the LCC can be done on a simple, ordinal scale, using a few codes expressing the relevance of the applied matrix element for the given land cover class:

n/a:	The matrix element is not applicable or relevant to the class, being logically excluded, therefore not applicable.

x: The matrix element must not occur in class, being excluded by class definition.

0: The matrix element is insignificant in the class (may still be present due to generalization).

1: The matrix element can be expected in the class but is not a defining mandatory element of the class.

2: The matrix element is a defining obligatory element of the class and must be present – selective OR-function, meaning at least one of tick-marked elements must be present.

3: The matrix element is a defining obligatory element of the class and must be present – cumulative AND-function, meaning if more than one elements are tick-marked, then all of them must be present.

4: The matrix element is a defining obligatory element of the class and must be present - multiple cumulative AND-function, meaning out of the tick-marked elements more than one must be present.

Because the encoding result appears as a sequence of values [n/a, x, 0, 1, 2, 3, 4] it is called “Bar Coding”.

#### Examples for bar code values for CLC classes:

n/a : CLC class 335 (glaciers and perpetual snow) never contains patches of trees (LCC= trees), because they cannot survive there.

X : CLC class 321 (natural grassland) should never include fertilization (CH | Land Management |

Agricultural Management | Cultivation Measures | Fertilization), because class definition excludes it.

- : CLC class 211 (arable land) might contain small tree patches (LCC= trees) due to generalization, but it is not typical of the class.

- : CLC class 112 (discontinuous urban fabric) must contain some kind of vegetation or bare soil, usually also trees (LCC= trees), still there might exist villages without any trees.

- : CLC class 223 (olive groves) must contain trees.

3: CLC class 333 (sparsely vegetated area) must contain both bare natural surfaces (LCC=Natural Material Surfaces) and vegetation (LCC=Biotic/Vegetation)

4: CLC class 242 (complex cultivation patterns) must contain at least two of the three matrix elements from the LCH Crop Types | Arable Crops, Permanent Crops, Permanent Grassland.

For explanation of matrix application option b) (see further above), some examples for combination of LCC, LUA, and LCH to describe real landscape units follow:

- Parcel of woodland that has been partially damaged by storm: LCC: Trees;

LUA: Forestry;

LCH1: Leaf Form = needle leaved; LCH2: Foliage Persistence = evergreen. LCH3: Status | Damage Reason = tornados, hurricanes, strong winds

- Abandoned industrial site:

LCC: specific structures and facilities; LUA: industrial use;

LCH: Status | abandoned

- Intertidal flat:

LCC1: clay, silt; LCC2: sand; LCC3: Marine Waters; LUA: not relevant;

LCH1: Water Characteristics | Tidal Influence = yes; 
LCH2: Water Characteristics |Salinity = saline

Rural village settlement (see Figure 3):

LCC1: Buildings; LCC2: open sealed surfaces; LCC3: Biotic/Vegetation;

LUA1: permanent residential; LUA2: Agriculture | production for own consumption; LCH: Spatial Patterns |Built-up Pattern = scattered single houses, discontinuous.

Figure 3: Example for a rural village. Photo copyright: George Büttner

- Inland wetland (see Figure 4):

LCC1: standing water; LCC2: reeds, bamboos and canes; LUA: natural areas not in economic use;

LCH1: Ecosystem Types = inland marshes; LCH2: Salinity (Water or Soil)= fresh water

Figure 4: Neusiedler See, Austria. Photo copyright: Barbara Kosztra

- Skandinavian heathland (see Figure 5):

LCC1: dwarf shrubs, LCC2: Mineral Fragments = boulders, stones; LCC3: Lichens, Mosses and Algae;

LUA: no economic use;

LCH1: Habitat Types = Heathland, shrub and tundra

Figure 5: Alpine Calluna heath, border between NO and SE. Photo copyright: Norwegian Forest and Landscape Institute.

  - Golf course (see Figure 6):

LCC1: trees, LCC2: regular graminoids, LCC3: standing water; LCC4: Mineral Fragments =

Sand

LUA: Sports Infrastructure | golf course;

LCH1: Spatial Distribution Patterns = mosaic, LCH2: Mowing + LCH3: recurrent frequency =

weekly.

Figure 6: Example for a golf course. Source: http://www.bilyanagolf.com/gloria_golf_club_old_course.php

  - Arable crop land (see Figure 7):

LCC1: graminoids, LCC2: trees; LUA: Commercial agricultural production;

LCH1: Crop Types = wheat; LCH2: Spatial Distribution Patterns = mosaic; LCH3: Cultivation Measures | Irrigation Method = sprinkler (e.g.); LCH4: Cultivation Measures | FertilizatingType = unknown, LCH5: Linear Landscape Patterns = rows of trees

Figure 7: Wheat fields with rows of trees. Foto copyright: George Büttner

# Part B: Thematic Content and Definitions of EAGLE Model Elements

Under this heading follows the main part of the explanatory documentation. All three thematic blocks of the EAGLE model – the Land Cover Components, the Land Use Attributes, and the Characteristics – with their model elements are explained here in part I, part II and part III. Short definitions indicate their semantic content and meaning as well as their role within the data model. For an easier reading and navigation through the model, all elements (LCCs, LUAs, LCHs) are listed in a hierarchically numbered order, following a logical categorization with sub types. The sequence of elements (e.g. code list values) on the same heading level do not indicate any principle of dominance or first choice. The overall sequence of numbering mirrors the arrangement of matrix elements (as they are displayed there in the table columns, reading from left to right).

It is recommended to read the EAGLE matrix or UML chart of data model in parallel with this documentation for better understanding.

## Structuring of Land Cover Components:

The LCC part aims at being mutually exclusive and exhaustive, meaning that any kind of land cover type can be expressed by a LCC or a combination of them. They are ordered and subdivided in a logical and hierarchical manner. Every level of LCC can be used as a modeling element to either semantically describe a class definition or to map landscape (e.g. biotic vegetated areas or woody vegetation or trees).

## Structuring of Land Use Attributes:

The LUA follow in principle the Hierarchical INSPIRE Land Use Classification System (HILUCS). In some cases the HILUCS classes have been extended or modified to fit the purpose of the EAGLE concept, e.g. being able to describe CLC classes with the EAGLE model.

The hierarchical number coding that originates from HILUCS classes is kept here in documentation (in addition the document-internal heading numbers); it shall help to find the corresponding classes in the INSPIRE data specifications of Land Use.

## Structuring of Characteristics:

When referring to single items of groups of matrix elements we the following

- terminology: Blocks are the top Categories (LCC, LUA, LCH).

- Segments are the thematically grouped Elements (e.g. LCH land management, spatial pattern, (bio-)physical characteristics, status etc.) as sub-categories under the three main blocks (in UML terms called code lists and code lists). The segments can be of different hierarchical levels of semantic aggregation or a more detailed differentiation (sub-segments: e.g. LCH cultivation practices, built-up patterns, water regime).

- Elements stand in the lowest hierarchy of the matrix (single column) under the segments, expressing a single type (coded value in UML terms, e.g. crop rotation, mosaic spatial pattern, salt water, burned areas).

# I. Matrix block: LAND COVER COMPONENTS (LCC)

Abiotic / Non-Vegetated Surfaces and Objects

Any unvegetated surfaces, either covered with man-made artificial structures or geologically natural material surfaces (with or without anthropogenic influence or impact).
Back to top

Artificial Surfaces and Constructions

All surfaces where natural landscape has been changed by or is under influence of human construction activities by replacing natural surfaces with artificial 2D/3D constructions or abiotic artificial materials. Artificial parts of urban and rural areas, where mankind has built settlement infrastructures. 
Includes: 
Sealed areas (buildings, other constructions and sealed flat surfaces) and non-sealed areas (no buildings, artificial and unsealed). 
Excludes:
Urban greenery may be artificial and under human maintenance and form part of settlements, but after all it is vegetation and not to be placed here but under LCC-2 Biotic Vegetation. 
Back to top

Sealed Artificial Surfaces and Constructions

In the EAGLE context Sealed Surface is seen as the sole part of space that is covered with artificial constructions like a building or surfaces like a pavement. Built-up areas in a narrow sense (excluding associated non-sealed areas). Sealed Artificial Surface includes therefore all impervious and sealed surfaces that are covered mainly by buildings and artificial constructions (3D) or impervious surfaces (2D). 
Excludes: 
Non-sealed areas associated to the Artificial Surfaces and Constructions.
Back to top

Buildings

Constructions above ground that are intended or used for the shelter of humans, animals, things, the production of economic goods or the delivery of services and that refer to any structure permanently constructed or erected on its site (based on INSPIRE Data Specification Buildings).
Covered by roof, of human origin, made of and covered by artificial material (e.g. concrete, brick, metal) or natural material (e.g. rock, pale, soil/green roof, wood). 
Back to top

conventional buildings

The conventional buildings generally are hosting human activities (residential, industrial, commerce and services) and being of large or medium size (around 15-20 m2 and more). (INSPIRE TWG BU data specifications).

Includes:

e.g. dwelling houses, blocks of flats, city street blocks, stores, supermarkets, office buildings, agricultural buildings, studs, farms, industrial buildings. 
Back to top

Specific buildings

The specific (significant) buildings are the buildings of significant size or height with specific physical aspect that make them usable as landmarks and required by use cases such as mapping or travel safety (INSPIRE TWG BU data specifications).

Includes:

e.g. stadiums, churches, towers, greenhouses (Permanent or temporal installation for crop plantation purposes, mainly with light material like either glass or plastic folia). 
Back to top

specific structures and facilities

Self-standing man-made construction that is not considered a building and has a dimensional extension, and typically stands on mainly sealed ground.

Includes:

e.g. bridges, antennae, cranes, chimneys, city walls, fences, monuments, protective dikes and dams, pylons, transformers, power plants, refineries, water sewage plants, recycling facilities, waste storage facilities, bottom-sealed dumpsite, storage tanks, pipelines, solar panels. 
Back to top

open sealed surfaces

Flat surfaces covered by any type of impervious material that is used for artificial surface pavements (e.g. asphalt, concrete, tarmacadam).
Applicable for:
e.g. paved roads, parking lots, squares, storage areas, airport runways, quays, sealed bottom pools, unvegetated cemeteries.
Excludes:

gravel bed of railway track. It is an artificial surface but not impervious. 
Back to top

Non-Sealed Artificial Surfaces

Any artificial areas which are not sealed with built-up or non-built-up constructions. Non-sealed is here seen as permeable to water. However, non-sealed in this sense can still mean that vegetation is hindered to grow. Applies for areas, where the natural surface has been replaced by artificial material, or for areas covered with natural material which has been taken from its place of origin elsewhere and used here for a man-made non-sealed (non-impervious) and non-built-up artificial surface. Includes also waste materials. 
Back to top

Open Non-Sealed Artificial Surfaces

Any open areas where natural surface material has been replaced by artificial material or natural material removed from its place of origin as result of human activity forming a non-sealed (pervious) and non-built-up (basically flat) surface. Although the surface may be compacted it still can be permeable for water. This component also applies for semi-sealed (semi-pervious) surfaces like e.g. grass pavers.
Applicable for: 
Logistic and storage areas, festive squares, unpaved roads and parking lots, unvegetated sport fields.
Includes:
Surfaces covered by e.g. gravel, pebble, crushed stone, compacted soil, including non-sealed gravel railway tracks, grass pavers. 
Back to top

waste materials

Areas covered by waste materials. Surfaces covered by e.g. household/communal/industrial waste.
Includes:
construction rubble, slag, cinder, metal, wood, dead organic material. 
Back to top

Natural Material Surfaces

Any kind of surface material that remains in its natural consistence or form, either with or without anthropogenic influence. Consolidated and unconsolidated surfaces.

Includes:

unvegetated rocky mountainous regions, sand, quarries and extraction sites, etc. 
Back to top

Consolidated Surfaces

Consolidated Surfaces are natural stone materials of geological origin free of vegetation cover that have continuous solid surface and are less permeable to water compared to other natural surface. As they are natural, this character is not supposed to be given for 100% of such surfaces.
Back to top

bare rock

The rock surface is continuous except perhaps for a few cracks in the material. Some areas may be covered by shallow layers of soil or there could be isolated pockets of soil or a mixture of both. (ISO 19144-2 LCML, ISO/TC 211 N 3265, 2011-12-16, p. 104).

Includes:

e.g. solid (closed) rock formations, fresh lava flows, quarries, mineral extraction sites, open pit mines. 
Back to top

hard pan

Hardpans are particular soil layers or surfaces that have been indurated due to chemical or physical processes. Their hardness at the surface is irreversible. They form impenetrable layers for water and/or plant roots. In the context of LCML, these layers are only described when occurring at the surface (ISO 19144-2 LCML, ISO/TC 211 N 3265, 2011-12-16, p. 104). 
Back to top

Un-Consolidated Surfaces

Any surface with loose mineral particles of any size range, either as outcome of natural physical sedimentation processes or human activity. 
Includes:
E.g. mountain slope debris, glacier moraines, river pebble banks, beaches, sand dunes, gravel bed of rail way tracks. 
Back to top

Mineral Fragments

Mineral Fragments come to be through mainly physical disintegration of geological formations and are the result of becoming smaller and smaller along time. They are accumulated on site due to sedimentary processes or human activity. Further subdivision is made according to their particle size and constellation. 
Back to top

boulders, stones

Big mineral fragments, normally too heavy to be moved or carried a long distance by human strength, can only be moved by gravitative processes, glaciers or heavy floods, or stay on the spot till they further disintegrate. Particle size of ca. 20 to 200 cm or bigger. 
Back to top

pebbles, gravel

Middle sized mineral fragments that are moved and accumulated through erosive and sedimentary processes in geomorphological sense. The particle size ranges between 2 cm and 20 cm.
Includes: 
e.g. mountain slope debris, gravel river banks, open pit pebble mining of fossil river banks or fluvial sediments, volcanic lapilli fields
Back to top

sand, grit

Small and very small mineral fragments, loose and shifting sand and grit, can be moved by aeolian erosion and moderate erosive water currencies. Particle size between 0,06 mm and 2 cm.
Includes: 
e.g. sand dunes, sand beaches, river sand banks, volcanic ash
Back to top

clay, silt

Very small mineral fragments, loose and shifting silt, clay surfaces, can be moved by aeolian erosion and moderate erosive water currencies. Particle size is below 0,06 mm.
Includes: 
e.g. clay and silt flats, beaches, river sand banks, fine volcanic ash. 
Back to top

mixed unsorted material (moraines, till)

Unsorted mineral fragments ranging in size from silt-sized glacial flour to large boulders most frequently unconsolidated debris of glacial origin. 
Includes: 
e.g. any type of moraine formed by contemporary or former glaciers, any unsorted glacial sediment - till
Back to top

bare soils

Mixture of mineral and organic material that is fertile enough and capable of sustaining plant life, but being unvegetated at the moment of observation. 
Back to top

Natural Deposits

Natural deposits are substrates that accumulate either by processes of chemical sedimentation (in combination with evaporation and crystallization) or by incomplete decay of biotic dead matter. 
Back to top

inorganic deposits

Salt, gypsum and other substrates as a residual of water evaporative processes.
Back to top

organic deposits (peat)

Accumulated partially decayed vegetation material (mostly Sphagnum moss, but also other plants) formed in wetland conditions, where lack of oxygen slows down decomposition rate. 
Back to top

Biotic / Vegetated Surfaces

Any vegetated land surface, either naturally grown, semi-natural or artificially planted vegetation (e.g. crops, urban parks), with or without anthropogenic influence. Vegetation is subdivided into further components based on plant life forms i.e. woody, herbaceous, lichens/mosses. 
Back to top

Woody Vegetation

The distinction of woody vegetation into broad leaved plants, needle leaved plants and palm leaf plants can be done by combination with an attribute value of LCH Leaf Form under vegetation with the LCH phenology. Further, the LCH Foliage Persistence can indicate whether a woody plant species is deciduous or evergreen. 
Back to top

trees

Perennial woody plant with single, self-supporting main stem or trunk, containing woody tissue and branching into smaller branches and shoots.

Includes:

Needle-leaved, broad-leaved and palm-leaved forestry species, fruit trees, ornamental trees.

Excludes:

Giant bamboo.
Back to top

Bushes, Shrubs

Perennial woody plants with shrub growth form i.e. multiple stems arising at or near the base, height usually less than 5 meters. Leaf type can be needle leaf, broadleaf or palm leaf, phenology either evergreen or deciduous, leaf surface type can be regular or sclerophyllous.

Includes:

wild-growing and cultivated (agricultural, ornamental) species.
Back to top

regular bushes

Perennial woody plants with multiple stems arising at or near the base, with an open and spreading growth form, height is usually less than 5 meters. 
Back to top

dwarf shrubs

Short woody plants characterized by chamaephyte (dwarf-shrub/subshrub) growth form, bearing their buds on persistent shoots close to the ground, usually no more than 25 centimeters. Applicable for stress-tolerant plant groups, making up alpine, arctic or dry ecosystems, growing on nutrient- poor, often acidic soils or rock; also for prostrate shrubs, whose branches lie upon or just above the ground usually because of strong wind.

Includes: heathland/tundra species of e.g. the Calluna, Erica, Vaccinum, Salix groups, Mediterranean dwarf- shrub-form species such as Thymus, Lavander, Rosmarinus, alpine species of e.g. Rhododendron, Leontopodium. 
Back to top

Herbaceous Vegetation (grass-like, forbs, ferns)

Annual, biennial or perennial plants that do not have a persistent woody stem above the ground (in botanical term: herb). In contrary to woody plants, which have stems above ground that remain alive during the dormant season and grow shoots the next year from the above-ground parts, shoots of herbaceous plant die down at the end of growing season, so they regenerate themselves from tissues left above or under the ground (e.g. bulbs, rhizomes, tubers, seeds). 
Back to top

Graminoids (grass-like)

Grasses, or more technically graminoids, are monocotyledonous, usually herbaceous plants with narrow leaves growing from the base. They include the true grasses, of the Poaceae (or Gramineae) family, as well as the sedges (Cyperaceae) and the rushes (Juncaceae). The true grasses include cereals, bamboo and the grasses of lawns (turf) and grassland. Sedges include many wild marsh and grassland plants, and some cultivated ones. Belonging here regardless of being wild-growing - forming natural grasslands or being component of other biomes (e.g. wetlands, forest, tundra) or cultivated – forming cropland (arable, meadow, pasture) or grass surfaces / lawn for sports / recreation. 
Back to top

grasses, sedges, rushes, cereals

Annual or perennial graminoid plants, naturally growing or cultivated, with potential height not exceeding 2 meter.

Includes: most grass species and cereals (e.g. wheat, barley, maize, rice).

Excludes: reeds and bamboo. 
Back to top

poaceae, grasses, cereals

Back to top

Cyperaceae, Sedges, Rushes

Back to top

reeds, bamboos, canes

Perennial graminoid plants potentially capable of reaching > 2 m height.

Includes:

common reed (Phragmites australis), high-growing bamboo species, tall bulrushes and sedges, sugar cane.

Special case giant bamboo:

The giant bamboo reeds grow significantly higher (10-40 m) than one would expect from “normal” herbaceous vegetation types. Therefore, areas where giant bamboo is growing are often addressed as “forest” areas, because of the sheer height of the plants. The habitat type is different from normal herbaceous plants, as there can be a second layer understory growth like in tree forests underneath and in the shadow of the bamboo “canopy”. In such cases, it is important to enter the height of objects as a parameter in the EAGLE matrix / model (see LCH ) for a clear distinction and to flexibly include or exclude such areas from target classes according to any given classification system. Giant bamboo areas might be included to “Forest” areas respectively might be excluded from classes like “Grasslands”, although biologically speaking bamboo belongs to herbaceous vegetation.

Non-graminoids (forbs, ferns)

Naturally growing or cultivated herbaceous plants that are not graminoids, basically forbs and ferns. Many arable crop types – apart from cereals – are non-graminoid plants.

Includes: e.g. sunflower, vegetables, dry pulses, hop, strawberries, oil crops. 
Back to top

Succulents, Cacti

Water-retaining plants adapted to dry climatic conditions by storing water in their leaves, stems and roots, called succulents or fat plants. They occur in steppe and semi-desert, alpine conditions, but also on soils with high mineral (salt) content, such as seashores, salt lakes, salt marshes. Applicable for indigenous as well as for introduced species.

Includes:

cacti, agave, species of Crassulaceae, Sempervivum, Sedum groups, succulent halophytes (salt- tolerant plants) e.g. Salicornia. 
Back to top

Lichens, Mosses, Algae

lichens

Composite organisms formed by a symbiotic relationship of a fungus and a photosynthetic partner (usually green algae or cyanobacteria). 
Back to top

mosses

Non-vascular plants in the land plant division Bryophyta. They are small (a few centimeters tall) herbaceous (non-woody) plants that absorb water and nutrients mainly through their leaves and but also photosynthesize. 
Back to top

Algae

Algae are a very large group of different eukaryotic plant organisms that usually live in a habitat under or close to the water surface. There are also some algae that live on solid ground like rocks or tree stems in sufficiently humid micro climate conditions. From a biological perspective it is difficult to group them systematically, which is out of scope here. They can have all kinds of different plant sizes and growth forms. For this data model it seems reasonable to distinguish between macro algae like seaweeds and kelp that have similar twig-like shape to terrestrial plants, and micro algae. 
Back to top

macro algae (seaweed, kelp)

Macro algae are water plants that grow in a twig-like manner with “leaves” and reach a certain size. The plants can reach several meters of length. They grow either rooted under water and/or are floating on the water surface. Seaweed or kelp are commonly used terms for macro algae. 
Back to top

micro algae (plankton)

Micro algae are organisms normally bound to water in the form of uni-cellular or small multi-cellular organisms or clusters. They can also grow in terrestrial surrounding on solid surfaces like rocks or tree stems. 
Back to top

Water Surfaces

The chemical substance of H2O, either in liquid or in frozen solid state of aggregation. 
Back to top

Liquid Water Bodies

Water (H2O) in liquid state of aggregation regardless of location, shape, salinity and origin (natural or artificial). This element applies for at least to some extend stable formations, that may or may not be permanently filled with water. Whether it is a permanent or only episodic or periodic water body can be further described with the Water Regime segment under Land Characteristics of LCH block in EAGLE matrix.

Excludes: During flood events temporarily flooded areas, which are normally occupied with other Land Cover Components and which are not part of the water body bed, are not considered a water body. 
Back to top

Inland Water Bodies

All kinds of water surfaces on the inland with no direct interference or interchange with open sea water, regardless of salinity and origin (natural or artificial). 
Back to top

water course

Water surfaces in fluent state.
Includes: rivers, streams, creeks, channels, waterways.
Special case Estuary: 
Estuaries is not represented within the EAGLE model as an explicit LCC. It is more a geographical term than a LCC, and moreover rather difficult to delineate. Also, how to categorize an estuary as coastal water / transitional water / marine water depends somehow on the users perspective. 
However, the EAGLE group suggests to describe an estuary with LCC water course in combination with water characteristics (salinity, tidal influence). Basically, the water course is the primary source and reason for the existence of an estuary. Therefore it is considered reasonable to use LCC water course. 
Back to top

standing water

Water surfaces of non-flowing water, mainly lakes and ponds, or cut-off river meanders.

Includes: natural lakes (both freshwater and salty), fishponds, man-made reservoirs, oxbow lakes, pools with non-sealed bottom, irrigation ponds, ponds for artificial snow production, rivers dammed for hydroelectric power production, ponds for firefighting. 
Back to top

marine waters

Open sea, zone seaward of the average high tide line. May stand under tidal influence, forming intertidal flats along cost lines. 
Back to top

Solid waters

Solid water is H2O in frozen form. 
Back to top

snow

Snow cover that persists throughout the year, above or beyond the climatic snow line (nival zone). 
Back to top

ice, glaciers

Persistent ice cover formed by accumulation and compaction of snow over time. 
Back to top

# II. Matrix block: LAND USE ATTRIBUTES (LUA)

This matrix block contains all kinds of land use types, here called Land Use Attributes (LUA). It is best to select the lowest possible land use type, as far as information about it is available. Still, they can be selected on any higher hierarchical level if the lowest more detailed level is not known. Also, they can occur in combinations. Technically speaking, their cardinality is 1..n (one to many).

1_Primary Production Sector

Areas where the production of goods is directly based on local natural resources. The primary sector transforms natural resources into primary products. Most products from this sector are either raw materials for other industries (e.g. food, metal, wood industry), or are directly consumed by end- users. Major subsectors of primary production are agriculture, animal husbandry, agribusiness, fishing, forestry and mining and quarrying activities. Areas where the manufacturing industries aggregate, package, purify or process the primary products close to the primary producers are to be included, especially if the raw material is unsuitable for sale or difficult to transport long distances. 
Back to top

1_1_Agriculture

Production of crop (plants, fungi, etc.) and animal products for food, for sale, own consumption or industrial purposes. It includes plants for biofuels and growing of crops in open fields as well as in greenhouses. Also set-aside fallow land in the crop rotation belongs to this class. The preparation of products for the primary markets is included, field construction (e.g. agricultural land terracing, drainage, preparing rice paddies etc.) as well as landscape care and maintenance. 
Back to top

1_1_1_Commercial Agricultural Production

Arable land, permanent crops and grasslands in agricultural use (both sown and self-seeded grasslands, in open land as well as in greenhouses). The products can be used for human or animal feed, for bio-energy production or industrial purposes. 
Back to top

1_1_2_Farming Infrastructure (buildings, facilities)

Farm dwellings, animal husbandry infrastructure (animal dwellings and processing infrastructure linked to farms), manure storage and other farming infrastructure (e.g. buildings linked to plant handling and processing in farms). 
Back to top

1_1_2_1_Animal Husbandry

This sub-class applies for animal dwelling and shelter, also for organized feeding.

It includes raising of mammals, birds, land mollusks, insects.
[EAGLE extension, not in HILUCS] 
Back to top

1_1_2_2_Farming Storage

This sub-class applies for storage areas or buildings like maize silage or barns. [EAGLE extension, not in HILUCS] 
Back to top

1_1_2_3_Other farming infrastructure

This sub-class applies for farming infrastructure that is not for animal dwelling, or storage, but for plant handling and processing and similar processes.
[EAGLE extension, not in HILUCS] 
Back to top

1_1_3_Agricultural Production For Own Consumption

Production of plants or animals for own consumption (kitchen gardens, private animal sheds etc.) 
Back to top

1_2_Forestry

Production of round wood and other wood based primary products. Besides the production of timber, forestry activities result in products that undergo little processing, such as firewood, charcoal and round wood used in an unprocessed form (e.g. pit-pro

ps, pulpwood etc.). Forest tree nurseries, storage and transport areas linked to logging trees and woody plants for bio fuels are also included. These activities can be carried out in natural or planted forests. 
Back to top

1_3_Mining And Quarrying

Mining and quarrying in the form of the extraction of minerals and materials occurring naturally as solids (coal, ores, gravel, sand, salt), liquids (petroleum), gases (natural gas) or biomass (peat).

Extraction can be achieved by different methods such as underground or surface mining or extraction, well operation etc.

Originally, in the HILUCS classes this use type was subdivided into three subtypes, where the EAGLE matrix has now a different subdivision. 
Back to top

1_4_Aquaculture And Fishing

This class includes professional fishing and aquaculture. 
Back to top

1_4_1_Aquaculture

This class includes areas used for fish hatcheries and managed grow-out sites, as well as for amphibians, crocodiles, water mollusks. 
Back to top

1_4_2_Professional Fishing

This class includes water areas used for professional fishing. 
Back to top

1_5_Other Primary Production

Professional hunting, gathering of wild growing non-wood forestry products, husbandry of migratory animals and any other primary production not included in the values 1_1_Agriculture, 1_2_Forestry, 1_3_MiningAndQuarrying, 1_4_AquacultureAndFishing or any of their narrower values. 
Back to top

1_5_1_Hunting

This class includes areas used for professional hunting. The areas can be fenced or open. 
Back to top

1_5_2_Management Of Migratory Animals

Areas used for keeping and feeding migratory animals such as reindeer and deer. 
Back to top

1_5_3_Picking Of Natural Products

Areas which are used for picking up natural non wood based products such as not cultivated berries, mosses, lichen etc.) for commercial purposes. 
Back to top

1_5_4_Apiculture (Bee hives)

This class indicates that a certain area is used for honey making, with one or more bee populations present. Similar to hunting or fishing it is difficult to delineate the outer boundary if it.
[EAGLE extension, not in HILUCS] 
Back to top

2_Secondary Production Sector

Industrial and manufacturing activities which take the output of the primary sector and manufacture finished goods and intermediate products for other business. This class also includes the storage and transport areas linked directly to manufacturing activities. The branches of industries covered by this class are the processing of: food, textile, leather, wood and wood product, pulp, paper, publishing, printing, recording, petroleum and other fuels, chemicals, chemical products, man-made fibers, rubber and plastic products, non-metallic mineral products, basic metals and metal products, fabricated metal product, machinery and equipment, electrical and optical equipment, transport equipment and furniture. 
Back to top

2_1_Manufacturing / producing industry

The three subtypes of secondary production sector in HILUCS (raw, heavy end, light end production) have received an intermediate LUA heading in EAGLE matrix, embracing them three altogether, which differs here from original HILUCS structure. 
Back to top

2_1_1_Raw Industry

This class includes the industrial activities transforming the output primary sector into manufactured raw products. In the EAGLE matrix, manufacturing / industries are only listed down to level 3. Further subdivisions (rooted in the HILUCS classes) as shown here, are not implemented in the matrix. For completeness and explanation reasons, they are still listed as differentiated according to HILUCS. 
Back to top

2_1_1_1_Manufacturing Of Textile Products

This class includes areas used for the preparation and spinning of textile fibers, sewing threads, textile weaving, and for the tanning and dressing of leather.  
Back to top

2_1_1_2_Manufacturing Of Wood And Wood Based Products

This class includes the areas used for sawmilling and planning of wood, manufacturing of veneer sheets, plywood, laming boards, fiber boards, carpentry and joinery, cork, straw and plaiting products.  
Back to top

2_1_1_3_Manufacturing Of Pulp, Paper And Paper Products

This class includes the areas used for the manufacturing of pulp, paper, paperboard, paper based sanitary goods, wallpapers.  
Back to top

2_1_1_4_Manufacturing Of Coke, Refined Petroleum Products And Nuclear Fuel

This class includes the areas used for the manufacturing of coke, refined petroleum and processing of nuclear fuel.  
Back to top

2_1_1_5_Manufacturing Of Chemicals, Chemical Products, Man-Made Fibers

This class includes the areas used for the manufacturing of basic chemicals, agro-chemicals, paints, pharmaceuticals, soap, detergents, glues, other chemical products and man-made fibers.  
Back to top

2_1_1_6_Manufacturing Of Basic Metals And Fabricated Metals

This class includes the areas used for the manufacturing, processing and casting of iron, steel and basic precious and non-ferrous metals. It also includes the manufacturing of metal products.  
Back to top

2_1_1_7_Manufacturing Of Non-Metallic Mineral Products

Manufacturing of glass, bricks, ceramics, concrete, cement, lime, plaster, cutting and shaping of stone and other non-metallic mineral products.  
Back to top

2_1_1_8_Manufacturing Of Rubber Plastic Products

Areas used for manufacturing of tyres, tubes, plastic packing goods and other rubber and plastic products.  
Back to top

2_1_1_9_Manufacturing Of Other Raw Materials

Areas for the production of raw materials not included in any other of the narrower values of 2_1_RawIndustry. 
Back to top

2_1_2_Heavy End Product Industry

Areas used for activities transforming raw manufactured products into heavy manufactured products. In the EAGLE matrix, industries are only listed down to level 3. Further subdivisions (rooted in the HILUCS classes) as shown here, are not implemented in the matrix. For completeness and explanation reasons, they are still listed as differentiated according to HILUCS. 
Back to top

2_1_2_1_Manufacturing Of Machinery

Manufacturing of machines for production, agricultural, forestry and other purposes (excluding aircrafts and vehicles), manufacturing of weapons, ammunition and domestic appliances. 
Back to top

2_1_2_2_Manufacturing Of Vehicles And Transport Equipment

This class includes the areas used for the manufacturing of motor vehicles and transport equipment like aircrafts, space crafts, ships, boats, railway and tramway equipment, motorcycles, bicycles and other transport equipment. 
Back to top

2_1_2_3_Manufacturing Of Other Heavy End Products

Production of other heavy end products not included in any other of the narrower values of 2_2_HeavyEndProductIndustry. 
Back to top

2_1_3_Light End Product Industry

Areas used for activities transforming raw manufactured products into light manufactured products. In the EAGLE matrix, industries are only listed down to level 3. Further subdivisions (rooted in the HILUCS classes) as shown here, are not implemented in the matrix. For completeness and explanation reasons. 
Back to top

2_1_3_1_Manufacturing Of Food Beverages And Tobacco Products

This class includes areas used for the manufacturing of meat, fish, fruit and vegetables, oils and fats or derived products, dairy products, grain mill and starch products, prepared animal feeds, other food products, beverages and tobacco products. 
Back to top

2_1_3_2_Manufacturing Of Clothes And Leather

Manufacturing of wearing apparel, leather clothes, dressing, accessories, dyeing of fur and manufacturing of fur products, luggage, bags, saddles and footwear. 
Back to top

2_1_3_3_Publishing And Printing

This class includes the areas used for publishing and printing of books, newspapers, journals and the publishing and reproduction of sound recordings. 
Back to top

2_1_3_4_Manufacturing Of Electrical And Optical Equipment

This class includes the areas used for the manufacturing of office machinery, computers, motors, generators, electricity distribution and control apparatus, wires and cables, accumulators, batteries, lamps, radios, TVs, phones, electronic valves and tubes, medical, precision and optical instruments, watches and other electrical and optical equipment. 
Back to top

2_1_3_5_Manufacturing Of Other Light End Products

This class includes the manufacturing of furniture, jewelry, musical instruments, sports goods, games, toys and other miscellaneous products. 
Back to top

2_2_Energy Production

This class includes the areas used for production of (electric) energy. 
Back to top

2_2_1_Nuclear Based Energy Production

This class includes areas where nuclear power plants are operated
Back to top

2_2_2_Fossil Fuel Based Energy Production

Power plants using fossil fuels (coal, oil, natural gas, peat and other fossil fuels) 
Back to top

2_2_2_1_Coal Lignite Based

Power plants using coal or lignite as combustion material. [EAGLE extension, not in HILUCS] 
Back to top

2_2_2_2_Mineral Oil Based

Power plants using mineral oil as combustion material. [EAGLE extension, not in HILUCS] 
Back to top

2_2_2_3_Mineral Gas Based

Power plants using gas as combustion material. [EAGLE extension, not in HILUCS] 
Back to top

2_2_2_4_Peat And Other Fossil Fuel Based

Power plants using peat and others as combustion material. [EAGLE extension, not in HILUCS] 
Back to top

2_2_3_Biomass Based Energy Production

Combustion power plants using biomass based fuels (wood and other plant based solid and liquid fuels, biogas and other biofuels). 
Back to top

2_2_4_Renewable Energy

Hydro-, solar, wind, thermal (aero, geo and hydro), tidal, wave etc. energy and other renewable energy (except biomass energy, which is covered by the value 2_4_3_BiomassBasedEnergyProduction). 
Back to top

2_2_4_1_Water Energy

[EAGLE extension, not in HILUCS] 
Back to top

2_2_4_2_Solar Energy

[EAGLE extension, not in HILUCS] 
Back to top

2_2_4_3_Wind Energy

[EAGLE extension, not in HILUCS] 
Back to top

2_2_4_4_Geo-Thermal Energy

[EAGLE extension, not in HILUCS] 
Back to top

2_2_4_5_Tidal Wave Energy

This class applies for areas in coastal zones that are used to produce electric energy by making use of the water currents caused by tidal waves and flows to spin turbines. [EAGLE extension, not in HILUCS] 
Back to top

2_2_5_Waste Combustion Energy

This class includes areas where energy is produced by burning waste. [EAGLE extension, not in HILUCS] 
Back to top

2_5_Other Industry

Production of other industrial products not included in any other of the narrower values of 2_SecondaryProduction. 
Back to top

3_Tertiary Service Sector

Services that are products for other businesses and consumers both private and public services. It encompasses whole sale and retail trade, repair services, hotels and restaurants, financial services, real estate, business services, rental services, public administration, defense and social security, education, health and social work and other community, social and personal services. 
Back to top

3_1_Commercial Services

This class includes the areas used for the provision of commercial services
Back to top

3_1_1_Wholesale And Retail Trade, Repair Of Vehicles And Personal And Household Goods

Wholesale and retail sale of motor vehicles, fuel, agricultural raw materials, live animals, ores, metals, chemicals, timber , machinery, ships, furniture, household goods, textiles, food, beverages, tobacco products, pharmaceutical products, second hand goods, other products, waste and scrap. This class also includes the repair of vehicles, personal and household goods.

[not included in EAGLE matrix] 
Back to top

3_1_2_Real Estate Services

This class includes the areas used for the provision of real estate and renting services.

[not included in EAGLE matrix] 
Back to top

3_1_3_Accommodation And Food Services

This class includes the areas used for provision of hotel, holiday village, camping site, restaurant, bar and canteen services.

[not included in EAGLE matrix] 
Back to top

3_1_4_Other Commercial Services

Other commercial services not included in any other of the narrower values of 3_1_CommercialServices, such as beauty and wellbeing services.

[not included in EAGLE matrix] 
Back to top

3_2_Financial Professional And Information Services

This class includes areas used for the provision of financial, professional or information services. 
Back to top

3_2_1_FinancialAndInsuranceServices

This class includes the areas used for the provision of banking, credit, insurance, and other financial services.

[not included in EAGLE matrix] 
Back to top

3_2_2_Professional Technical And Scientific Services

This class includes the areas used for the provision of IT consulting, data processing, research and development, legal, accountancy, business management, architectural, engineering, advertising, testing, investigation, consulting, research, development and other professional services.

[not included in EAGLE matrix]. 
Back to top

3_2_3_Information And Communication Services

This class includes the areas used for the provision of publishing, sound recording, TV-programme, motion picture, radio broadcasting, post and telecommunication, computer and data processing services.

[not included in EAGLE matrix] 
Back to top

3_2_4_Administrative And Support Services

This class includes the areas used for the provision of travel agency, rental, cleaning, security and other administrative and support services.

[not included in EAGLE matrix] 
Back to top

3_2_5_Other Financial Professional And Information Services

Other financial, professional and information services not included in any other of the narrower values of 3_2_FinancialProfessionalAndInformationServices.

[not included in EAGLE matrix] 
Back to top

3_3_Community Services

This class includes the areas used for the provision of services for the community. 
Back to top

3_3_1_Public Administration Defense And Social Security Services

This class includes the areas used for the provision of generic administrative, defense, justice, public security, fire and compulsory social security services. 
Back to top

3_3_2_Educational Services

This class includes the areas used for the provision of primary, secondary, higher, adult and other educational services. 
Back to top

3_3_3_Health And Social Services

This class includes the areas used for the provision of human and animal health and social work services. 
Back to top

3_3_4_Religious Services

This class includes the areas used for the provision of religious services. 
Back to top

3_3_4_1_House Of Worship

This class applies for churches, mosques, synagogues, and other house types of confession. This use type can include administrative buildings, mostly associated to the church, synagogue etc. itself.
[EAGLE extension, not in HILUCS] 
Back to top

3_3_4_2_Monastery

This class applies for monasteries where a group of people (mostly men and women separate, but not necessarily) live and organized themselves to dedicate their life time to the practicing of their religion according to certain religious principles and rituals.
[EAGLE extension, not in HILUCS] 
Back to top

3_3_4_3_Cemetery

This class applies for cemeteries of all kinds of confession, as well as historical cemeteries. [EAGLE extension, not in HILUCS] 
Back to top

3_3_5_Other Community Services

This class includes areas used for other community services (e.g. cemeteries). 
Back to top

3_4_Cultural Entertainment And Recreational Services

This class includes the areas used for the provision of cultural, entertainment or recreational services. 
Back to top

3_4_1_Cultural Services

This class includes the provision of artistic, library, museum, zoos, botanical gardens, historical sites and other cultural services. 
Back to top

3_4_1_1_Indoor Cultural Services

This class includes the provision of indoor cultural services such as libraries, museums, theatres.
[EAGLE extension, not in HILUCS]. 
Back to top

3_4_1_2_Outdoor Cultural Services

This class includes the provision of outdoor cultural services like e.g. zoos, botanical gardens. 
[EAGLE extension, not in HILUCS] 
Back to top

3_4_2_Entertainment Services

This class includes the provision of amusement parks, theme parks, betting and gambling activities (casinos) and other entertainment services. 
Back to top

3_4_3_Sports Infrastructure

This class includes the areas used for the provision of sports infrastructure, such as stadiums, sports halls, swimming pools, fitness facilities, ski resorts, golf courses and other sports infrastructure. 
Back to top

3_4_3_1_Golf Course

This class applies for golf courses.

(EAGLE extension, has been added as EAGLE-extension to HILUCS code list in UML model) 
Back to top

3_4_3_2_Ski Piste

This class applies for ski pistes. A ski piste is part of a slope for the execution of wintersport of alpine skiing, but also other for the usage of other sliding devices on snow surface like now boarding.
[EAGLE extension, not in HILUCS] 
Back to top

3_4_3_3_Outdoor Racing Course

This class applies for outdoor race courses like car race courses, bike courses, moto-cross or mountain bike courses, horse race course, skate parks.
[EAGLE extension, not in HILUCS] 
Back to top

3_4_3_4_Sport Hall

This class includes the areas of sports halls which have at least the size as a whole to perform team ball games like basketball, volleyball, handball etc. 
[EAGLE extension, not in HILUCS] 
Back to top

3_4_3_5_Stadium

This class includes the areas of stadiums, where a sports ground (turf, sand, artificial grass etc.) is surrounded by roofed or non-roofed tribunes for audience. 
[EAGLE extension, not in HILUCS] 
Back to top

3_4_3_6_Swiming Pool

This class includes the areas of indoor swimming halls or outdoor swimming pools. 
[EAGLE extension, not in HILUCS] 
Back to top

3_4_3_7_Sports Ground

This class applies for sport fields where outdoor sports are practiced. It is not roofed, and is not surrounded by a tribune structure, like it would be in a stadium.
[EAGLE extension, not in HILUCS] 
Back to top

3_4_3_8_ Fitness club

This class applies for indoor sports and fitness clubs (boxing, wrestling, fighting, body workouts, gymnastics etc.). It is different from sport halls which have at least the size as a whole to perform team ball games like basketball, volleyball, handball etc.
[EAGLE extension, not in HILUCS] 
Back to top

3_4_3_9_Yacht harbor, sport boat marina

This class applies for land areas that belong to a sport boat harbor. It can also be applied to the water area (harbor basin) that belongs to it.
[EAGLE extension, not in HILUCS] 
Back to top

3_4_4_Open Air Recreational Areas

This class includes open air recreational areas e.g. urban parks, playgrounds, national parks, and natural areas used for recreational purposes (e.g. forests, heathland, moors, mountains, agricultural areas, ponds, lakes, rivers). 
Back to top

3_4_4_1_Urban Greenery, City Parks, Playgrounds

This class includes open air recreational areas e.g. urban greenery, parks, playgrounds. 
[EAGLE extension, not in HILUCS] 
Back to top

3_4_4_2_Semi Natural Areas And National Parks

This class includes open air recreational areas e.g. national parks and natural areas used for recreational purposes (e.g. forests, heathland, moors, mountains, agricultural areas, ponds, lakes, rivers).
[EAGLE extension, not in HILUCS] 
Back to top

3_4_5_Other Recreational Services

Other recreational services not included in any of the other narrower values of 3_4_CulturalEntertainmentAndRecreationalServices. 
Back to top

3_4_5_1_Allotment Garden

This class includes small-parceled city gardens that can be used for subsistence, but mainly are used for leisure and recreational purpose.
[EAGLE extension, not in HILUCS] 
Back to top

3_4_5_2_Amateur Fishing

This class includes water areas used for amateur or hobby fishing. 
[EAGLE extension, not in HILUCS] 
Back to top

3_5_Other Services

This class includes the areas used for the provision of other services not included in parts 3.1-3.4 of HILUCS. 
Back to top

4_Transport Networks, Logistics And Utilities

Basic infrastructure and networks of the society. All the other sectors are using the infrastructure and networks to produce the goods and services and they are also vital for residential areas. It includes land used for water supply, collection, treatment and recycling of sewage and waste, transport, networks, storage and communication. 
Back to top

4_1_Transport Networks

This class includes the infrastructure related to transport. 
Back to top

4_1_1_Road Transport

This class includes the areas used for road transport e.g. roads, parking areas, service stations. 
Back to top

4_1_2_Railway Transport

This class includes the areas used for rail transport e.g. rails, railway stations and yards etc. 
Back to top

4_1_3_Air Transport

This class includes the areas used for air transport e.g. airports and related services. 
Back to top

4_1_4_Water Transport

This class includes the areas used for water transport e.g. ports, rivers, docks and related services. 
Back to top

4_1_5_Other Transport Network

Areas used for other transport not included in any of the other narrower values of 4_1_TransportNetworks. 
Back to top

4_2_Logistical And Storage Services

This class includes areas used for the storing of industrial or trading goods, which are not directly connected and associated to industrial areas. It includes as well the installations that are needed to facilitate the logistics and storage. 
Back to top

4_3_Utilities

This class includes the infrastructure related to utilities. 
Back to top

4_3_1_Power Distribution Services

This class includes the areas used for the distribution of electricity, gas and thermal energy. This class includes the pipelines used for transporting mineral oil and gas. 
Back to top

4_3_1_1_Electricity Distribution

This class addresses the areas used for the distribution of electricity. 
[EAGLE extension, not in HILUCS] 
Back to top

4_3_1_2_Gas Distribution

This class addresses the areas used for the distribution of gas. 
[EAGLE extension, not in HILUCS] 
Back to top

4_3_1_3_Thermal Energy Distribution

This class addresses the areas used for the distribution of thermal heat. 
[EAGLE extension, not in HILUCS] 
Back to top

4_3_2_Water And Sewage Infrastructure

This class includes the areas used for the extraction, collection, purification storage and distribution of water and collection and treatment of sewage (including the pipelines). 
Back to top

4_3_2_1_Drinking Water Facility

This class applies for facilities and associated areas that are dedicated to the extraction and treatment, storage and distribution of drinking water.
[EAGLE extension, not in HILUCS] 
Back to top

4_3_2_2_Sewage Water And Treatment Facility

This class applies for water sewage facilities and associated areas that are dedicated to the treatment of waste water.
[EAGLE extension, not in HILUCS] 
Back to top

4_3_2_3_Water Run-Off Retention Basin

This class applies for water basins with the intention to hold back water run off in a water course. The aim is here to lower the hazard of flooding.
[EAGLE extension, not in HILUCS] 
Back to top

4_3_2_4_Artificial Snow Water Pond

This class applies for water ponds that are maintained to store water for the production of artificial snow in ski resort areas.
[EAGLE extension, not in HILUCS] 
Back to top

4_3_2_5_Irrigation Or Fire Fighting Water Pond

This class applies for water basins that store water for the purpose of fire extinction or for the irrigation of agricultural (or even forest) land.
[EAGLE extension, not in HILUCS] 
Back to top

4_3_3_Waste Treatment

This class includes the areas used for the collection, treatment and recycling of waste (dumpsites, waste incineration, composting, hazardous waste treatment and recycling facilities). 
Back to top

4_3_3_1_Dump Site

This class applies for areas used for the collection and deposit of waste. 
[EAGLE extension, not in HILUCS] 
Back to top

4_3_3_1_1_hazardous waste

[EAGLE extension, not in HILUCS] 
Back to top

4_3_3_1_2_inert (non-hazardous) waste

[EAGLE extension, not in HILUCS] 
Back to top

4_3_3_2_Recycling Facility

This class includes the areas used for the recycling of waste or used material that is prepared to be re-introduced to a product life cycle. This material for example can be paper, all kinds of plastic packaging and plastic bottles, glasses, but also biomass or biological waste turned into humus.
[EAGLE extension, not in HILUCS] 
Back to top

4_3_4_Other Utilities

This class includes areas used for other utilities not included in the classes 4.3. Utilities. 
Back to top

5_Residential Use

This class includes areas used dominantly for housing of people. The forms of housing vary significantly between, and through, residential areas. These areas include single family housing, multi-family residential, or mobile homes in cities, towns and rural districts if they are not linked to primary production. It permits high density land use and low density uses. This class also includes residential areas mixed with other non-conflicting uses and other residential areas (e.g. temporarily used areas). 
Back to top

5_1_Permanent Residential Use

This area includes residential areas dominated by houses of any size (detached houses surrounded by gardens and/or yards, a mix of single houses, semi-detached houses, terraced houses, town houses, row houses and blocks of flats, high raised houses) used as permanent residence. 
Back to top

5_2_Residential Use With Other Compatible Uses

This class includes residential areas mixed with other non-conflicting uses (e.g. various services, light industries etc.). 
Back to top

5_3_Other Residential Use

This class includes areas dominantly for temporary dwellings (camps of migrant people), holiday residences (summer cottages), etc. 
Back to top

5_3_1_Temporary Residential, Permanent Structure

This class includes areas for holiday residences (summer cottages), etc. 
[EAGLE extension, not in HILUCS] 
Back to top

5_3_2_Temporary Dwelling, Non-permanent Structure

This class includes areas dominated e.g. by areas used for temporary dwellings like refugee camps or for migrant people.
[EAGLE extension, not in HILUCS] 
Back to top

5_3_3_Informal Dwelling

This class includes areas dominated informal dwelling structures like slums, favelas etc. that appear inside or at the outskirts of bigger cities and agglomerations. Basically, they are constructed outside any legal or planning framework, but due to strong settlement pressure and lack of alternatives, these areas over time become “quasi-permanent” settlement areas.
[EAGLE extension, not in HILUCS] 
Back to top

6_Other Uses

Areas not included in the values 1_PrimaryProduction, 2_SecondaryProduction, 3_TertiaryProduction, 4_TransportNetworksLogisticsAndUtilities, 5_ResidentialUse or any of their narrower values. Areas under construction are included here. 
Back to top

6_1_Transitional Areas Under Construction

This class includes areas under construction. This class should be used only for existing land use and not for planned land use. 
Back to top

6_3_ Areas Without Any Economic Use

This class includes areas which are in natural state and not in other economic use
Back to top

6_4_Flood Protection

This class includes areas which are used for flood protection purposes, e.g. polder which can be flooded on purpose to lower the peak inundation wave of a river stream.
[EAGLE extension, not in HILUCS] 
Back to top

# III. Matrix block: LAND CHARACTERISTICS (LCH)

# Built-up Characteristics

Soil sealing degree

Data type: integer percentage value

The parameter for soil sealing degree indicates the proportion of sealed surface in relation to a given spatial unit that it refers to. The soil sealing degree is therefore express with percentage values. An example for such a dataset is the Copernicus product High Resolution Layer for Imperviousness (“Soil Sealing”). 
Back to top

Built-up Pattern

Data type: code list

The category of built-up patterns describes the size and distribution of buildings in space. Five different variations of built-up patterns can be distinguished: 
Back to top

scattered single houses, discontinuous

Data type: code list value

The area characterized with this built-up pattern contains some few buildings, which are distributed in space with clear distance between them. The houses are not dominant features in the area. This character is meant to be applied in sparsely populated rural areas. 
Back to top

single blocks, discontinuous

Data type: code list value

The area characterized with this built-up pattern contains buildings of large size with several floors and can have more than one entry door. The need for efficient usage of space for apartments or offices results often in the being built high. The buildings normally have some free space in between them for day light and fresh air circulation. Typically these block buildings occur in urban agglomeration zones and give housing or office room for a relatively high number (hundreds and more) of persons. An extreme example is a sky-scraper. 
Back to top

suburban row houses/terraced/semi-detached houses

Data type: code list value

This built-up pattern occurs mainly in suburban parts of settlements. A significant part of the parcel is occupied by house gardens. The front of houses may be closely constructed, but the bigger part of the total area is not sealed. 
Back to top

city street blocks, closed front

Data type: code list value

This built-up pattern occurs mainly in city centers and densely populated city quarters. A street block is here considered as the area surrounded by streets on all sides. The street block is continuously filled with buildings side to side along the street. Typically, the houses have 2 or more stores. The house fronts along the streets closed with buildings with no room between them. Also the backyards are mainly occupied with sealed surfaces. Some vegetated parts may occur in the inner part of the block. 
Back to top

large complex buildings, big halls

Data type: code list value

This kind of building type embraces very large buildings which cover several thousands of square meters and are mainly stand-alone objects, or parts of large industrial areas.

Examples: football stadium, congress hall, Cathedral, airport terminals, train stations etc. 
Back to top

Building Nature

Data type: code list

In this character type the nature of a building is expressed. It is based on the INSPIRE data specifications on buildings (BU) and addresses building sub-types like arch, bunker, canopy, castle, cave building, chapel, church, greenhouse, lighthouse, mosque, shed, silo, stadium, synagogue, temple, tower, windmill. The list can be extended. 
Back to top

Building Nature Value

Data type: code list value

This character is connected to LCC 1.1.1.1.2 Specific buildings and can be used to further describe it.

For practical reasons the code list values from INSPIRE theme Buildings are not listed here entirely. 
Back to top

Other Construction Nature

Data type: code list

In this character type the nature of artificial constructions other than buildings is expressed. It is based on the INSPIRE data specifications on OtherConstructionNatureValue and addresses construction types like acoustic fence, antenna, bridge, dam, chimney, city wall, crane, storage tank, monument, open air pool, protective structure, pylon, retaining wall, solar panel, substation, tunnel, wind turbine. The list can be extended. 
Back to top

Other Construction Nature Value

Data type: code list value

This character is connected to LCC 1.1.1.2 specific structures and facilities and can be used to further describe it. For practical reasons the code list values from INSPIRE theme Buildings are not listed here entirely. 
Back to top

Artificial Surface Material Type

Data type: code list

This category contains a list of man-made surface material, which are common in urban and settlement fabric. It is basically meant to address soil sealing materials and roof materials. As some material both can occur on the ground and on roofs, the code list shall be used for both possible locations. In terms of data model relations, these characters are supposed to be used either in connection with LCCs of Sealed or Non-Sealed Artificial Surfaces. The list contains categories and sub types, which both can be used as a value to characterize an LCC, depending on available information details about the material itself. 
Back to top

Mineral Compound

Data type: code list value

Mineral Compounds are artificial materials made out of mined earthen matter that is processed by mixing several components and/or adding water or heat.
Back to top

glass

Data type: code list value

#### ceramic (clay tiles, bricks)

Data type: code list value

Ceramic is a form of burnt clay minerals, which is used e.g. for tiles or bricks.

#### concrete

Data type: code list value

asbestos fibre cement

Data type: code list value

non-asbestos fibre cement

Data type: code list value

Solar Panels (silicium based)

Data type: code list value

Metal

Data type: code list value
Back to top

Copper

Data type: code list value

Zinc

Data type: code list value

Aluminium

Data type: code list value

Steel

Data type: code list value

Iron

Data type: code list value

Lead

Data type: code list value

Hydrocarbon Compound (Crude oil based)

Data type: code list value

Hydrocarbon compounds are artificial materials that are based on crude oil. They are the residues of refined mineral oil after extraction the more volatile substances. Sometimes mixed with mineral fragments (sand, gravel) they are used to on artificial surfaces like roads or roofs to seal and/or harden them, to make them impermeable. 
Practically, such surfaces can also have a natural origin, where mineral oil emerges by nature on the surface, combines with sands, and hardens out to some kind of natural asphalt, but those spots are very rare.
Back to top

Tar

Data type: code list value

Asphalt

Data type: code list value

Bitumen

Data type: code list value

Polymer, Plastic, Synthetic Fiber

Data type: code list value

This segment lists artificial materials that are artificially created by chemical processing of solid hydrocarbon-based components but also other substances in an industrial manner. Polymers are formed out of monomers through polyreactions.
Back to top

hard rubber

Data type: code list value

hard plastic

Data type: code list value

plastic foil

Data type: code list value

synthetic fiber

Data type: code list value

Mineral Material

Data type: code list value

These kinds of mineral materials come out of quarries, mountain slopes, mining lakes, river beds, sand dunes, beaches or any other natural mineralization or sedimentation ground in an unchanged form without any chemical processing besides extraction the material itself. They are basically stones from geological formations or volcanic activities in different fragmented, bigger or smaller sizes and shapes.
Back to top

natural stone

Data type: code list value

slate

Data type: code list value

pebbles, gravel

Data type: code list value

sand

Data type: code list value

ashes, slag

Data type: code list value

Organic

Data type: code list value
Back to top

Wood

Data type: code list value

reed/straw/leaves

Data type: code list value

Artificial Surface Material Arrangement

Data type: code list

This list contains some types of material arrangements which indicate how the materials itself or elements made out of a chosen material are put together, be it on ground surface or on roofs. 
Back to top

seamless sealing

Data type: code list value

The material brought onto surface in a seamless manner without any interruption in form of a continuous layer. 
Back to top

closed pavement

Data type: code list value

The material brought onto surface in form of single elements that have direct contact to each other. In between, they build seams, but no gaps. 
Back to top

permeable paving, grass pavers

Data type: code list value

The paving material (impermeable itself) is arranged in such a manner that water can infiltrate in between the elements through a pattern of small gaps. 
Back to top

tiles

Data type: code list value

The paving or roofing material is arranged in form of tiles, that may or may not overlap partly with each other. 
Back to top

Building Greening Type

Data type: code list

This category applies for greening measures on buildings and constructions on roof tops and theoretically also on facades. 
Back to top

Greened Roof

Data type: code list value

This character applies for roof tops that are greened with vegetation. 
Back to top

Low Growing Roof Vegetation

Data type: code list value

This character applies for roof tops that are greened with low growing vegetation like herbaceous plants, lichens and mosses, or dwarf shrubs. 
Back to top

High Growing Roof Vegetation

Data type: code list value

This character applies for roof tops that are greened with high growing vegetation like woody plants such as trees and bushes. They may appear as a roof gardens. 
Back to top

Transportation Network Characteristics

Data type: code list value

This segment contains a number of characteristics that apply for the transportation land use sector. 
Back to top

Road Network Type

Data type: code list

This category contains a list of characteristics that indicate the main mean of road transportation as road network type, including pedestrian walk ways and cycle ways. 
Back to top

Fast Transit Road, Highway

Data type: code list value

Back to top

Pedestrian Zone

Data type: code list value

This area is reserved for pedestrians only, sometimes also open for bicycles. Motorized traffic is forbidden or restricted to a very limited extent (e.g. logistics for retails shops). Normally, pedestrian zones are located in city centers, squares or shopping streets and often occupy the entire open space between buildings or artificial surfaces in that so assigned area.
Back to top

Associated Walk- Or Cycleway

Data type: code list value

Linear part of the transportation road network which is associated to a road and runs parallel to the road, can be separated from it by a narrow strip of vegetation. It is used for pedestrians and/or as bicycles lanes. 
Back to top

Walk- and Cycleway

Data type: code list value

Linear part of the transportation network which is used both for pedestrians and as bicycles lanes. It has its own track, independent and in some distance from a road.
Back to top

Cycleway

Data type: code list value

Linear part of the transportation network which is used only as bicycles lanes. 
Back to top

Walkway

Data type: code list value

Linear part of the transportation network which is foreseen only for pedestrians. 
Back to top

Open Square

< Data type: code list value

Non-linear part of the transportation network, mostly with a rectangular or rounded shape. Squares can have a broad variety of spatial extent. They may be assigned to pedestrian zone, or can also function as a roundabout in road traffic. 
Back to top

### placeholder>

### <placeholder>

### Harbour Type

Data type: code list

This category contains a number of characteristics that apply for the shipping transportation sector, especially for areas where ships and vessels can dock or be loaded/unloaded . 
Back to top

Cargo Port

Data type: code list value

Port where traded goods are shipped in and out. Usually, container terminals are associated with cargo ports. 
Back to top

Passenger Port

Data type: code list value

Port where people are shipped in and out as passengers. 
Back to top

Fishing Port

Data type: code list value

Port for ships that bring seafood from there fishing tours in the open sea. 
Back to top

Naval Port

Data type: code list value

Port for military ships. 
Back to top

Marina

Data type: code list value

Port for yachts and sport boats, for reciational and leisure purpose. 
Back to top

Local Multifunctional Harbour

Data type: code list value

Port which is not specified to a specific type of ships o boats can harbor 
Back to top

Shipyard

Data type: code list value

Areas at the shore or along rivers, normally associated to a port, where vessels are built, maintained or repaired. For that purpose, vessels are fixed in a normal dock (floating in water) or in a dry dock, without water contact.
Back to top

Jetty

Data type: code list value

Lengthy man-made construction towards the sea as a landing stage connected to the shore line, where vessels can dock to be loaded and unloaded, or passengers can go on and off board. Jetties are also to unload explosive or hazardous substances like mineral gas or oil, in a secure distance from vulnerable infrastructure on land. 
Back to top

Physical Characteristics

Permafrost zone

Data type: Boolean value

Area where the ground is frozen because of temperature below 0° C throughout the period of at least two years without melting. It can be found either in polar/sub-polar regions or in high mountain areas in the nival height zone.
Back to top

Biotic / Vegetation Characteristics

Leaf form

Data type: code list

The leaf form helps to characterize woody plants by their leaves. It is a commonly used criterion to further distinguish trees and shrubs. Here three kinds of leaf forms are differentiated: 
Back to top

needle leaved

Data type: code list value
Back to top

broad leaved

Data type: code list value
Back to top

palm leaved

Data type: code list value
Back to top

non-leafy

Data type: code list value

This character applies for leave-less plants. 
Back to top

Foliage persistence

Data type: code list

The foliage persistence indicates if a plant loses its leaves at the end of a vegetation period (in Europe normally during autumn times) and grows new leaves in spring time, or if the plant keeps the leaves leaves/needles of a longer period (several years). The process of dropping or shedding the leaves is called abscission, and applies both for needle and broad leaved plants. 
Back to top

evergreen

Data type: code list value

The plant keeps its leaves/needles over a multi-annual period. 
Back to top

deciduous

Data type: code list value

The plant loses its leaves/needles at the end of every vegetation cycle, basically every year. 
Back to top

winter deciduous

Data type: code list value

The plant loses its leaves/needles yearly at the end of every vegetation period. Winter deciduous plants lose their leaves because of freezing temperatures in winter times, when water turns into ice below 0° C and could not circulate anymore within the plant, and would also destroy plant tissue during freezing. 
Back to top

summer deciduous

Data type: code list value

The plant loses its leaves/needles yearly before or during the summer time, when temperatures are so hot that the plant would lose too much water through transpiration and dry out. 
Back to top

Leaf anatomy

Data type: code list

The leaf anatomy describes the outer “architecture” of the plant leaf, how the skin of a leaf is “designed”.
Back to top

sclerophyllous

Data type: code list value

Sclerophyllous leaves are adapted to long periods of dryness and heat. The plants feature hard leaves, short internodes (the distance between leaves along the stem) and leaf orientation which is parallel or oblique to direct sunlight. The leaves are characterized by their relatively small, stiff and have a leathery or waxy coating, to reduce loss of moisture due to transpiration, in adaptation to dry and hot weather periods throughout the year.
Sclerophyllous is not seen as a sort of leaf form and is not contrary to broad-leaved or needle-leaved. In fact, a plant can be sclerophyllous and broad- or needle (narrow) leaved at the same time. That is why it is handled here as stand-alone matrix element. 
Examples are cork oaks or olive trees.
Back to top

Phenological Plant Life Span

Data type: code list

This character code list expresses the duration of the life time of a plant. 
Back to top

annual plant

Data type: code list value

Annuals are plants that go through their entire lifecycle in one growing season. 
Back to top

biennial plant

Data type: code list value

Biennial are plants whose lifecycle spans two years, so they flower and produce seeds in their second year. 
Back to top

perennial plant

Data type: code list value

Perennials are plants that can survive unfavorable season (winter) and live for more than two years. 
Back to top

ephemeral plant

Data type: code list value

Ephemeral is a plant that has several life cycles in a growing season and can increase in numbers rapidly. 
Back to top

Plant Location

Data type: code list

The plant location character describes where the vegetation is growing: on solid ground (terrestrial), in the water as aquatic submerge plants (under water surface) or as aquatic emergent plants that float on the water surface and/or grow above it. 
Back to top

terrestrial plant

Data type: code list value

Terrestrial vegetation grows on solid ground of land. Also, herbaceous plants that can be found in wetlands and grow in standing or running water (like reeds) are considered as terrestrial. 
Back to top

epiphyte plant

Data type: code list value

Epiphyte plant is an organism that grows on the surface of a plant and derives its moisture and nutrients from the air, rain, water (in marine environments) or from debris accumulating around it. Epiphytes take part in nutrient cycles and add to both the diversity and biomass of the ecosystem in which they occur, like any other organism. They are an important source of food for many species. Typically, the older parts of a plant will have more epiphytes growing on them. Epiphytes differ from parasites in that they grow on other plants for physical support and do not necessarily affect the host negatively. 
Back to top

aquatic submerged plant

Data type: code list value

Aquatic submerged vegetation grows under the water surface. Their main life space is under water without reaching the surface. Typical examples are algae. 
Back to top

aquatic emergent plant

Data type: code list value

Aquatic emergent vegetation are water plants, but they grow mainly floating on the water surface or have their main life space floating on water surface. An example is a sweet water sea rose. 
Back to top

Growth Form/Habit

Data type: code list

This segment lists different kinds of plant growth forms, describing the plants shape and how the plant forms its branches.
Back to top

erect growth (single stem)

Data type: code list value

Growth form of a woody plant that has a single main stem. Under normal condition the stem grows in an upright fashion. Typical example for erect growth form is a tree.
Back to top

open/spreading/dense growth (multi-stem)

Data type: code list value

Growth form of a woody plant that has a several smaller stems that grow from the ground in a spreading manner upwards and to the sides. Typical examples for spreading growth form are bushes and shrubs.
Back to top

prostrate (creaping)

Data type: code list value

The branches or shoots of a prostrate plant mostly lie upon or just above the ground and grow horizontally, rather than being held erect (as it would be with the branches of trees). There are both woody as well as herbaceous plants that show such growth form.
Back to top

clump-forming

Data type: code list value

A clumping plant forms a mound or thicket, with their shoots growing outward from the center and maintaining a compact shape. Clumping plants encompass many varieties, from tall bamboos down to smaller sedges and rushes. Also some grass species grow in clumps.
Back to top

cushion-/mat-forming

Data type: code list value

A cushion plant is a compact, low-growing, pillow-like or mat-forming plant that is found in alpine, subalpine, arctic, or subarctic environments. The growth is limited in height above the ground (a few inches at most), and have slow growth in a nutrient-poor environment with delayed reproductivity and reproductive cycle adaptations. The branches of the plant are held close together, and none of them stands out to reduce exposure to unfavorable environmental climate conditions. It can be found on dwarf woody shrubs and non-woody plants. 
See also: https://en.wikipedia.org/wiki/Cushion_plant 
Back to top

climbing

Data type: code list value

Growth form of a plant that uses other plants or objects as physical sustaining support to climb towards the sunlight, because it cannot develop its own sustaining stems (unlike a tree). Such plants can either live as parasites and extract nutrition from their host plant (e.g. a tree), or feed themselves from the ground. Examples for climbing growth form are ivy, lianas, pepper, vanilla plant or grapevine.
Back to top

Raunkiaer Life Form

Data type: code list
For detailed definitions see https://en.wikipedia.org/wiki/Raunki%C3%A6r_plant_life-form 
Back to top

phanerophyte

Data type: code list value

For detailed definitions see https://en.wikipedia.org/wiki/Raunki%C3%A6r_plant_life-form
Back to top

epiphyte

Data type: code list value

For detailed definitions see https://en.wikipedia.org/wiki/Raunki%C3%A6r_plant_life-form
Back to top

chamaephyte

Data type: code list value

For detailed definitions see https://en.wikipedia.org/wiki/Raunki%C3%A6r_plant_life-form
Back to top

hemicryptophyte

Data type: code list value

For detailed definitions see https://en.wikipedia.org/wiki/Raunki%C3%A6r_plant_life-form
Back to top

cryptophyte

Data type: code list value

For detailed definitions see https://en.wikipedia.org/wiki/Raunki%C3%A6r_plant_life-form
Back to top

therophyte

Data type: code list value

For detailed definitions see https://en.wikipedia.org/wiki/Raunki%C3%A6r_plant_life-form
Back to top

aerophyte

Data type: code list value

For detailed definitions see https://en.wikipedia.org/wiki/Raunki%C3%A6r_plant_life-form
Back to top

Vegetation cover transition

Data type: code list

This segment addresses the transition status, in which a land unit is, regarding its vegetation cover, whether it is regenerating through natural succession, or it is degrading over time. 
Back to top

natural succession

Data type: code list value

This Character addresses the process of and land unit being in the transition state of natural succession. 
Back to top

vegetation degradation

Data type: code list value

This Character addresses the process of and land unit being in the transition state of vegetation degradation. 
Back to top

Plant Species Origin

Data type: code list

This segment gives information about the origin of plant species from the perspective of its current growing site where it is found growing in this particular case. 
Back to top

native plant

Data type: code list value

This species origin type indicates that this plant species can be found in this specific geographically determined region, area or environment. The presence of the respective species is not necessarily exclusively restricted to this particular area, where it grows (in contrary to endemic). Normally, native (indigenous) plants are found on this particular site since long time (centuries). 
Back to top

non-native plant

Data type: code list value

This species origin type indicates that a plant species normally cannot be found in this specific geographically determined region, area or environment. 
Back to top

endemic plant

Data type: code list value

This species origin type indicates that a plant species can only and exclusively be found in this specific geographically determined region, area or environment. It is a special case of native species. A threshold for the minimum or maximum scale of such an endemic area does not exist; it can go from micro habitat level to continental dimension
Back to top

invasive plant

Data type: code list value

This species origin type indicates that a plant species that has its original life zone in another region and is intruding at the moment into another area. In many cases, the distribution of invasive plants is

– intentionally or not – supported by human transportation activities. This process often goes along with outcompeting and repression of other native plants and a disturbance of the ecological equilibrium of native plants associations. 
Back to top

migratory plant

Data type: code list value

This species origin type indicates that the growing conditions of the original habitat type of a plant or plant association is changing, e.g. due to external climatic or ecological factors. This process makes the species adapt to the new habitat conditions and migrate into another (neighboring) area, where the migrating species find comparable conditions similar to its former native habitat. The phenomenon of migrating species follows rather long-term effects based on changes in natural environment (man-made or not), in contrary to invasive behavior. 
Back to top

Genetically Modified Plant

Data type: code list value

This character applies for plant species that are not entirely of natural origin, but have genetically modified, as it is used in crop species types to raise the yield or make them more resistant to toxic substances or unfavorable environmental conditions like drought. 
Back to top

Plant Species Type

Data type: code list

It is suggested here to address the issues of plant species types as handle in the INSPIRE data specifications for the theme Species Distribution. Under the data type SpeciesNameType the species type can be described with a collection of code lists. 
Back to top

Plant Species

Data type: code list value / text string

Plant Community Type

Data type: code list

This character is not yet finalized. The European Vegetation Survey (EVS) scheme could be taken as the reference scheme. As a reference the PhytoSociologicalScheme can be used. 
Back to top

Plant Community

Data type: code list value / text string

Plant Growing Season

Data type: date

With this character (and in combination with the temporal parameters under LCH-9_3_3 Period, Period Start Date respectively Period End Daten) the start and ending of the growing season of native and local plants in natural outdoor climate and soil conditions can be indicated. Regarding crop cultivation cycles with the purpose of harvest, another character LCH-5_1_8 Crop Season shall be used. 
Back to top

plant growing season ongoing

Data type: calendar date

This LCH indicates whether according to a given data definition the plant growing seasons for a certain plant or plant community is ongoing or not, according to its natural average phenological appearance. For crop growing season please refer to “LCH-5_1_8 Crop Season”.
Back to top

growing season end

Data type: calendar date

This LCH indicates when (on which date in a year) the growing seasons for a certain plant or crop type ends, according to its average phonological appearance. 
Back to top

Crown Cover Density

Data type: integer percentage value

The parameter for crown cover density indicates the share of a certain spatial land cover unit that is over-shielded by the crowns of trees (or bushes). Clearly to say that underneath the crown cover there is other land cover besides the tree (other trees, bushes, bare soil, grasses, mosses etc.). It is up to the user himself whether he wants to add up all area to no more than 100%, or if he allows the surface to have a third (vertical) dimension and reach area values above 100% of the area after adding up all kinds of vegetation. 
Back to top

Water Characteristics

This heading contains a collection of characters and parameters that apply for water surfaces or have a relation to the physical element of water. 
Back to top

Water Body Formation

Data type: code list

This character describes the degree of “naturality” of a water course or water body and to what extend its existence has its origin on nature or if it has been influenced by or embedded in anthropogenic structures. Three subtypes can be distinguished: 
Back to top

Natural water body

Data type: code list value

The river or lake has its natural extent, no artificial river banks are present, the water regime is not influenced by any water retention installations. 
Back to top

Controlled / regulated / heavily modified water body

Data type: code list value

The lake or river embedded in man-made structures, which have the function of stabilizing the river banks or controlling the water regime. Most navigable rivers are in a way controlled or regulated.

man-made water body

Data type: code list value

This character shall be applied for entirely artificial man-made linear or area-shaped water bodies, where without human interaction no natural water course or body would occur. Examples – among others – are artificial canals for shipping transportation or water reservoirs for irrigation, firefighting, artificial snow making in alpine regions. 
Back to top

Water Regime (Hydrological Persistence)

Data type: code list

This segment can store the kind of water regime, in other words the hydrological persistence, expressing the frequency and duration of how long a water body or water course is present and fluent (This character can also be used in combination with temporal parameters under Time Dimension Type). Four subtypes of water regimes are distinguished: 
Back to top

dry water regime

Data type: code list value

The water body bed is normally dry without any water being present. Surface water flow occurs very unregularly and only after heavy rainfall. Water body bed can be filled with sand, gravel or may be vegetated. Common phenomena in arid and semi-arid climate regions. Some expressions for a dry creek bed are “wadi” [arab.] or “rambla” [span.]. 
Back to top

ephemeral (episodic) water regime

Data type: code list value

Surface water presence occurs not regularly but only during or after precipitation when there is enough surface water run-off or accumulated surface water. All waterways that cease surface flow at some points in time and space along their course. Still a certain level of seasonality can be observed. 
Back to top

intermittent (periodic/seasonal) water regime

Data type: code list value

Surface water is present through a part of the year, typically the wet season (e.g. rainy season, snow melt season). Besides the water not being constantly present, it still occurs regularly every year. 
Back to top

perennial water regime

Data type: code list value

Surface water is present through the whole year. The river or lake is filled constantly with water (variations on water level included). 
Back to top

Water dynamics

Data type: code list

This character can store the dynamics of a water regime expressing if water is constantly flowing/streaming from one point to another point. Such water current occurs because of the inclination of the terrain, or doesn´t occur when the terrain has no inclination. Sometimes also tidal influence causes water currents. Three types of water dynamics are described: 
Back to top

running water

Data type: code list value

Water body with constant constantly moving water particles due to a current that leads to an exchange or renewal of water. at the observed location. Running water normally comes along with a certain degree of erosive effect on the ground.

Examples: River, Stream, Creek
Back to top

almost standing water

Data type: code list value

Minimal movement of water particles. It occurs basically in flat terrains where inclination is almost but not equal to 0. Almost standing water is still moving, but with such low velocity that it cause practically no or very little erosive effect.

Examples: regulated river, canal, drainage ditch
Back to top

standing water

Data type: code list value

Stagnant water body .without any movement of water particles. . E.g. lakes, puddles, ponds
Back to top

Soil moisture regime (Wetness)

Data type: code list

The following wetness characters are categorized referring to the Environmental Qualifiers of the General Habitat Categories (GHC), as laid out in the “Manual for Habitat and Vegetation Surveillance and Monitoring”. The soil moisture is dependent on external and internal factors like precipitation, evaporation, capillary rise and infiltration. Further, to measure soil moisture, two complementary parameters are of importance: the volumetric water content (VWC) and the soil water tension (SWT). VWC expresses the share in % of water volume in a given soil volume, and SWT measured the force necessary for plant roots to extract water from the soil.
Back to top

aquatic (water at surface)

Data type: code list value

This character indicates that surface water is present on a particular land unit, for example inside wetlands or other landscape types which are not addressed as a waterbody. It means a piece of land has such a high degree of soil wetness that shallow water is covering the surface due to high groundwater level, specific terrain and soil conditions (peat, naturally impermeable soil layers). 
Back to top

waterlogged/saturated ground

Data type: code list value

This character indicates that the soil is saturated (soaked) with water, meaning basically all soil pores are filled with water in the absence of soil air. This is the case for example because ground water level is high in general, or because of frequent heavy or enduring rainfall that makes the ground more or less permanently wet. 
Back to top

wet

Data type: code list value

This character indicates that the soil condition is wet, meaning that many soil pores are filled with water, but still soil air is also present in the pores. Through capillary flow, a substantial amount of water can move upwards into the upper soil layers from ground water levels near the surface. This soil condition is present throughout the entire year.
Back to top

seasonally wet

Data type: code list value

This character indicates that the soil condition is wet (see explanation of “wet”), meaning that many soil pores are filled with water, but still soil air is also present in the pores. The difference is that such soil conditions are only seasonally present and not throughout the entire year. The reason for this may be a humid raining season, or seasonally flooding, which both cause a high ground water level.
Back to top

mesic

Data type: code list value

In soils with a mesic regime, sufficient soil moisture is almost constantly available for vegetation. The soil is often well drained, but provides enough soil water to not put plants under drought stress which are not used to it. This regime goes together with temperate climate zones. Precipitation is well balanced and distributed more or less equally over the year(s). 
Back to top

semi-dry

Data type: code list value

Semi-dry soil regimes are characterized by a sequence of rainy and dry episodes throughout the year, where dry episodes dominate. The ground is often very well drained, so that precipitation infiltrates quickly into lower levels of the ground where it is out of reach for surface vegetation. The soil has a low moisture storage capacity, with only little capillary flow. 
Back to top

dry

Data type: code list value

This character indicates that the soil moisture condition is dry, meaning that throughout the year hardly any moisture is available for plants. The situation of drought can last over several years. However, with the event of occasional precipitation, some biotic vegetation is capable to survive and withstand the long periods of drought.

… 
Back to top

very dry

Data type: code list value

This soil moisture condition does not allow any grow of vegetation and is basically free of any biotic life form. It occurs typically in dry deserts.

xeric

Data type: code list value

This character applies for the typical Mediterranean climate conditions with humid and cool winters and hot and dry summers.

… 
Back to top

Wetness source

Data type: code list

This character can store the source of the wetness of the terrain. From which source water is coming that causes wet conditions. Three types of wetness sources are described: 
Back to top

Rainfed source

Data type: code list value

Wetness relying on rainfall. 
Back to top

surface water source

Data type: code list value

Wetness relying on inflow of surface water. 
Back to top

subterranean/groundwater source

Data type: code list value

Wetness relying on subterranean or ground water, in combination with capillary flow. 
Back to top

Salinity (Water or Soil)

Data type: code list

This parameter can store the value for the salinity of water or soil. Usually it is expressed by a value measured in per percent (pct) or parts per thousand (ppt). Salinity is the saltiness or dissolved salt content of a body of water. Generally, it is the concentration of mineral salts dissolved in water. Salinity may be expressed in terms of a concentration, the physical method to conclude on the concentration is by capturing the electrical conductivity. 
Back to top

brine

Data type: code list value

Brine water is saturated or nearly saturated with salt, with a concentration of 50 ppt and above. Also inland plains or lakes with no water outflow drainage in arid or semi-arid climate zones may contain water that can have even a higher degree of salinity than ocean water.
Example: Dead Sea, salt concentration ca. 330 ppt. 
Back to top

saline

Data type: code list value

In the common understanding, saline water is water in the ocean and terrestrial salt lakes. It has a salt concentration between 30 and 50 ppt. Saline water contains a significant concentration of dissolved salts. 
Examples: Atlantic Ocean, Pacific Ocean: average salt concentration 35 ppt; Red Sea: ca. 40 ppt; Mediterranean Sea: ca. 38 ppt; Salars in the Anden mountains. 
Back to top

brackish

Data type: code list value

Brackish water condition is in between saline and fresh water, with a salinity range between 5 to 30 ppt. Brackish water is water that has more salinity than fresh water, but not as much as seawater. It may result from mixing of seawater with fresh water, as in estuaries, or it may occur in brackish fossil aquifers. 
Example: Lagoons, Estuaries under tidal influence, or also lakes in plains without outflow drainage may have brackish water conditions. Baltic Sea: salt concentration 8 ppt; Black Sea: salt concentration 18 ppt. 
Back to top

fresh

Data type: code list value

Fresh water has a very low degree of salinity from 0.1 to 0.5 ppt. It is a precondition for water to be drinkable (ca. 0.1 ppt).

Example: Running water or ground water have naturally fresh water condition, if not disturbed by saline intermixture. 
Back to top

ultra fresh (rain water)

Data type: code list value

Ultra-fresh water is practically salt-free, as it is with rain water. Water with nearly no salinity. The salinity is equivalent or nearly equivalent to that of rain water. All kinds of precipitation (rain, snow, dew, hoar frost etc.) fall as ultra-fresh water. Water vapor turns into ultra-fresh water when it condensates. 
Back to top

pH value

Data type: Numeric value

The pH value is used to describe the acidity respectively the basicity of an aqueous (water based) solution. It is a measure for the concentration of hydrogen ions in the solution, how acid or alkaline it is. The scale reaches from 0 (acidic) to 14 (basic), where 7 is neutral (pure water). 
Back to top

Tidal Phenomena

Data type: heading

This category contains information about characteristics of areas that stand under tidal influence, meaning the regular and cyclic flooding by the tidal waves that sweep around the globe drawn by gravitational forces. 
Back to top

tidal influence

Data type: Boolean value

This character contains the information if a piece of land is under tidal influence. It can be either expressed by only a Boolean value (yes/no) or in combination with an integer value to store the average difference in meters/centimeters of water level height caused by the tides. 
Back to top

tidal range

Data type: Numeric value

This character gives information about the in-situ height difference in meters of the water level between low tide and high tide water level. 
Back to top

Groundwater

Data type: heading

Ground water table level

Data type: numeric value

This character gives information about the height in meters of the water table level below and relative to the terrain surface. 
Back to top

Snow Characteristics

Data type: heading

This category contains information about characteristics of snow covered surfaces. 
Back to top

Snow Height

Data type: numeric value

Snow height in meters. 
Back to top

Ice Characteristics

Data type: heading

This category contains information about characteristics of ice covered surfaces (land and water). [So far, the heading is a placeholder for future entries on ice characteristics in the model/matrix.]
Back to top

Land Management

Agricultural Land Management

Agricultural Cultivation Form

Data type: code list

The term agriculture cultivation type addresses the form and appearance of agricultural land, which is influenced by the crop types which are cultivated or grown on it, and by the sum of cultivation measures that give this type of land its character. There are 3 main cultivation forms of agricultural land:

    - Arable cropland

    - Permanent cropland (both summed up under cropland in general) and

    - Managed permanent grassland
Back to top

cropland

Data type: code list value

This cultivation form “cropland” embraces both arable crop land and permanent cropland. It is land suitable or used for the cultivation of crops. It contains all cultivated land where any kind of crop is planted, cultivated and harvested. Permanent grassland (managed) does not belong to this type and is handled separately. 
Back to top

arable crop land

Data type: code list value

The term arable crop land describes a kind of crop land which is used for planting annual or multi- annual crops. It may underlay crop rotation cycles. The soil needs to be ploughing or elsewise prepared for sowing, normally with the help of some sort of machinery or manual tools. Arable crop land includes not only actively cropped land but also set aside and fallow land (according to EU Common Agriculture Policy), as it is part of crop cycles on arable land. Also the waterlogged cultivation such as rice fields is included here. 
Back to top

permanent crop land

Data type: code list value

The term permanent crop land describes a kind of crop land which is used for planting permanent crops, mostly woody crop species like fruit/ nuts/ olive trees, vineyards etc. The plants are often planted in a regular pattern that allows enough space for growing and for harvesting. 
Back to top

managed permanent grassland

Data type: code list value

Permanent grassland is land used permanently (for several consecutive years, normally 5 years or more) to grow herbaceous plants for the purpose of fodder, forage or bio-energy plants and alike. It can be intentionally be sown or naturally self-seeded and is not included in the crop rotation on the holding. Normally it is not ploughed, if so, only with limited frequency or in specific periods. No crops are grown. .

If information is available, with LCH temporal duration it can be indicated for how many years minimum it is considered to be grassland. 
Back to top

Cultivation Practice

Data type: code list

Cardinality: 1..n

Cultivation practices form a collection or sequence of cultivation measures It gives also indication on the cropping cycle applied and the spatial arrangement of the plants. 
Back to top

crop rotation

Data type: code list value

This characteristic indicates if a piece of land is under crop rotation. This kind of measure stretches over a multi-annual period and can follow several different sequential variants. The character of crop rotation is meant to be applied on arable land where usually annual crops are grown. The change from arable cropland to permanent crop land is not understood as “rotation”, but as a change in the cultivation practice. 
Back to top

no crop rotation

Data type: code list value

This characteristic indicates if a piece of land is not under crop rotation. Instead, the same crop is cultivated year after year over a longer period.. 
Back to top

plantation

Data type: code list value

This form of planting aims at high economic efficiency in terms of industrialized practices for minimal resources input and high yields. It is a large-scale estate meant for farming specialized in cash crops. Crop plants are arranged normally in rows. Depending on the crop species, branches of the plants may be tightened to wires or poles to grow them in a homogenous way for rationalized crop production. 
Back to top

extensive cultivation practice

Data type: code list value

Extensive orchards are an extensive form of using fruit / nut tree, partly also berry shrubs, for fruit / nut production, or also olive trees. This kind of cultivation practice is of value for habitats of insects, birds and small mammals.

German expression for this kind of cultivation pattern is “Streuobstwiese” (with grass underneath) or “Streuobstacker” (with ploughed topsoil).. 
Back to top

agroforestry

Data type: code list value

As the name already indicates, this type of land use is a combination of agricultural land use with forestry-like use. The ground between openly standing or scattered trees is grazed by cattle or life stock, or arable crops are planted between the trees. . In German speaking countries this landscape type is called “Hain”, “Hudewald” or “Waldweide”. In the Mediterranean countries it is called Dehesa or Montado and still can be found more often. 
Back to top

shifting cultivation (slash&burn)

Data type: code list value

Shifting cultivation is a type of crop rotation in combination with a fallow land phase which is applied mainly by subsistent farmers in tropical regions. Rain forest is cut down to gain space for growing crops. The biomass that has been cut down is burnt, the ashes serve as nutrition for the crops. The cultivation period is limited to a few years due to the limited soil fertility. After a crop cultivation period the soil is left fallow so a secondary vegetation succession can take place. After a certain time, this land undergoes again the cycle of slash and burn practice. 
Back to top

intercropping

Data type: code list value

Intercropping is a multiple cropping practice that involves growing two or more crops in proximity at the same time and in the same space. The most common goal of intercropping is to be more space-efficient with the resource of cultivated land. The space in between plants of one crop type, that is not needed, can be occupied by another crop type. Like this, the production results in overall higher yields compared to for example growing crops one after the other in a crop rotating manner.. Further, intercropping can reduce the risk of soil erosion or of plant pests, and can balance the content enhancement and exploitation of soil nutrients through the combined crop species. Examples of intercropping strategies are planting a deep-rooted crop with a shallow-rooted crop, or planting a tall crop (nut trees) with a shorter crop that requires partial shade (arable crop). 
Back to top

kitchen garden

Data type: code list value

Kitchen garden is a term to describe a form of horticulture basically for own consumption, in contrary to commercial crop production, where small sized patches of all kinds of fruits, vegetables and herbs are grown. 
Back to top

paddy field cultivation (rice fields)

Data type: code list value

A paddy field is a temporarily flooded parcel of arable land used for growing semi-aquatic rice. In Europe this is the only method applied for rice production. 
Back to top

Cultivation Installation

Data type: code list

This list of characters contains a number of technical installations that help to foster plant growth, raise the yield or help to protect the fruits. 
Back to top

greenhouse (under glass/ foil)

Data type: code list value

This character applies for crops that are grown under permanently installed glass or mounted foil. Like that, the growing conditions are not affected by external climate or weather conditions. Instead, temperature, artificial insolation, water supply as well as the provision of chemical substances are in control of the farmer. 
Back to top

under foil (on ground)

Data type: code list value

This character applies for crops that are grown on arable land under foil, where the foil is not suspended but lays directly on the ground or over the plant. This way a greenhouse-like microclimate is created under the foil, which protects the soil and crop from direct rainfall impact, frost or high evapotranspiration. 
Back to top

plantation protection net

Data type: code list value

This character indicates that the cultivated crops, mostly permanent crops, are protected against external factors (e.g. birds or hail damage) that could diminish the quality or quantity yield. 
Back to top

espalier / trellis / lattice

Data type: code list value

The espalier cultivation practice is a form of controlling the growth of woody plants (here basically fruit trees and shrubs) by pruning and tying branches to a frame, which results in a more or less 2-dimensional growth of the plants, arranged in a linear form.

hydroculture

Data type: code list value

Hydroculture is a method of growing plants, usually crops, in the absence of natural soil, by using mineral nutrient solutions in an aqueous solvent. Terrestrial plants may be grown with only their roots exposed to the nutritious liquid, or, in addition, the roots may be physically supported by an inert medium. 
Back to top

vertical farming

Data type: code list value

This character applies for the indoor cultivation of crops in multiple stores, either in a multi-store building with greenhouse conditions, or in a greenhouse with vertically arranged multiple racks or shelf layers. 
Back to top

Cultivation Measure And Activities

Data type: code list

Cultivation measures are activities that are applied to treat the soil and maintain the crop land in a favorable, fertile and productive condition. It are all the processes involved in the production of plant-based foods, from planting to harvesting. Normally, these generic measures are widely applied and come along with common agricultural land use.

There are a number of subtypes of cultivation measures: 
Back to top

Ploughing and tillage

Data type: code list

This list of values indicates whether or not a parcel is ploughed, and – if known – in which direction according to the slope. 
Back to top

no ploughing

Data type: code list value
Back to top

ploughing applied, direction unspecified

Data type: code list value
Back to top

across the slope (parallel to contour lines)

Data type: code list value
Back to top

oblique direction (diagonal to slope)

Data type: code list value
Back to top

in direction of slope (perpendicular to contour lines)

Data type: code list value
Back to top

#### Fertilization

Data type: code list

Fertilization is an agricultural measure to increase or maintain the fertility in the soil by bringing inin nutrients from plant residues, manure or chemical fertilizers. This character indicates, if a piece of land is fertilized or not. 
Back to top

no fertilizing

Data type: code list value

This character indicates that this piece of land is not under fertilization activity. 
Back to top

fertilization applied, type unspecified

Data type: code list value

This character indicates that a land parcel is fertilized in principle, but the kind of used fertilizer (see above) is not known. 
Back to top

organic fertilizer

Data type code list value

Organic fertilizer is natural material in the form of organic biomass or animal excrements/manure which is brought onto the parcels and/or ploughed in. 
Back to top

animal manure

Data type: code list value

Animal manure is a natural kind of fertilizer where organic material in the form of animal excrements / manure is brought onto the parcels and/or ploughed in. 
Back to top

green manure

Data type: code list value

Green manure is organic biomass that is grown on or brought onto the parcels and ploughed in. Can be organic residue from other sources or grown as interim crop in between two cropping phases or after main crop. 
Back to top

synthetic fertilizer

Data type: code list value

Synthetic industrial fertilizers are made out of chemical substances based on minerals through industrial processes. 
Back to top

Weed Control

Data type: code list

Weed control is an agricultural measure to protect the planted crop from concurrent consumer vegetation (e.g. other herbaceous plants) regarding consumption of water, light or nutrients. This can be done mechanically, biologically or chemically by application of herbicides. 
Back to top

no weed control

Data type: code list value

This character indicates that this piece of land is not under weed control. 
Back to top

weed control applied, method unspecified

Data type code list value

Weed control is applied but the method is unspecified or unknown. 
Back to top

mechanical / biological weed control

Data type code list value

Weed control is applied by mechanical (e.g. tilling) or biological (make use of natural weed enemies) techniques. 
Back to top

chemical weed control

Data type code list value

Weed control is executed by application of chemical substances, like herbicides. 
Back to top

Pest Control

Data type: code list

Pest control is an agricultural measure to protect the planted crop from harmful insects or other organisms that damage the crop and thus lower the yield or shorten the lifetime of the crop plant. This can be done mechanically, biologically or chemically by application of pesticides. 
Back to top

no pest control

Data type: code list value

This character indicates that this piece of land is not under pest control. 
Back to top

pest control applied, method unspecified

Data type code list value

Pest control is applied but the method is unspecified or unknown. 
Back to top

mechanical / biological pest control

Data type code list value

Pest control is applied by mechanical (e.g. traps) or biological (make use of natural enemies) techniques. 
Back to top

chemical pest control

Data type code list value

Pest control is executed by application of chemical substances, like pesticides. 
Back to top

Irrigation

Data type: code list

Irrigation is an agricultural measure of bringing out water on the land. It is applied to temporarily or permanently increase the humidity of the top soil and to enable or enhance the plants´ transpiration and capture of nutrients from the soil. This character indicates, if a piece of land is irrigated.

The irrigation method describes the way how water is brought onto the field for plant irrigation. Subtypes of irrigation methods are: 
Back to top

no irrigation

Data type: code list value
Back to top

irrigation applied, method unspecified

Data type: code list value

Irrigation of the plants is present, but particular method is unspecified or unknown. 
Back to top

gravity surface irrigation (flooding, channels)

Data type: code list value

Through surface irrigation the water is leaded to the plants along the ground, either by flooding the whole area or leading the water along small furrows between the crops, using gravity as a force. 
Back to top

sprinkler irrigation (periodic, mobil)

Data type: code list value

Irrigation of the plants by propelling the water under high pressure as artificial rain over the parcels. 
Back to top

irrigation carousel

Data type: code list value

Irrigation of the plants by means of a normally permanently installed sustaining structure above the field, which circulates regularly around a center point of the installation, and distributing water over the field in a circular movement (e.g. clock-wise). 
Back to top

drip irrigation

Data type: code list value

Irrigation of the plants by leading the water directly to the plants drop by drop with perforated fine tubes on the ground or with micro-sprayers to create fog-like conditions. 
Back to top

subsurface drip irrigation

Data type: code list value

Irrigation of the plants by leading the water directly to the plants through perforated fine tubes or tapes that are installed under ground at or below the plant root level. 
Back to top

manual irrigation

Data type: code list value

Irrigation of the plants applied by hand, manually. 
Back to top

Irrigation Water Source

Data type: code list

The Irrigation Source describes where the water for irrigation is taken from. 
Back to top

groundwater

Data type: code list value

Irrigation water is taken out of the ground from ground water aquifers, normally by wells or pumps. 
Back to top

reservoir

Data type: code list value

Irrigation water is taken from a water reservoir like an artificial lake hold back by a dam, or a natural lake. The reservoir can be fed by a natural water course or also can be a rain-fed water cistern. 
Back to top

water course

Data type: code list value

Irrigation water is taken from a running water course like a river or creek. 
Back to top

Drainage

Data type: code list

Drainage as an agricultural measurement is the discharge of water from the soil over and through the soil, resulting in a lowering of the groundwater level. The water can be discharged through drains or ditches. 
Back to top

no drainage

Data type: code list value

This character indicates that no drainage activity takes place on the piece of land. 
Back to top

drainage applied, method unspecified

Data type: code list value

Drainage is apparently installed, but particular method is unspecified or unknown. 
Back to top

open ditch drainage

Data type: code list value

This character indicates that the land is drained by open lengthy ditches or trenches where water can seep out of the surrounding terrain; water may or may not be present on surface of ditch. 
Back to top

subsurface tube drainage

Data type: code list value

This character indicates that land is drained with perforated tubes or pipes that are dug in the ground under the surface where water can percolate into the tube to be discharged. 
Back to top

filled ditch drainage

Data type: code list value

This character indicates that the land is drained by ditches or trenches that are filled with loose material where water can easily percolate through. The filling of this material levels out the surface above the bottom of the ditch with the surrounding terrain on both sides, so that the ditch itself is not an obstacle for trespassing. 
Back to top

Mowing

Data type: heading

This character expresses the kind of mowing activities on a piece of land. A collection of subtypes describe the frequency of mowing during a year. 
Back to top

Mowing Frequency

Data type: code list

no mowing

Data type: code list value

Parcel is not mown. 
Back to top

mowing applied, frequency unspecified

Data type: code list value

Mowing activity is present, but intensity / frequency is not known. 
Back to top

extensive mowing (1 x per year)

Data type: code list value

Parcel is mown one time per year. 
Back to top

medium intensity mowing (2 x per year)

Data type: code list value

Parcel is mown two times per year. 
Back to top

intensive mowing (> 2 x per year)

Data type: code list value

Parcel is mown more than two times per year. 
Back to top

mowing frequency

Data type: integer

Indicates, how often a land unit is mown per year. 
Back to top

code listcode list valuecode listcode list valueshrub clearance

Data type: boolean value

The practice of shrub clearance is applied on grassland areas with the aim to avoid the stepwise succession growing of bushes, shrubs or trees. Like this, the area is kept as an open grassland which is often used for grazing. 
Back to top

biomass burning

Data type: boolean value

Biomass burning is a form of growth control or removal of un-wanted plants on cultivated land by setting it on fire. This measure has also a soil ameliorating effect of by leaving the ashes of burnt biomass on the ground. 
Back to top

liming

Data type: boolean value

Liming is a form of ameliorating the soil by bringing out calcium carbonate with the effect of lowering the soil acidity. It is typically used in coniferous forest areas, but application also in agricultural areas is possible. 
Back to top

pruning

Data type: boolean value
Back to top

Pruning is a kind of horticultural and silvicultural measure for plant growth control which is used to control the growth of plants. It involves the selective removal of parts of a plant by cutting the branches, twigs, buds, or roots. Reasons to prune plants include deadwood removal, shaping (by controlling or directing growth), improving or maintaining health, reducing risk from falling branches, preparing nursery specimens for transplanting, and both harvesting and increasing the yield or quality of flowers and fruits. It is often used in combination with sustaining installation on permanent woody crops and aims at keeping the branches low for easier fruit harvest and also to stimulate the plant´s crop or biomass production. 
Back to top

set aside parcel

Data type: boolean value

This character indicates if a  cultivated land parcel is set aside (i.e. taken out of crop production), meaning that the crop production  is paused for one or more growing seasons, and the parcel is either exposed to natural succession or covered with an intermediate vegetation cover to protect the soil from erosion or improve the soil structure or soil fertility. The purpose of setting aside a parcel is to let the soil recover from intensive cultivation. It can be part of a crop rotation cycle. 
Back to top

Cultivation Purpose

Data type: code list

This category lists a number of cultivation purposes, what the crops are grown for. It can be that the very same crop (e.g. maize) is grown on one parcel as fodder crop, on another parcel as energy crop, and on yet another parcel as food article sugar maize. 
Back to top

alimentary crop production

Data type: code list value

This sub-class applies for all kinds of crop production that are meant to be further used or processed in the human food chain, for example potatoes, cereals, fruit.
Back to top

Grain Production Crop

Data type: code list value

This sub-class applies for all kinds of crop production that are meant to be further used or processed in the human food chain, for example potatoes, cereals, fruit.
Back to top

Table Fruit Crop

Data type: code list value

This sub-class applies for all kinds of crops that are produced for the consumption of whole fruits.
Back to top

Liquefication, Fermentation or Brewing of Crop

Data type: code list value

This sub-class applies for all kinds of crops that are produced for the liquefication, fermentation or brewing of crop, for example to making juice, wine, edible oil, aromatic extract and alike.
Back to top

fodder crop production

Data type: code list value

This sub-class applies for all kinds of crop production that are meant to be fed to animals in the context of husbandry, fodder beet, silage maize or alfalfa. 
Back to top

Harvested For Silage

Harvested For Dry Fodder (Hay, Straw)

Harvested Green For Direct Forage

Temporary Grasses And Grazings

industrial crop production

Data type: code list value

This sub-class applies for all kinds of crop production that are meant to be used in industrial or manufacture processes, for example cotton, linen. 
Back to top

energy crop production

Data type: code list value

This sub-class applies for all kinds of crop production that are meant to be used for energy production, e.g. in a bio-gas facility. 
Back to top

Bio Gas Crop

Liquid Bio Fuel Crop

Solid Bio Fuel Crop

ornamental plants

Data type: code list value

This sub-class applies for all kinds of plant production for ornamental and functional purposes, e.g. to be planted later in urban parks, in residential gardens or other in public spaces. 
Back to top

functional crop purpose

Data type: code list value

This sub-class applies for crop production with multiple purposes. E.g. crops produced for alimentation and where the residues are used for energy production. 
Back to top

Soil Protective Cover Crop

Soil Nutrient Storage Crop

Biodiversity Preservation Crop

Carbon Sequestration Crop

Seed Production Crop

Main Crop

Secondary Crop

Crop Type

Data type: code list

The crop types can be ordered in different ways. The here applied manner aims to focus on the form of the crop, or which part of the plant is harvested. This crop type list follows a compromise between botanical aspects and commonly used aspects to group crops by their apparent form. The latter aspect does not always go conform with botanical hierarchy. For example almonds or coconuts botanically belong to stone fruit, but are grouped here with “nuts”, because they are called so and not the flesh of fruit but the core is the targeted part of the fruit. Olives are also stone fruits, grapes are a sort of berries but both of them receive a stand-alone position because of their outstanding importance in European crop production.

Another reason of a revised crop type list is to clean the list from cultivation purposes. That aspect has been removed and has been given its own matrix segment. 
Back to top

Arable crops

Data type: code list value

Cereals

Data type: code list value

common wheat and spelt

Data type: code list value

durum wheat

Data type: code list value

rye

Data type: code list value

barley

Data type: code list value

oats

Data type: code list value

maize

Data type: code list value

rice

Data type: code list value

millet

Data type: code list value

other cereals

Data type: code list value

Pulses and protein crops

Data type: code list value

peas

Data type: code list value

beans

Data type: code list value

lentils

Data type: code list value

sweet lupins

Data type: code list value

soya

Data type: code list value

chickpeas

Data type: code list value

peanuts

Data type: code list value

other dry pulses

Data type: code list value

Root and Tuber Crops

Data type: code list value

potatoes

Data type: code list value

sweet potatoes

Data type: code list value

sugar beet

Data type: code list value

fodder beet

Data type: code list value

Carrot

Data type: code list value

Parsnip

Data type: code list value

Horseradish

Data type: code list value

Radish

Data type: code list value

Beetroot

Data type: code list value

White Turnip

Data type: code list value

Celery Root

Data type: code list value

Bulb Vegetable Crops

Data type: code list value

Onions

Data type: code list value

Garlic

Data type: code list value

Leeks

Data type: code list value

Cole Crops, Brassica oleraceaData type: code list value

White, Green, Red Cabbage

Data type: code list value

Brussels Sprout

Data type: code list value

Stem Cabbage, Kohlrabi

Data type: code list value

Cauliflower

Data type: code list value

Broccoli

Data type: code list value

Savoy Cabbage

Data type: code list value

Leaf, Stalk, Flowering Vegetable Crops

Data type: code list value

Lettuces, Salads

Data type: code list value

Asparagus

Data type: code list value

Spinach

Data type: code list value

Rods Celery

Data type: code list value

Rhubarb

Data type: code list value

Artichoke

Data type: code list value

Romanesco

Data type: code list value

Fruiting Vegetable Crops

Data type: code list value

Tomato

Data type: code list value

Bell Pepper

Data type: code list value

Chili Pepper

Data type: code list value

Eggplant

Data type: code list value

Cucurbit Crops

Data type: code list value

Cucumbers

Data type: code list value

Zucchini

Data type: code list value

Melons

Data type: code list value

Pumpkins

Data type: code list value

Oilseed or Fibre Crop

Data type: code list value

tobacco

Data type: code list value

hops

Data type: code list value

rape and turnip rape

Data type: code list value

sunflower

Data type: code list value

linseed (oil flax)

Data type: code list value

cotton

Data type: code list value

flax

hemp

Data type: code list value

other oilseed or fibre crops

Data type: code list value

aromatic, medicinal, culinary herbs

Data type: code list value

Flowers and ornamental plants

Data type: code list value

strawberries

Data type: code list value

sugarcane

Data type: code list value

other arable crops

Data type: code list value

fallow land (not crop)

Data type: code list value

Meadow Grasses and Herbs

Data type: code list value

Permanent crops

Data type: code list value

Pome Fruits

Data type: code list value

Apple

Data type: code list value

Pear

Data type: code list value

Quince

Data type: code list value

Medlar

Data type: code list value

Pomegranate

Data type: code list value

Drupes, Stone-Like Fruits

Data type: code list value

Sweet Cherry

Data type: code list value

Sour Cherry

Data type: code list value

Plum

Data type: code list value

Mirabelle, Spillin

Data type: code list value

Peach, Nectarine

Data type: code list value

Apricot

Data type: code list value

Avocado

Data type: code list value

Mango

Data type: code list value

Litchi

Data type: code list value

Date Fruit

Data type: code list value

Oil Palm

Data type: code list value

Nuts

Data type: code list value

Walnuts

Data type: code list value

Chestnuts

Data type: code list value

Hazelnuts

Data type: code list value

Cashew Nuts

Data type: code list value

Brazil Nuts

Data type: code list value

Pekka Nuts

Data type: code list value

Pistachios

Data type: code list value

Almond

Data type: code list value

Coconuts

Data type: code list value

Citrus fruits

Data type: code list value

Oranges

Data type: code list value

Lemons

Data type: code list value

Limes

Data type: code list value

Pomelos

Data type: code list value

Grapefruits

Data type: code list value

Mandarine

Data type: code list value

Clementine

Data type: code list value

Berry Crops

Data type: code list value

Raspberries

Data type: code list value

Blackberries

Data type: code list value

Currants

Data type: code list value

Gooseberries

Data type: code list value

Blueberries

Data type: code list value

Cranberries

Data type: code list value

Juniper

Data type: code list value

Elderberries

Data type: code list value

Cornel

Data type: code list value

Passion Fruit

Data type: code list value

Grapes

Data type: code list value

Olives

Data type: code list value

Other Fleshy Fruits

Banana

Fig Fruit

Kiwi

Papaya

Ananas

Custard Apple

Prickly Pear

Persimmon

Exotic Aromatic and Spice Crops

Coffee Bush

Cocoa Tree

Pepper

Cinnamon Tree

Vanilla

Cloves

Cardamom

Ginger

Leaf Crops from Woody Plants

Tea Bush

Coca Bush

Mate Tree

Woody Plant Nursery

Christmas Trees

Willow, Poplar and Alike Wood Crops

Other permanent crops

Data type: code list value

Aquatic crops

Data type: code list value

Algae crops

Data type: code list value

Unspecified crop

Data type: code list value

Plant growth stage

Data type: code list

The different vegetative (or development) stages of the crops like germination, seedling, vegetative, budding, flowering and ripening. 
Back to top

Sowing the seeds

Data type: code list value
Back to top

germination (sprouting)

Data type: code list value
Back to top

vegetative phase (leaf growth, shooting, tillering)

Data type: code list value
Back to top

reproductive phase (booting, heading, flowering)

Data type: code list value
Back to top

ripening phase (fruit development, maturation, senescence)

Data type: code list value
Back to top

crop residues, harvested field

Data type: code list value

<placeholder>

Crop Season

Data type: code list
Back to top

Crop Season Ongoing

Data type: code list value
Back to top

Summer Crop Season

Data type: code list value
Back to top

Winter Crop Season

Data type: code list value
Back to top

Year-round Crop Season

Data type: code list value
Back to top

Number Of Crop Seasons

Data type: integer value
Back to top

Grazing

Data type: heading

This character expresses if a piece of land is used for grazing, subtypes indicate the grazing intensity (according to the EU IACS).
Back to top

Grazing Intensity

Data type: code list
Back to top

no grazing

Data type: code list value

No grazing activity is observed for this piece of land. 
Back to top

grazing present, intensity unspecified

Data type: code list value

Grazing is present, but intensity is not known. 
Back to top

extensive/ freerange grazing

Data type: code list

Extensive grazing means two or less livestock units per hectare. 
Back to top

intensive grazing

Data type: code list value

Intensive grazing means more than 2 livestock units per hectare. 
Back to top

grazing intensity

Data type: integer

Indicates, how many livestock units per hectare graze on a particular land unit. 
Back to top

Grazing Lifestock per ha

Data type: integer value

Indicates, how many lifestock units (animals) per hectare are grazing on a particular parcel.
Back to top

Grazing Cycle

Data type: code list
Back to top

All Year-round Grazing

Data type: code list value
Back to top

Seasonal Grazing

Data type: code list value
Back to top

Rotational Grazing, Paddocks

Data type: code list value
Back to top

Occasional Grazing

Data type: code list value
Back to top

Unmanaged Natural Grazing

Data type: code list value
Back to top

Grazing Routine

Data type: code list
Back to top

Day Grazing

Data type: code list value
Back to top

Night Grazing

Data type: code list value
Back to top

Day and Night Grazing

Data type: code list value
Back to top

Lifestock Species Type

Data type: code list
Back to top

Cattle

Data type: code list value
Back to top

Horses

Data type: code list value
Back to top

Deer

Data type: code list value
Back to top

Pigs, Boars

Data type: code list value
Back to top

Sheep

Data type: code list value
Back to top

Goats

Data type: code list value
Back to top

Poultry

Data type: code list value
Back to top

Other Grazing Animals

Data type: code list value
Back to top

Forest Land Management

Forest Land includes all land with woody vegetation consistent with thresholds used to

define Forest land in the national  gas inventory. It also includes systems with a

vegetation structure that currently fall below, but in situ could potentially reach the threshold

values used by a country to define the Forest Land category. (Source: 2006 IPCC Guidelines) According to FAO 2015 Forest land must span more than 0.5 hectares with trees higher than 5 meters and a canopy cover of more than 10 percent, or trees able to reach these thresholds in situ.

This segment addresses different characters that further describe the form specific techniques of forestry management. By combining the following characteristics listed under this segment different types of forest constellations can be described.

To separate between native forests (forests that do not contain any non-native trees) and non-native forest (forests containing both native and non-native tree species) use the code list 3.9 Plant Species Origin. 
Back to top

Forest Age Structure

Data type: code list

The forestry age structure gives indication about the cultivation cycles of the addressed forest area and its structure, which is influenced by the forestry practice, silvicultural measures, forestry rotation system, and silvicultural regeneration method. 
Back to top

even-aged stands

Data type: code list value

This characteristic indicates that on a forest parcel all trees are of the same age, because they were planted at the same time, or are by nature of the same generation of trees. 
Back to top

uneven-aged stands

Data type: code list value

A forest parcel where trees are of different ages. 
Back to top

all-aged stands (old-growth forest)

Data type: code list value

In principle, this character applies for forest parcels predominantly composed of self-sown native tree species, independently from any human intervention or afforestation planting. The age of trees and species combination result from natural growth conditions.. Naturally aged forest might be managed to some degree, or be entirely unmanaged (untouched, non-intervention forest, or a strict forest reserve). (Natural Forest standard, Glossary of Terms V1.2). 
Back to top

Forest Age Stage

Data type: code list

This Character addresses the level of approximate age that the trees have reached at the moment of observation. 
Back to top

seedlings

Data type: code list value

This character expresses a very early stage of tree plant growth with an age range of 0 – 5 years. This threshold is not meant to be hard fixed, but as an orientation. 
Back to top

young / mid-aged trees

Data type: code list value

This character expresses an early stage of tree plant growth with an age range of 5 – 15 years. This threshold is not meant to be hard fixed, but as an orientation. 
Back to top

grown-up / mature trees

Data type: code list value

This character expresses an advanced stage of tree growth with an age range of from 20 years and beyond. A tree becomes mature when it starts producing fruits or flowers. How long it will stay productive depends on the species. This threshold is not meant to be hard fixed, but as an orientation. Additional age stages can be added in the model. 
Back to top

Forest Composition

Data type: code list

This Character indicates whether a forest parcel is composed of a mixture of different tree species (can of the same leaf type), or if a forest parcel consists of more or less the same tree species. 
Back to top

mixed forest stands

Data type: code list value

This Character indicates that the forest parcel is composed of a mixture of different tree species (can of the same leaf type). 
Back to top

unmixed forest stands

Data type: code list value

This Character indicates that the forest parcel consists of more or less the same tree species. 
Back to top

Forestry Practice

Data type: code list

Under this segment the main forestry practices are listed, describing the overall strategy how to principally cultivate and harvest woodland. 
Back to top

coppice / Low forest

Data type: code list value

A coppice forest (de: Niederwald) is a traditionally managed woodland, normally with broadleaved trees. This form involves cutting back the trees every 15 to 25 years. The method exploits the capacity of many species of trees to put out new shoots from their stump or roots if cut down. In a coppiced wood, which is called a copse, young tree stems are repeatedly cut down to near ground level, resulting in a stool. New growth emerges, and after a number of years, the coppiced tree is harvested, and the cycle begins anew. 
Back to top

coppice-standard combination

Data type: code list value

This form of forest practice (de: Mittelwald) combines Coppice forest (see above) and High Forest (see below). 
Back to top

high forest

Data type: code list value

High forest (de: Hochwald) is a type of woodland where tree growth originated from self-seeded or planted seedlings and developed in an erect form of the stem. In contrast to a low forest (coppice forest, see above), a high forest in a grown-up stage usually consists of tall, mature trees with a closed canopy. High forests can occur naturally or they can be the result of a plantation and maintenance process. Under human forest management, two form of high forest exist, the parcel-based high forest, or the Plenter forest (see also silvicultural harvesting and regeneration methods) 
Back to top

Forestry Rotation System

The forest rotation system indicates, how long the time intervals are, within which the trees are grown before the trees are felled. The term rotation stands here for forestry cycle.

Data type: code list
Back to top

short rotation coppice (SRC) plantation

Data type: code list value

Short rotation Coppicing (SRC) refers to the growing of trees (usually willow or poplar) in extremely dense stands, harvested at 2-5 years intervals and regenerated from the stools, which are expected to survive 5 rotations at least, before plants need to be replaced be re-planting. 
Back to top

short rotation forestry (SRF) plantation

Data type: code list value

Short rotation forestry (SRF) consists of planting a site and then felling the trees when they have reached a size of typically 10-20 cm diameter at breast height. Depending on tree species this usually takes between 6 and 20 years, and is therefore intermediate in timescale between Short rotation Coppicing (SRC) and conventional long rotation forestry. 
Back to top

intermediate / long rotation forestry

Data type: code list value

Intermediate or long rotation forest is the most common type of forest management practice. The harvesting of the wood takes place when stands have reached an age around 40-50 years and beyond. 
Back to top

continuous cover forestry (CCF)

Data type: code list value

This rotation system is an approach to the sustainable management of forests whereby forest stands are maintained in a permanent manner and with irregular structure, which is created and sustained through the selection and harvesting of individual trees. Clear cutting is not applied. These stands normally involve a mixture of different tree species and ages. 
Back to top

Forest Rejuvenation

Data type: code list

seeded / planted

Data type: code list value

Forest plants are seeded or planted by intention. The rejuvenation process of the forest is initiated by human interaction. In some cases not the entire forest unit is planted, but only in parts, combined with some degree of natural succession. In such a case, the parcel can be described by choosing both types of rejuvenation (man-made and natural). 
Back to top

natural forest succession

Data type: code list value

Forest rejuvenation takes place on a natural basis, no human intervention is involved in the regeneration of the forest. The vegetation reproduces itself through its seeds and fruits. (Does not exclude interventions at a later stage of growth). 
Back to top

self-seeded succession

Data type: code list value

Forest rejuvenation takes place on a natural basis, no human intervention is involved in the regeneration of the forest. The vegetation reproduces itself through its seeds and fruits. (Does not exclude interventions at a later stage of growth). 
Back to top

re-sprouting

Data type: code list value

A special form of natural succession – although eventually triggered by human intervention of coppicing – is the regrowth of shoots out of the roots or the cut down stump of a tree stem. 
Back to top

Silvicultural Measure

Data type: code list

This segment contains a list of silvicultural measures that are applied to the forest get the aimed results of forestry. They are applied to the plants and the soil (analog with the cultivation measures under Agricultural Land Management).

Analog to cultivation measures also forestry measures are activities that are applied to forest parcels in order to support a healthy and efficient growth of trees or to maintain the forest in the intended condition. 
Back to top

Forest cleaning / weeding

Data type: code list value

Cleaning and weeding take place in a very early growth stage in the first years of seedlings developing to young forest stands. The parcel is cleared of unfavorable and concurrent consumer vegetation (herbaceous vegetation, other tree exemplars) that grow in direct surrounding of favorable forest plants, regarding their unwanted consumption of light, water, nutrients or growing space. This procedure can be done in a mechanical or chemical manner. 
Back to top

Forest thinning

Data type: code list value

Thinning is applied to a forest stand to reduce the number of individual trees and to provide better growing conditions to the best developed exemplars. In reducing the number of trees, the provided light, water, nutrients and space. Such selective removal of trees not only help to improve the growth rate but also the health condition of the remaining trees. Overcrowded trees are under competitive stress from their neighbors. Thinning can also contribute to increase the resistance of the stand to environmental stress. 
Back to top

Forest pruning

Data type: code list value

Pruning is a horticultural and silvicultural method involving the selective removal of certain parts of a plant, such as branches, buds, or roots. Pruning is applied with the aim of either influence the growth form of a plant, remove dead or damaged parts of the plant, or to raise the production of fruit. In the context of silviculture, pruning helps also to create greater timber value by reducing the number of branches and knots, and to make the tree focus in vertical growth or crown development. 
Back to top

ring-barking / girdling

Data type: code list value
Back to top

interplanting

Data type: code list value

Diversification of tree species among monocultures with plantation of young mixed stands. Often applied to alter monoculture forestry stands into mixed (needle leaved & broad leaved tree stands).
Back to top

dead wood removal

Data type: code list value
Back to top

forest irrigation

Data type: code list value
Back to top

forest drainage

Data type: code list value
Back to top

forest liming

Data type: code list value
Back to top

forest firebreak

Data type: code list value
Back to top

Silvicultural Harvest and Regeneration Method

Data type: code list

In Forestry, the harvest of timber is very much connected and interlinked with methods of forest regeneration. Therefore, these two aspects come together in the heading of this segment. Here, several forms of forest harvest are listed. The list tries to capture the most common methods, many other variations exist besides. 
Back to top

clearcutting

Data type: code list value

With the clear-cut method, all trees of a parcel are felled at once, no other trees are left standing for further growth. A clear cut is therefore the last step in the process of timber production. After a clear cut – if forestry use is continued on the land unit - , a new plantation cycles starts with replanting seedlings or leave the unit to natural succession. Other than forestry continuation, also orther land use type can take over after a final clear cut. Then it is not anymore under forestry management. This harvesting method can be seen connected with all forestry rotation systems. 
Back to top

shelterwood cutting

Data type: code list value

Shelterwood (de: Schirmschlag)cutting is applied to a mature forest stand. It refers to the progression of forest cuttings by felling more and more single trees, step by step in an evenly distributed manner throughout the woodland parcel. The process stretches over several years, and each time after a felling event, more open space and light becomes available for the rejuvenation on the ground, which takes place in the shadow of the left over still standing shelter trees. Like that, it leads to the establishment of a new generation of seedlings of a particular species or group of species through natural succession without planting. The result of this harvest and regeneration method is a new even-aged forest stand (with only few years difference in age). This harvesting method is connected with long rotation forestry. 
Back to top

femel cutting

Data type: code list value

In the process of Femel cut (Femelschlag), which is similar to the shelterwood cut (Schirmschlag), the mature trees are also felled over a period of a few years. The difference is in the island-like or grouped distribution of harvested trees. I that manner, a mix of shadow-affine and sunlight affine tree species can regenerate in a balanced manner on the parcel. Through the subsequent phases of cutting, the initial spots of clear areas grow bigger in a concentric way, and all of these spots are distributed in a mosaic pattern on the woodland parcel. This harvesting method is connected with long rotation forestry. 
Back to top

strip cutting

Data type: code list value

Also the strip cutting method (de: Saumschlag) is applied to fell the trees in mature forest stands not all at once, but stepwise, stretched over several years. The progression of the felling starts at the outer edge of a woodland parcel, and moves year by years towards the opposite side of the parcel. In doing so, the light and micro-weather conditions change slowly, as more trees are taken out, and also different species can succeed, having different ecological requirements. The cutting is executed in such a way, that the still standing trees protect the cleared area with the succeeding seedlings from the main wind direction. This harvesting method is connected with long rotation forestry. 
Back to top

selection cutting

Data type: code list value

With the selection logging method (de: Plenterwald), not all trees of a parcel are felled at once, but only a number of selected trees that are chosen to be already in the right mature condition for harvesting. Some other trees are left standing either because they have not yet any value for logging or for further growth. This harvesting method is connected with long rotation forestry. Decisions on the felling or keeping of trees are made not only based on their maturity and timber value but also based on the characteristics and ecological processes inherent to the stand site (nature-oriented forestry). The selection cutting is system is seen to be a very sustainable form of forestry, where felling of trees, rejuvenation and regrowth throughout different age classes and mixed species is in an equilibrium, which ensures a continuous forest cover. However, if selection logging is applied continuously intensive beyond any sustainable rationale, selection logging leads to forest degradation, where natural succession can´t keep up due to ecological conditions. 
Back to top

coppicing

Data type: code list value

Coppicing (de: Niederwald) is a woodland management measure of repeated cutting down of tree stems near the ground, generating regrowth of new shoots from the coppiced tree (stool). New shots are harvested in 3-50 year cycles, cycle length being dependent on species (e.g. willow, birch, hazel, poplar, hornbeam, beech, ash, alder, oak) and use of harvested wood (e.g. brushwood, charcoal, poles, withies for wicker-work, firewood, timber). This harvesting method is connected to short rotation forestry systems. 
Back to top

pollarding

Data type: code list value

Pollarding is similar to coppicing, with the difference that the tree is not cut near ground level, but a few meters above ground. The effect of pollarding causes the tree to re-sprout at the head level new thin shoots. The purpose behind this method is to make the tree produce biomass with a lot of leafs which can be used as fodder or stray. Nowadays, it is also applied for esthetic reasons, e.g. along alleys or in urban green areas. With pollarding, the plant can be held in a more or less juvenile state, whereas the trunk can have a high age. 
Back to top

Forest Product Type

Data type: code list

The forestry product type indicates for what kind of products: wood, fiber, bio-energy and/or non-wood forest product  the trees are used. 
Back to top

Wood Product

Data type: code list value
Back to top

timber wood

Data type: code list value

Timber is harvested wood which is used as construction material, e.g. for furniture, buildings, bridges or any other sustaining structure. 
Back to top

energy wood

Data type: code list value

Energy wood is harvested to be used as a combustible material to generate energy through burning the wood or for charcoal production. 
Back to top

wood fibre

Data type: code list value

Fibre as the forestry product type indicates that the harvested wood is used for the production of cellulose as a basic material for pulp or paper. 
Back to top

Non-wood Product

Data type: code list value

Goods derived from forests that are tangible and physical objects of biological origin other than wood. Specifically includes the following regardless of whether from natural forests or plantations: gum arabic, rubber/latex and resin; Christmas trees, cork, bamboo and rattan. Generally excludes products collected in tree stands in agricultural production systems and any woody raw materials and products (FAO 2015). 
Back to top

cork

Data type: code list value

The so called “virgin cork” (to obtain insulating cork boards), is harvested when the cork oak tree is already 25/30 years old, The cork bark used to manufacture the cork stoppers, the so called „female cork“ is harvested 10-12 years. A tree can be harvested a dozen times in its lifetime. 
Back to top

resin

Data type: code list value

Resin is usually collected by causing minor damage to the tree by making a hole far enough into the trunk to puncture the vacuoles, to let sap exit the tree, known as tapping, and then letting the tree repair its damage by filling the wound with resin. 
Back to top

latex, natural rubber

Data type: code list value

Latex is the milky fluid that oozes from any wound to the tree bark of the rubber tree (Hevea brasiliensis or others). Basically cultivated on plantations in the tropics and subtropics, especially in South America, Southeast Asia and Western Africa. 
Back to top

incense

Data type: code list value

A Sort of resin which carries etheric aromas, extracted from Boswellia tree species. It is used for religious or cultic rituals when burnt to spread aromatic smoke.
Back to top

Forest History Type

Data type: code list

primary or virgin forest

Data type: code list value

Naturally regenerated forest of native species, where there are no clearly visible indications of human activities and the ecological processes are not significantly disturbed. There has been no known significant human intervention or the last significant human intervention was long enough ago to have allowed the natural species composition and processes to have become re-established. . Natural damage to the forest as through fire, avalanche, drought or other natural hazards are not in contradiction to the character of primary forest. 
Back to top

secondary or naturally regenerated forest

Data type: code list value

Naturally regenerated forest where there are clearly visible indications of human activities. Includes selectively logged-over areas, areas regenerating following agricultural land areas or recovering from human-induced fires. Includes forests with a mix of naturally regenerated trees and planted/seeded trees, and where the naturally regenerated trees are expected to constitute more than 50 percent of the growing stock at stand maturity or forests where it is not possible to distinguish whether planted or naturally regenerated. 
Back to top

reforestation

Data type: code list value

Re-establishment of forest through planting and/or deliberate seeding on land classified as forest or already in forestry use. Includes planting/seeding of temporarily unstocked forest areas as well as planting/seeding of areas with forest cover. Includes coppice from trees that were originally planted or seeded. 
Excludes natural regeneration of forest (FAO 2015). According to UN IPCCC´s LULUCF (Land Use, Land Use Change, Forest) reporting practices, a time interval of under 20 years can be used here as a threshold for “new” forest.
Example: New planting of seedlings after a clear cut event on forestry used area for timber production, or after a storm damage event. 
Back to top

afforestation

Data type: code list value

Establishment of forest plantations through planting and/or deliberate seeding on land that, until then, was not classified as forest. Implies a transformation from non-forest to forest. According to UN IPCCC´s LULUCF (Land Use, Land Use Change, Forest) reporting practices, a time interval of under 20 years can be used here as a threshold for “new” forest.
Example: Long tradition agricultural areas, grasslands or fallow lands are planted with trees; re- naturated extraction sites are filled up with earth or sand to grow additional forest. 
Back to top

Forest Tree Species

Data type: code list
Back to top

Abies

Data type: code list value
Back to top

Acer

Data type: code list value
Back to top

Aesculus

Data type: code list value
Back to top

Alnus

Data type: code list value
Back to top

Betula

Data type: code list value
Back to top

Carpinus

Data type: code list value
Back to top

Castanea

Data type: code list value
Back to top

Cedrus

Data type: code list value
Back to top

Corylus

Data type: code list value
Back to top

Cupressoideae

Data type: code list value
Back to top

Eucalyptus

Data type: code list value
Back to top

Fagus

Data type: code list value
Back to top

Fraxinus

Data type: code list value
Back to top

Ginkgo

Data type: code list value
Back to top

Juglans

Data type: code list value
Back to top

Larix

Data type: code list value
Back to top

Picea

Data type: code list value
Back to top

Pinus

Data type: code list value
Back to top

Platanus

Data type: code list value
Back to top

Populus

Data type: code list value
Back to top

Pseudotsuga

Data type: code list value
Back to top

Quercus

Data type: code list value
Back to top

Salix

Data type: code list value
Back to top

Taxus

Data type: code ist value
Back to top

Tilia

Data type: code list value
Back to top

Ulmus

Industrial Land Management

Data type: code list

Mining Technique

Data type: code list

Surface Open Pit Mining

Data type: code list value

Underground Mining

Data type: code list value

Under Water Mining

Data type: code list value

Saline Extraction Site

Data type: code list value
Back to top

Leaching/Solution Mining

Data type: code list value
Back to top

Fracking

Data type: code list value
Back to top

Mining Product Type

Data type: code list

This category contains a list of various mining product types, partly grouped. 
Back to top

Fossil Hydrocarbon Fuels

Data type: code list value

Back to top

Coal Lignite

Data type: code list value

Back to top

Crude Oil

Data type: code list value

Oil Sands

Data type: code list value

Mineral Gas

Data type: code list value

Peat

Data type: code list value

Metal Ores

Data type: code list value

Iron Ores

Data type: code list value

Nonferrous Metal Ores

Data type: code list value

Stones Earths

Data type: code list value

Chemical Minerals

Data type: code list value

Salt

Data type: code list value

Other Mining Products

Data type: code list value

Mining End Use Potential Type

Data type: code list

Mining End Use Potential Type Value

Data type: code list value

See the code list in the data specification of the INSPIRE theme Mineral Resources (MR)

Fossil Fuel Type

Data type: code list

Fossil Fuel Type Value

Data type: code list value

See the code list in the data specification of the INSPIRE theme Energy Resources (ER)

Terrain Modification and Land Engineering Measure

Data type: code list

terraced terrain

Data type: code list value

This linear character describes the modification of land surface to equal out a slope with regularly arranged platforms in a stepwise manner. It requires earthworks to make the terraces. Terraces are mainly arranged for agricultural purposes (but not exclusively) in areas where the original relief energy is too high for agricultural activities like sowing, mowing, ploughing, harvesting or flooding irrigation. Typically, the terraces´ edges follow the isohypes of equal height of the slope. 
Back to top

artificial terrain modification

Data type: code list value

This character describes heavily transformed surface, for example with the purpose to level out too steep terrain or in contrary to create higher relief energy.

A special case of application is the leveling and earth modelling of ski pistes in mountainous regions. 
Back to top

artificial snow preparation

Data type: code list value

This character indicates if a piece of land, in particular a ski piste, is prepared with artificial snow, which is made with snow cannons along the ski piste. It is done to meliorate the skiing conditions over the season and also to prolong the duration of skiing season. For this kind of snow-making, often water reservoirs are necessary that can be found nearby the skiing area as small ponds in the slope. 
Back to top

snow farming area

Data type: code list value

This character indicates areas which are used to either collect snow (artificial or natural) over the cold (winter) season, with or without using particular installations or terrain modification. The snow is then covered with specific substances to preserve the snow from melting throughout the warm summer season.
Back to top

avalanche prevention or protection installation

Data type: code list value

This character indicates the presence of specific technical installations or constructions that prevent avalanches to occur or protect area affected or damaged by avalanches. 
Back to top

Administrative regulations, Use constraints

Data type: code list

Access restriction

Data type: code list
Back to top

civil / open

Data type: code list value

Areas of this LCH are publically open to civil society, meaning all people have access to this area. 
Back to top

civil and military

Data type: code list value

Areas of this LCH are accessible to both civilians and to military. 
Back to top

temporary restricted

Data type: code list value

Areas of this LCH are bound to access restrictions of a limited time span, for example after a severe natural disaster or a contamination event. 
Back to top

military only / restricted

Data type: code list value

Areas of this LCH are only accessible by the military, others have no access to this area, (or only under special allowance conditions). 
Back to top

Protected area type

Data type: code list 
Back to top

nature conservation area

Data type: code list value

This area is under strict nature protection status by law. Many human activities like e.g. hiking of the trials, fire making, agricultural, forestry, industrial of urban construction activities etc. are forbidden. 
Back to top

landscape conservation area

Data type: code list value

This area is under landscape protection status. It means that human activities are allowed to some extend (including agriculture and forestry), but the overall appearance of the landscape character shall be preserved. This can include access restrictions for individual traffic by car. 
Back to top

Natura2000

Data type: code list value

This area is listed under European law as a Natura 2000 protected site. 
Back to top

Birds retrieval area

Data type: code list value

This area has strict access limitations, either throughout the entire year or to protect birds during their breeding phase. 
Back to top

Biosphere reserve

Data type: code list value

In areas designated as UNESCO Biosphere Reserves, the focus is on the harmonised management of biological and cultural diversity. It can have a zonal structure, where the inner core areas underlay more strict protection regulation than the outer areas around the center of the reserve area. 
Back to top

Geopark

Data type: code list value

UNESCO Global Geoparks are single, unified geographical areas where sites and landscapes of international geological significance are managed with a holistic concept of protection, education and sustainable development. 
Back to top

National Park

Data type: code list value

A national park is a larger area where nature conservation is the primary purpose, often in combination with recreational use. It stands under authoritative administration, who also provides decent touristic infrastructure. Often it is a reserve of natural, semi-natural, or developed land that a sovereign state declares or owns. The applied rules that count inside a national park, and how it is administered may differ from country to country. Throughout all national parks the common idea is the conservation of 'wild nature' for posterity and as a symbol of national pride. 
Back to top

other protected area types

Data type: code list value 
Back to top

Status / Condition

Data type: code list

Under this segment of LCH, a number of characters are listed that describe the status or the condition of a land unit, and – if damaged – also the reason why. 
Back to top

Land use status

Data type: code list

This Characteristic indicates the status of a Land Unit, in which status or condition it is. 
Back to top

under construction

Data type: code list value

This character applies for areas under construction development, like soil or bedrock excavations or earthworks. It is assigned for areas where landscape is affected by human activities, changed or modified into artificial and man-made surfaces, being in a state of anthropogenic transition. The final use and form of the area being built is already determined, but not necessarily known by one who captures the status. 
Back to top

not in use (never been)

Data type: code list value

Area is not in use and has never been, as far as no traces of human activities can be recognized.
Back to top

out of use (temporary)

Data type: code list value

Area is temporary out of use, but probably will return to being used.
Back to top

abandoned

Data type: code list value

This character expresses that an area belongs is not in use and can't be used any more for the original purpose without major reparation/renovation work. 
Back to top

clear cut

Data type: code list value

Area has been cleared of woody vegetation cover (trees or shrubs).
Back to top

collapsed / destroyed / damaged

Data type: code list value

Status of an object which is not intact or functional anymore due to destruction or collapse. It refers mostly to infrastructural objects or utilities like buildings, constructions or complexes. 
Back to top

managed nature restoration

Data type: code list value

This character applies for areas which are brought back to a natural or at least semi-natural state, so that for example soil sealing elements are removed, ground is decontaminated, vegetation is seeded, naturally given soil moisture condition are restored, etc. Examples are the restoration of natural beds of water courses by giving them more space for meandering, flooding or filling of open pit mines, eventually combined with new vegetation cover etc. 
Back to top

Damage Reason

Data type: code list

This segment contains a collection of possible damage reasons that can affect land units. It is oriented towards the INSPIRE code list on NaturalHazardCategoryValues, that focuses on natural disaster events and calamities. Here, the list is extended to also man-made damaging impact factors. 
Back to top

Geological or Hydrological

Data type: code list value

Processes that have a geological (geosphere) or hydrological (hydrosphere) nature (or origin).

Some of the processes here included are clearly addressed as geological in the scientific literature, such as volcanic hazards or earthquake hazards, whereas other processes cannot be understood without geological and hydrological input, such as certain types of landslides (that can be triggered and mobilised by water), or floods (highly dependent on soil infiltration properties, topography, water table fluctuations), etc. 
Back to top

tsunami

Data type: code list value

Long wave disruption in a large water body reaching emerged land. It can occur as a massive destructive ocean wave caused by sub-marine earthquake or volcanic eruption, or also as an inland water wave caused by a huge landslide plunging into a lake, or by a meteorite impact. 
Back to top

volcanic eruption

Data type: code list value

An opening, or rupture, in the Earth's crust that allows hot magma, ash and gases to escape. Process directly linked to volcanic eruptions are expected to be included in this category, such as lahars, pyroclastic flow, ash fall, volcanic explosions, freatic explosions, etc. 
Back to top

earthquake

Data type: code list value

Earthquake hazards involve the propagation of elastic waves at or near the surface after the release of tectonic stress or other natural sources, such as volcanic explosions or meteorite impacts. Liquefaction, ground shaking and other effects directly caused by seismic waves should also be included in this category. 
Back to top

subsidence and collapse

Data type: code list value

Subsidence and collapse involve mainly vertical downwards ground movement of the surface of the Earth due to different processes of rock or soil weathering or rock compaction to a point where the rock structure cannot bear its own load (collapse) or causing relatively slow downwards movements (subsidence). Subsidence and collapse can be associated with carbonate rocks in karstic areas, but it can occur in other regions such as loessic soils or compressible soils. 
Back to top

landslide

Data type: code list value

Processes of downhill slope movements of soil, rock, and organic materials related to different types of ground failure. Some common terms used for describing different types of landslides include but are not restricted to slides, rock fall, debris flow. 
Back to top

snow avalanche

Data type: code list value

A snow mass with typically a volume greater than 100 m3 and a minimum length of 50 meters that slides rapidly downhill. Snow avalanches usually incorporate materials swept along the path of the snow avalanche, such as trees, rocks, etc. Avalanche formation is the result of a complex interaction between terrain, snow pack and meteorological conditions. 
Back to top

flood

Data type: code list value

Processes of inundation of usually dry (emerged) land, or temporary covering by water of land not normally covered by water. Floods can be of many types (flash floods, river overflow, tidal floods), and can have many triggers (precipitation, natural water reservoir dam failure, river channel obstruction, etc). Tsunamis and a storm surges are usually considered as a different natural hazard. 
Back to top

Meteorological or Climatological

Data type: code list value

Processes that have a meteorological (atmospheric) or climatic (changes in the long-run of environmental variables) nature (or origin). 
Back to top

drought

Sustained and extensive occurrence of below-average water availability, caused by climate variability. Drought should not be confused with aridity, which is a long-term average feature of a dry climate. Likewise, drought should not be confused with water scarcity, which reflects conditions of long-term imbalances between water availability and demands. Droughts can affect both high and low rainfall areas and can develop over short periods of weeks and months or much longer periods of several seasons, years and even decades. 
Back to top

extreme temperature (heat / frost)

Data type: code list value

An abnormal temperature rise or decrease lasting longer than usual temperature rise or drop. Heat waves or cold waves. 
Back to top

storm damage (Tornados, Hurricanes, strong Winds)

Data type: code list value

Violent (high speed) winds. 
Back to top

lightning

Data type: code list value

Discharge of atmospheric electricity, mostly on connection with thunderclouds (cumulonimbus), but can also occur in dry conditions (absence of rain clouds). 
Back to top

storm surge

Data type: code list value

Water pushed from the sea onto the land caused by an atmospheric disruption such as a hurricane or a rapid change in atmospheric pressure. Although a storm surge is a kind of flood, it is usually considered as a separated class. 
Back to top

Fire

Data type: code list value

This category includes all types of processes that involve the occurrence and spreading of fire. 
Back to top

forest fire, wildfire

Data type: code list value

Fire occurrence and spreading on vegetated land. Forest fire means fire which breaks out and spreads on forest and other wooded land or which breaks out on other land and spreads to forest and other wooded land. The definition of forest fire excludes prescribed or controlled burning, usually with the aim of reducing or eliminating the quantity of accumulated fuel on the ground (Regulation EC 2152/2003 - Forest Focus). Wildland Fire: Any fire occurring on wildland regardless of ignition sources, damages or benefits (FAO, 2011, Wildland Fire Management Terminology, FAO, updated September 2010). 
Back to top

underground fire

Data type: code list value

Fire spreading below the surface, typically occurring in soils rich of peat or coal. 
Back to top

Biological

Data type: code list value

Processes that are directly linked to living organisms or products produced by living organisms. 
Back to top

insect infestation

Data type: code list value
Back to top

mould infestation

Data type: code list value
Back to top

epidemic

Data type: code list value

An outbreak of a disease that spreads rapidly among individuals in an area or population. 
Back to top

allergens

Data type: code list value

Biological products or substances (such as pollen) that might cause allergy over a large number of people. 
Back to top

animal browsing / overgrazing

Data type: code list value

Damaged vegetation by animals biting and chewing off plant parts, or animals too extreme grazing (tearing out even root parts) leaves vegetation no time to recover. 
Back to top

animal trampling

Data type: code list value

Animal trampling causes degeneration of vegetation cover. 
Back to top

Cosmic

Data type: code list value

Processes from outer space. 
Back to top

meteorite impact

Data type: code list value

Solid materials from outer space reaching the Earth. 
Back to top

magnetic disruption

Data type: code list value
Back to top

solar and cosmic radiation

Data type: code list value

Radiation from outer space (UV, gamma ray, etc). 
Back to top

Contamination or pollution

Data type: code list 
Back to top

radioactive

Data type: code list value
Back to top

chemical / toxic

Data type: code list value
Back to top

soil salinization

Data type: code list value
Back to top

oil contaminated

Data type: code list value
Back to top

explosives (military ammunition / industrial)

Data type: code list value
Back to top

plastic (micro & macro)

Data type: code list value
Back to top

Intentional / Accidental Structural Damage

Data type: code list value
Back to top

structural decay

Data type: code list value
Back to top

demolition, explosion

Data type: code list value
Back to top

Degenerative process

Data type: code list
Back to top

deforestation

Data type: code list value

Loss of tree cover by natural or man-made processes like drought, storm damage, clear cuts, soil erosion etc.
Back to top

soil erosion

Data type: code list value

Physical loss of (fertile) top soil through erosive transport of material due to wind, water flow or gravitational processes.
Back to top

desertification

Data type: code list value

Transition from vegetated land to desert conditions. It happens due to long-lasting lack of precipitation over the stretch of several years due to climate change or droughts, often in combination with human or animal impacts like salinization of the soil (due to bad irrigation management), cutting off water streams, overgrazing or deforestation. Like that, the surface loses continuously its vegetation cover and turns into bare and dry surface, often accompanied by aeolian sediments (sand and dust) that cover the surface. 
Back to top

glacier melting

Data type: code list value

Areas where glaciers have reduced in their spatial (vertical or horizontal) extent by melting due to raising average temperatures.
Back to top

Geographical Characteristics

Habitat Context

Data type: Code list

The entries under habitat context describe the environmental surrounding in its role as a habitat where human beings, animals or plants live, roam or grow. The definitions of the habitat context largely follow the EUNIS habitat types (https://eunis.eea.europa.eu/habitats.jsp) 
Back to top

Marine Habitats

Data type: Code list value
Back to top

Coastal Habitats

Data type: Code list value
Back to top

Inland Surface Waters

Data type: Code list value
Back to top

Mires, Bogs and Fens

Data type: Code list value
Back to top

Grasslands and Lands Dominated By Forbs, Mosses Or Lichens

Data type: Code list value
Back to top

Heathland, Scrub and Tundra

Data type: Code list value
Back to top

Woodland, Forest and Other Wooded Land

Data type: Code list value
Back to top

Inland Unvegetated Or Sparsely Vegetated Habitats

Data type: Code list value
Back to top

Regularly Or Recently Cultivated Agricultural, Horticultural and Domestic Habitats

Data type: Code list value
Back to top

Constructed, Industrial and Other Artificial Habitats

Data type: Code list value
Back to top

Habitat Complexes

Data type: Code list value
Back to top

Lagoon

Data type: Code list value
Back to top

Estuary

Data type: Code list value
Back to top

Marine Inlets, Fjords

Data type: Code list value
Back to top

Coastal Salt Marsh

Data type: Code list value
Back to top

Intertidal Flat

Data type: Code list value
Back to top

Height Zone / Altitudinal Zone

Altitudinal zoning in general is applied to give structure to the phenomenon that nature changes with increasing altitude in mountainous regions due to varying environmental and climate conditions. Factors like air temperature, air humidity, soil/rock composition, solar radiation, geographical aspect/exposure (azimuth) are important factors in determining altitudinal zones.

Altitudinal zones therefore do not simply relate to absolute elevation or measurable height above sea level. On the same height about sea level, two different altitudinal zones can occur in the southern and northern exposure side of a mountain range.

Data type: code list
Back to top

planar

Data type: code list value
Back to top

collin

Data type: code list value
Back to top

submontane

Data type: code list value
Back to top

montane

Data type: code list value
Back to top

high montane

Data type: code list value
Back to top

subalpine

Data type: code list value
Back to top

alpine

This zone he zone stretches between the upper tree line, where trees end to grow, and lower snowline, where nival zone begins. This zone can further be broken down into Sub-Nival and Treeless Alpine (low-Alpine). Vegetation cover is restricted to herbaceous and sporadic dwarf shrubs or dwarfs trees.
Sub-Nival: This zone is under influence of low temperatures, seasonal frost and strong winds. Vegetation cover is patchy, restricted to favorable growing locations, and limited to herbaceous vegetation types typical for arctic regions. Snow cover is found for part of the year.

Low-Alpine: Vegetation cover is continuous, dominated by alpine meadows, shrubs and sporadic dwarf trees. Due to consistent freezing temperatures tree growth is severely limited.

Data type: code list value
Back to top

nival

Highest altitudinal zone, throughout the entire or most of the year covered with snow / ice. This zone is the origin of glaciers. Vegetation is non-existent in this zone, apart from extremely scarse micro organisms.

Data type: code list value
Back to top

Geographical Context

Data type: code list

The geographical context gives an indication about the larger context of a land unit, in what kind of spatial surrounding it is embedded. The borders of such regions are often rather fluent, and it is rather difficult to draw a distinct boarder line between them. Still, it can help to understand the overall landscape situation in general. These categories are not connected to any formalistic terms that may have fiscal or legal consequences for the described land unit. This code list is open and may be extended by the user. Also, the geographical context types are on purpose not mutually exclusive in their meaning and can overlap in space. 
Back to top

inland

Data type: code list value

The inland areas are the opponent of coastal areas. These areas are distant enough from the coast and are not anymore directly influenced by it. 
Back to top

coastal

Data type: code list value

The coastal context gives an indication about land being close to the sea shore areas. The vicinity to the coast has an influence on the location of cities, harbors as well as on socio-economic activities factors like commercial and industrial activities, tourism, but also cultural aspects. Along coastal areas, specific landscape types or habitats can be found in contrary to more distant locations from the coast. 
Back to top

island

Data type: code list value

Island areas are under influence of the surrounding sea. This has an effect on the transportation connectivity to the mainland. It is a special geographic situation. An island also can have inland and coastal areas. 
Back to top

oceanic

Data type: code list value

Oceanic areas are located at open sea, with considerable distance to any coastline, from the continental shelf zone towards deep sea areas. 
Back to top

urban

Data type: code list value

Urban addresses the context of cities and their sub-urban areas. Besides some other landscape types, areas occupied with settlements have a dominant appearance. Also more or less dense transportation networks are part of urban areas. The population density is rather high. 
Back to top

rural

Data type: code list value

Rural areas are dominated by agricultural land or forests. Settlement areas are present but play an inferior role. This type of landscape context does not include bigger cities (which would be urban context). The population density is rather medium to low. 
Back to top

riparian

Data type: code list value

Riparian areas are found along river banks. They are exposed to a rivers variance in water level and to seasonal or occasional inundations. The terrain is mostly flat. In riparian areas specific types of vegetation with hydrophilic plants can be found that are adapted to the situation of being flooded. 
Back to top

mountain

Data type: code list value

Mountain regions are under the influence of mountains. These kind of spatial context includes the valleys between the mountains themselves. Mountainous regions are characterized by high relief energy of the terrain. Different climate and vegetation zones as well as different forms of human settlements and cultivation practices can be found here. 
Back to top

Geomorphological landform type

Data type: heading

With this character, a geomorphological form can be noted manually as text string. In the current version of EAGLE matrix / model this character is a kind of placeholder. To cover all kinds of geomorphological forms and make a comprehensive code list is at this stage not foreseen. It may be considered to integrate such a code list at a later stage. 
Examples: 
Cave, Cliff, Coral reef, Doline (Karstic), Drumlin, Fumarole, Geysir, Glacier moraine, Gorge, Gully, Lava field, Meteor crater, Sand bank, Sand dune, Volcanic caldera, Volcanic cone, Volcanic crater. 
Back to top

Geomorphological landform

Data type: text string
Back to top

Littoral Zone

Data type: code list

This category applies for coastal shorelines as well as for inland river banks and lakesides. It helps to subdivide the littoral complex into zones that can be used to describe habitat types (basically wetlands and alike) in the vicinity of water surfaces. 
Back to top

Epilittoral

Data type: code list value

The epilittoral zones is beyond any direct influence of water cover along a sea shore line or a inland lakeside. The area is only influenced indirectly by a high ground water level and moist (and eventually salty) soil conditions. Only extreme storm surges reach here, or rare inland flood events along riparian areas. Vegetation cover, if present, is dominated by hygrophilous plants. 
Back to top

Supralittoral

Data type: code list value

The supralittoral zone is the splash and spray zone high enough above the water line to not be frequently covered with water or waves. Still this area is influenced by spray water, either in coastal areas from sea water or along inland water bodies. Only during storm surges or irregular water peak level events these areas are exposed to being flooded.

Example: Coastal saltmarshes or young dunes in transition from halophilous pioneer vegetation to halophile vegetation. 
Back to top

Eulittoral/Intertidal

Data type: code list value

In coastal areas, the eulittoral or intertidal zone stretches from the low tide line to the high tide water line. It is subjected to constant change between being submerged and uncovered from water by the tides. Connected to inland water bodies it is the river bank or lakeside where waves constantly run ashore. Due to its high erosive and/or sedimentary dynamics, plants usually are not able to take roots.
Back to top

Infralittoral/Sublittoral

Data type: code list value

As part of coastal areas as well as connected to inland water bodies, the sublittoral or also called subtidal zone addresses the area where the earths surface is normally constantly covered by water. In coastal areas the sublittoral zone stretches from the low tide line across the shallow sea towards the continental shelf. It is inhabited by algae, corals etc. where photosynthesis is still possible under water. In inland water bodies, the sublittoral or infralittoral zone embraces the area of a lake, where taller terrestrial plants can still build roots. This zone often is also the breading zone for water animals. 
Back to top

Physical Climate Zone

Data type: code list

This list contains different types of general climate zones in a coarse approach. Sometimes it may be useful o necessary to assign a climate zone to certain elements, like for example certain vegetation (or crop) types which are native in particular climate zones. 
Back to top

Tropical Climate Zone

Data type: code list value
Back to top

Subtropical Climate Zone

Data type: code list value
Back to top

Temperate Climate Zone

Data type: code list value
Back to top

Polar and Subpolar Climate Zone

Data type: code list value
Back to top

Terrain Characteristics

Terrain Parameters

Data type: Heading

This parameter collection carries parametric measurable information about elevation, slope and aspect of terrain point. These data is best handled in form of raster data.
Back to top

Terrain Elevation

Data type: numeric value

This parameter carries the absolute height above sea level in meters.
Back to top

Terrain Slope

Data type: numeric value

This parameter carries the steepness or gradient of an ascending terrain. It can be measured in percentage (ratio between vertical difference in height and horizontal distance) or in angle degrees, where 100% corresponds with 90°. In EAGLE context is proposed to use angle degrees. 
Back to top

Terrain Aspect, Exposition

Data type: numeric value

This parameter carries the orientation aspect of an inclined terrain surface towards cardinal directions. It is expressed in 0° to 360°, where 0° = North, 90° = East, 180° = South, 270° = West, among other values in between.
Back to top

Spatial Characteristics

Arrangement in space and the inner structure of land units. This character category is further subdivided into several pattern types and their instances. 
Back to top

Spatial Distribution Patterns

Data type: code list

Under spatial distribution patterns 2-dimensional spatial patterns give indications on how specific land cover components are distributed in space and what kind of spatial relation they have to each other. There are four kinds of spatial patterns preset, but can be extended: 
Back to top

Homogenous (evenly textured)

Data type: code list value

The spatial pattern “homogenous” indicates that a certain land surface unit is uniform in structure, and composition and evenly textured. Every spot inside the unit is supposed to be of the same kind.

Example: The lawn of a football field is homogenous. Fruit trees in a plantation are arranged in a homogenous way. 
Back to top

Heterogenous / mixed (unevenly textured)

Data type: code list value

The spatial pattern “heterogeneous” describes the more or less equal distribution of several features inside a land unit, with an uneven texture. Certain different land cover components that may occur inside a land unit are distributed in a mixed manner. A varying density in distribution can be the appearance of heterogeneity.

Example: a mixed forest consists out of mixed stands of broad leaved trees and needle leaved trees. 
Back to top

Mosaic (clearly distinct patches)

Data type: code list value

The spatial pattern “mosaic” describes the distribution of a feature in space in relation and together with other surrounding features. A land unit with a mosaic pattern contains several sub- units/components that can be identified as stand-alone sub-parts of the larger “mosaic” unit holding them together. It is applied for land surface units on a higher abstracted level. The single parts inside the mosaic may have their own spatial pattern (e.g. homogenous or heterogeneous) to be distinguishable. Altogether, the sub-units inside the characterized “mosaic” unit form a patchwork pattern, where each patch is clearly distinct.

Example: a forest area with appearing grassland clearings in between. 
Back to top

Scattered (sporadic texture)

Data type: code list value

The spatial pattern “scattered” describes the scattered distribution of a feature in space. It indicates that the specific feature does occur only sporadically, is not dominating and distributed not equally. 
Back to top

Regular spacing

Data type: code list value

The spatial pattern “regular spacing” describes the spatial arrangement of all kinds of landscape objects in a regular form, where the space between the objects follow an evenly distributed pattern. This can apply for artificial objects (e.g. houses in built-up areas, wind turbines, solar panels etc.) or woody crop plantations (e.g. olive or fruit trees, Christmas trees, vineyards), and others.
Back to top

Linear feature patterns

Data type: code list

This collection of linear landscape pattern has been set up to capture information about certain linear elements in landscape that are too small or narrow to be capture as single features, but still can have a significant influence on the character of a land surface unit. The linear patterns are not exclusive and may occur in combinations. Four subtypes are here to distinguish (others can be added): 
Back to top

hedge rows

Data type: code list value

The character hedge rows indicate that a land parcel is surrounded by or contains several hedge rows. Through them, the land parcel has a kind of sub-parcel structure. Mostly occurs in agricultural areas. 
Back to top

rows of trees

Data type: code list value

The character rows of trees indicate that a land parcel is surrounded by or contains several rows of trees. Through them, the land parcel has a kind of sub-parcel structure. Occurs for example in rural areas, but also along alleys inside urban zones. 
Back to top

stone walls

Data type: code list value

The character “stone walls” indicate that a land parcel is surrounded by or contains several stone walls. This pattern refers to man-made piled up dry stone walls that are part of old traditional agricultural areas, typically can be found in rural areas of southern European countries. From a habitat perspective, those stone walls are valuable objects that offer refugee for reptiles and insects. 
Back to top

Linear (technical) Networks

Data type: code list

This character describes the presence of some linear technical networks that are distributed within a certain land unit and that give a typical structure to it. It may have some small roads, ditches, fences, wires, pipelines or similar linear network elements. Some values are pre-defined, others can be added to the open code list. 
Back to top

roads / railways

Data type: code list value

A land unit can contain a clearly recognizable network of roads which give a special character to the land unit. The location or geometry of the road network is not necessarily known, but only its presence. 
Back to top

pipelines

Data type: code list value

A land unit can contain a single track or a network of pipelines. The location or geometry is not necessarily known, but only its presence. 
Back to top

Vertical position

Data type: code list

The elements under Vertical position describe the position of an object in landscape, whether it is located on the ground surface, suspended/elevated, or under the ground surface. Sometimes it may be necessary or useful to have information of such positioning of objects in landscape, for example when describing roads over bridges across rivers.
Back to top

on ground surface

Data type: code list value
Back to top

suspended / elevated

Data type: code list value
Back to top

under ground

Data type: code list value
Back to top

Object Characteristics

Geometric parameters

object area size

Data type: numeric value

This parameter expresses the area size of an object. The measuring unit is in square meters. 
Back to top

object length

Data type: numeric value

This parameter expresses the length of an object. The measuring unit is in meters. 
Back to top

object width

Data type: numeric value

This parameter expresses the width of an object. The measuring unit is in meters. 
Back to top

object height

Data type: numeric value

This parameter expresses the height of an object, meaning not height above sea level but how tall an object is. The measuring unit is in meters. 
Back to top

object area coverage

Data type: Integer percentage value

This parameter expresses the percentage share of a Land Cover Component within a larger land unit. 
Back to top

Occurrence Type

This segment describes how many of certain elements occur in the given land unit.

Percentage occurrence

Data type: percentage value

How big is the percentage share – expressed a relative figure - of these particular objects among all other distinct objects. 
Back to top

Countable occurrence

Data type: numeric value

How many of these objects occur counted in absolute numbers in the given land unit. 
Back to top

Present occurrence

Data type: boolean

The addressed type of element is at all present in the given land unit, however a discret number and quantity cannot be specified. 
Back to top

Temporal Parameters

Instant event date

Data type: TM_Position

This LCH stores the instant date of a certain event without considerable temporal duration, counted from 1st January onwards into the year. It applies for sudden events that lead to a temporary or long-term change of landscape situation. 
Back to top

duration

Data type: TM_PeriodDuration (from ISO 19108 – Temporal Schema)

This LCH can store the relative length (seasonal duration) of a certain situation or status of a land unit. Measure unit is to be chosen, e.g. days/ weeks/ months. 
Back to top

period

Data type: TM_Period

This LCH can store the period of an event or status which is limited by two definite positions in time. IT is determined by an explicit start and end date. 
Back to top

period start date

Data type: TM_Date

This LCH stores the start date of a certain period in form of the number of a day, counted from 1st January onwards into the year.
Back to top

period end date

Data type: TM_Date

This LCH stores the end date of a certain period in form of the number of a day, counted from 1st January onwards into the year. 
Back to top

recurring frequency

Data type: integer value

Seasonal frequency to store the number of changes per year, how often a situation is changing.

/recurring. 
Back to top

# IV. Matrix block: EAGLE METADATA (EMD)

# Feature life cycle

Data type: text heading

BeginLifespan

Data type: date

This Parameter expresses the beginning of existence of the real world feature. Due to a given data situation, this point in time can´t always be determined explicitly. 
Back to top

EndLifespan

Data type: date

This Parameter expresses the date when the existence of the real world feature has come to an end. Due to a given data situation, this point in time can´t always be determined explicitly. 
Back to top

Database edit dates

Data type: text heading

Entered in DB

Data type: date

This Parameter expresses the date when a feature has been entered in the database. This date may differ from the real date when the feature came to existence (see BeginLifeSpan). 
Back to top

Last Modified in DB

Data type: date

This Parameter expresses the date when a feature has been modified in the database and has existed already before. This date may differ from the real date when the feature came to existence (see BeginLifeSpan). 
Back to top

Deleted from DB

Data type: date

This Parameter expresses the date when a feature has been deleted and removed from the database, so to speak has been retired. This deletion date may differ from the real date when the existence of the real world feature came to an end (see EndLifeSpan). 
Back to top

Data Acquisition Date

Data type: date

This Parameter expresses the date when the source of information las been recorded, which was used to map a feature and/or determine its thematic content. 
Back to top

Data Source Type

Data type: code list

This Characteristic tells the source of information, which is the basis for the captured information on the Earth´s surface. 
Back to top

Satellite imagery

Data type: code list value
Back to top

Air-borne Sensor

Data type: code list value
Back to top

In-Situ data collection

Data type: code list value
Back to top

Geometric specifications

Data type: text heading

Minimum Mapping Unit (MMU)

Data type: Numeric value

The minimum mapping unit indicates which area size [in square meters] a feature at least has to have to be captured and stored as a geometry in a dataset. This parameter shall not fix a specific MMU that may fit for all mapping initiatives. Instead, this parameter is meant to be a placeholder, which shall be filled by the user who applies the matrix on a specific classification system with its specific data capture rules including a particular MMU. 
Back to top

Minimum Mapping Width (MMW)

Data type: Numeric value

The minimum mapping width indicates how broad [in meters] in spatial lateral extension a feature at least has to have to be captured and stored as a geometry in a dataset. This parameter shall not fix a specific MMW that may fit for all mapping initiatives. Instead, this parameter is meant to be a placeholder, which shall be filled by the user who applies the matrix on a specific classification system with its specific data capture rules including a particular MMW. 
Back to top

# References

Arnold, S., Kosztra, B., Banko, G., Smith, G., Hazeu, G., Bock, M., Valcarcel Sanz, N. (2013): The EAGLE concept – A vision of a future European Land Monitoring Framework.

In: R. Lasaponara, L. Masini and M. Biscione (Eds.). Towards Horizon 2020: Earth Observation and Social Perspectives. 33th EARSeL Symposium Proceedings, pp. 551-568. Matera: EARSeL and CNR.

EEA (2007): CLC2006 technical guidelines. EEA Technical Report No 17/2007. URL: http://www.eea.europa.eu/publications/technical_report_2007_17 (last access: 25.06.2013)

EEA (2017): Updated CLC illustrated nomenclature guidelines. Service Contract No 3436/R0-Copernicus/EEA.57441Task 3, D3.1 –Part 1. https://land.copernicus.eu/user-corner/technical-library/corine-land-cover-nomenclature-guidelines/docs/pdf/CLC2018_Nomenclature_illustrated_guide_20190510.pdf

EAGLE website URL: https://land.copernicus.eu/eagle

Ford-Robertson, F.C. (ed). (1971): Terminology of Forest Science, Technology Practice and Products. Society of American Foresters, Washington DC.

INSPIRE data specifications, collection of all themes: http://inspire.ec.europa.eu/index.cfm/pageid/2

INSPIRE, 2013a: D2.8.II.2 Data Specification on Land cover – Draft Technical Guidelines version 3 (Identifier D2.8.II.2_v3.0rc3).

URL:http://inspire.ec.europa.eu/documents/Data_Specifications/INSPIRE_DataSpecification_LC_v3. 0.pdf

INSPIRE, 2013b: D2.8.III.2 Data Specification on Buildings – Draft Technical Guidelines version 3 (Identifier D2.8.III.2_v3.0rc3).

URL:http://inspire.ec.europa.eu/documents/Data_Specifications/INSPIRE_DataSpecification_BU_v3. 0.pdf

INSPIRE, 2013c: D2.8.III.4 Data Specification on Land use – Draft Technical Guidelines version 3 (Identifier D2.8.III.4_v3.0rc3).

URL:http://inspire.ec.europa.eu/documents/Data_Specifications/INSPIRE_DataSpecification_LU_v3. 0.pdf

INSPIRE, 2013d: D2.8.III.12 Data Specification on Natural Risk Zones – Draft Technical Guidelines version 3 (Identifier D2.8.III.12_v3.0rc3).

URL:http://inspire.ec.europa.eu/documents/Data_Specifications/INSPIRE_DataSpecification_NZ_v3. 0.pdf

INSPIRE, 2013e: D2.8.III.18 Data Specification on Habitats and Biotopes – Draft Technical Guidelines version 3 (Identifier D2.8.III.18_v3.0rc3).

URL:http://inspire.ec.europa.eu/documents/Data_Specifications/INSPIRE_DataSpecification_HB_v3. 0.pdf

INSPIRE, 2013f: D2.8.III.16 Data Specification on Sea regions – Draft Technical Guidelines version 3 (Identifier D2.8.III.16_v3.0rc3).

URL:http://inspire.ec.europa.eu/documents/Data_Specifications/INSPIRE_DataSpecification_SR_v3. 0.pdf

ISO 19144-2, TC 211 N 3265, Geographic information — Classification systems — Part 2: Land Cover Meta Language (LCML) (version 2011-12-16).

Kuechler, A.W. & Zonneveld, I.S. (eds). (1988): Handbook of Vegetation Science. Dordrecht, the Netherlands: Kluwer Academic Publishers.