Keywords

Type hier de trefwoorden

Summary

An analytical report containing an overview of data and information holdings of the RSCs, an analysis of information flow processes and management systems at RSCs. Where appropriate, recommendations for further work to support harmonisation of data and information flows at regional and European level for mutual benefit will be made. This is a joint deliverable for Tasks 1 and 2 (as agreed in project contract).

This draft report D2 is prepared as a background document for the TG-Data meeting of 29-30 April, in Copenhagen. The current focus of the report is on Eutrophication and Hazardous substances. The project Tasks 1 and 2 are to include an inventory all substances of relevance for the MSFD reporting. The final version of D2 will take into account the response and comments to this draft report from the TG-Data meeting and it will be extended to include the complete inventory.

Referenties

Type hier de referenties

| Versie | Datum | Auteur | Paraaf | Review | Paraaf | Goedkeuring | Paraaf |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Apr. 2014 | Nicky Villars |  | Sharon Tatman |  | Frank Hoozemans |  |
|  |  | Myra van der Meulen |  |  |  |  |  |

State draft This is a draft report, intended for discussion purposes only. No part of this report may be relied upon by either principals or third parties.

Contents

1	Introduction	1

1.1	Background	1

1.2	Objectives	2

1.3	Methods	2

2	RSC and EEA data holdings and data flows related to eutrophication	5

2.1	RSCs eutrophication indicators in relation  to MSFD and EEA indicators	5

2.2	RSCs methodologies for eutrophication indicators and assessment tools	11

2.3	RSCs data streams analysis for identifying eutrophication indicators	25

2.4	Categorization of RSCs eutrophication indicators according to the DPSIR approach	35

3	RSC and EEA data holdings and data flows related to hazardous substances	37

3.1	RSCs hazardous substances indicators in relation to MSFD  and EEA indicators	37

3.2	RSCs methodologies for hazardous substances indicators and assessment tools	48

3.3	RSCs data streams analysis for identifying eutrophication indicators	56

3.4	Categorization of RSCs hazardous substances indicators according to the DPSIR approach	70

4	Summary	75

5	Following steps to complete the inventory	77

6	References	79

Appendices

A	Inventory of RSCs data streams on eutrophication 	A-1

B	Inventory of RSCs data streams on hazardous substances	A-2

# Introduction

## Background

The Marine Strategy Framework Directive (MSFD, 2008/56/EC) “establishes a framework within which Member States shall take the necessary measures to achieve or maintain good environmental status in the marine environment by 2020 at the latest” [Art1(1)]. For that purpose the Member States have to develop Marine Strategies which apply an ecosystem-based approach to the management of human activities. In addition, the MSFD is to “contribute to coherence between, and aim to ensure the integration of environmental concerns into, the different policies, agreements and legislative measures which have an impact on the marine environment” [Art1 (4)]. Such policies would include Community legislation and International Conventions.

The MSFD places a range of requirements on Member States, through the preparation of marine strategies, to assess the state of the marine environment, its pressures, environmental impacts and uses, and to establish monitoring programmes, environmental targets and measures in order to achieve good environmental status (GES). The directive requires Member States to cooperate within and across Europe's regional seas, including via the Regional Sea Conventions (RSC), in order to achieve the objectives of the directive in a consistent and coordinated manner.

The need to integrate information systems across policies in order to support policy implementation has been outlined at the 9th meeting of WG DIKE in the document “Integration and streamlining of marine reporting and data” (DIKE_9-2014-05). The document points out that in order to fully benefit from the reported information across all relevant policies and to ensure its preparation, reporting and access are as efficient as possible, it is necessary to more systematically review current reporting processes.

A review of the links between MSFD reporting requirements and reporting under other EU Directives and international agreements such as the Regional Sea Conventions (RSCs) has been undertaken by MRAG, UNEP-WCMC, URS (2013). This review focused on reporting requirements but whether or not Member States (MS) or Contracting parties (CP) are fulfilling their obligations was not investigated. As regards RSCs, the review did not include some aspects relevant to the MSFD reporting framework, such as objectives, assessments and targets that are undertaken collectively by each Convention rather than by each MS (as Contracting Party) as a reporting requirement.

The Service Contract “Development of a shared data and information system between the EU and the Regional Sea Conventions” is examining the data and information holdings within each of the four Regional Sea Conventions as well as the EEA, with the aim of characterizing the present information flow processes in place across Europe, in the light of their ability to support the MSFD and WFD objectives.  Hence the work includes information developed by each Convention and EEA, with a view to identifying those data and information streams of most relevance to both regional and MSFD needs (e.g. contributing to common indicator assessments).

The four Regional Sea Conventions which cover EU marine regions or sub-regions are:

- The Convention for the Protection of the Marine Environment in the North-East Atlantic of 1992 (further to earlier versions of 1972 and 1974) – the OSPAR Convention (OSPAR);

- The Convention on the Protection of the Marine Environment in the Baltic Sea Area of 1992 (further to the earlier version of 1974) – the Helsinki Convention (HELCOM);

- The Convention for the Protection of Marine Environment and the Coastal Region of the Mediterranean of 1995 (further to the earlier version of 1976) – the Barcelona Convention, implemented by UNEP/MAP;

- The Convention for the Protection of the Black Sea of 1992 – the Bucharest Convention, implemented by the Black Sea Commission (BSC).

- Together with the Eionet, EEA manages and updates the WISE-SoE data flows and has developed three pan European indicators: CSI 021 (nutrients), CSI 023 (chl-a) and MAR 001 (Hazardous substances in biota) which are regularly updated.

## Objectives

This report is part of Deliverable 2 ‘Review and analysis of RSC information systems’ of the Service Contract ‘Development of a shared data and information system between the EU and the Regional Sea Conventions‘ The objectives mentioned in the Service Request with respect to Deliverable 2 are:

a. Review the existing and anticipated data and information flows from MSs (as Contracting Parties) to RSCs (or developed directly by/within the RSCs) and assess their potential to contribute to MSFD implementation needs;

b. Review the processes and systems for acquiring, managing and making available these data and information and identify where these could be improved, including how these data and information flows could be streamlined and harmonised between the MSs, the RSCs and the EU/EEA in the context of MSFD and other relevant marine policies;

This report presents a review of the data and information holdings within each of the four RSCs, as well as the respective flow processes and management systems for the topics eutrophication and hazardous substances with the view to clarify to what extent RSC activities can support EU policies; in particular the MSFD and the WFD. The review addresses the following aspects:

- Content including data, data products, status assessments etc with focus on indicators.

- Processes. Who provides the data and how is information flowing and managed.

## Methods

Information was collected from the web sites of the RSCs and direct communication to the RSCs Secretariats. Meetings with the RSCs were held: Deltares and AZTI met with OSPAR on Feb 5-6; SYKE and Deltares met with HELCOM on March 4; HCMR held brief meetings with UNEP/MAP and BSC on January 28 followed by email communications.

Two inventories were conducted:

- one on data, data products, status assessments, targets, measures etc and

- one on data flows within the Regions relevant to eutrophication and hazardous substances.

The information included in the inventories was used to provide an overview of the indicators established or proposed by each RSC, what they are used for and on which methodologies and data they are based. The RSCs indicators were assessed in terms of relevance to the MSFD indicators, as well as to EEA indicators. Emphasis was given on the data flows that can produce MSFD and RSCs indicators. Thus existing RSCs data flows were assessed in comparison to parameters reported by the MS under articles 8, 9 and 10 of the MSFD using the results of the “In-Depth Assessment of the EU Member States’ Submissions for the Marine Strategy Framework Directive under articles 8, 9 and 10” undertaken by the Joint Research Centre (JRC) (Palialexis et. al, 2014).

Furthermore, the inventories are going to be used as input in the Information Flow Scheme that is currently in development under Task 3 of the project.

# RSC and EEA data holdings and data flows related to eutrophication

## RSCs eutrophication indicators in relation to MSFD and EEA indicators

RSCs have adopted (HELCOM, OSPAR) or are in the process of adopting (UNEP/MAP, BSC) key indicators for eutrophication covering objectives set by their strategies. Table 2.1 presents the objectives of the four RSCs in parallel to the corresponding criteria of MSFD for the GES Descriptor 5 Eutrophication. RSCs indicators for eutrophication as well as relevant MSFD and EEA indicators are shown in Table 2.2.

Commission Decision 2010/477/EU requires assessments of eutrophication in marine waters to combine information on nutrient levels and those direct effects and indirect effects that are closely linked to nutrient enrichment. The Decision sets out eight indicators to describe those three criteria. Nutrient inputs are not included in the list of indicators for D5 in the COM Decision 210/477/EU as such but article 8 on initial assessments and annex III of the MSFD specifies the importance of locating the sources of the nutrient inputs determining the level of eutrophication.

The HELCOM eutrophication core indicators have been selected on the basis of the HELCOM ecological objectives and the MSFD criteria for eutrophication (HELCOM, 2013a). The five eutrophication core indicators cover four of the five HELCOM ecological objectives and all three MSFD criteria of the Qualitative Descriptor 5. The recent thematic assessment of eutrophication in the Baltic Sea also utilised two biodiversity core indicators, namely the multimetric faunal indices and the macrophyte depth distribution, which in the Baltic Sea have a strong response to eutrophication. These indicators cover the eutrophication status; HELCOM assesses the inputs of nutrients (water- and airborne nutrient loads, point sources) trough a separate parallel process, pollution load compilation, PLC (HELCOM 2011). Inputs of nutrients have been included in the HELCOM first thematic assessment of eutrophication (HELCOM 2009). Pressure core indicator addressing nutrient inputs to the Baltic Sea are currently under development.

OSPAR has adopted a first set of common indicators including those that are contributing to Eutrophication indicators “Nutrients Levels”, “Direct Enrichment Effects” and “Indirect Enrichment Effects” based on the Common Procedure for the Identification of the Eutrophication Status of the OSPAR Maritime Area. The assessment is based on a two-step analysis starting with a “screening procedure”. All areas not being identified as non-problem areas with regard to eutrophication through the Screening Procedure are subject to the Comprehensive Procedure which comprises a checklist of qualitative parameters for a holistic assessment, integrating further supporting parameters into the total assessment. There is a large overlap between the data needs for the OSPAR assessment, and the requirements for the MSFD and therefore a great potential for streamlining of data and information flows. Potentially additional parameters that can be used in the comprehensive procedure which are not included in the EEA and MSFD eutrophication assessments include for example transboundary transport of nutrients and changes in ecosystem structure.

UNEP/MAP is in the process of adopting common indicators for eutrophication that is addressed by Ecological Objective 5 of the Ecosystem Approach (UNEP/MAP, 2012a). The common indicators have been proposed by the Secretariat taking into account practices of other RSCs and on the basis of experience already gained by the Contracting Parties through their regular MED POL (Programme for the Assessment and Control of Marine Pollution in the Mediterranean Region) monitoring activities as well as the experience gained by EU Mediterranean countries through their implementation of EU Directives such as the Marine Strategy Framework Directive and the Water Framework Directive. In the latest Integrated Correspondence Groups of GES and Targets meeting (UNEP/MAP, 2014a) two operational objectives and two indicators were proposed by UNEP/MAP Secretariat that cover two of the MSFD criteria (5.1. nutrient levels and 5.2 direct effects).

Under the Bucharest Convention, eutrophication is addressed in the Strategic Action Plan for the Rehabilitation and Protection of the Black Sea (BSSAP) 2009 by the Ecosystem Quality Objective3 (EcoQO3): Reduce eutrophication. BSC in the implementation of BSIMAP (Black Sea Integrated Monitoring and Assessment Program) used four indicators for eutrophication that cover the three MSFD criteria (BSC, 2010). An indicator on input of nutrients from point sources was also applied.  Additional indicators relevant to MSFD indicators 5.2.3, 5.2.4 and 5.2.1 were considered and adopted by the CBD AG (Advisory Group on Conservation of Biodiversity) in 2013 but further work is needed in this direction (personal communication with BSC Secretariat).

All RSCs use indicators similar to the two EEA eutrophication indicators CSI021 and CSI023. These indicators address the key policy questions 1) Are nutrient concentrations in our surface waters decreasing? and 2) Is eutrophication in European surface waters decreasing?

Indicator CSIO21- Nutrients in transitional, coastal and marine waters shows annual winter concentrations, classification of concentration levels (i.e. low, moderate, high) and trends in winter nutrient concentrations in the regional seas of Europe.

Indicator CSIO23- Chlorophyll in transitional, coastal and marine waters shows  annual mean summer surface concentrations,  classification of concentration levels (i.e. low, moderate, high) and trends in mean summer surface concentrations of chlorophyll-a in the regional seas of Europe.

The used regional and sub regional seas of Europe are in line with the geographical regions and sub-regions specified in the MSFD.

.

Table 2.1	Eutrophication objectives of the four RSCs and policy questions of the EEA in parallel to the corresponding criteria of MSFD for the GES Descriptor D5

| ΜSFD | OSPAR | HELCOM | UNEP/MAP | BSC | EEA |
| --- | --- | --- | --- | --- | --- |
| D5: Human‐induced eutrophication is minimised, especially adverse effects thereof, such as losses in biodiversity, ecosystem degradation, harmful algae blooms and oxygen deficiency in bottom waters. | Strategic objective: To combat eutrophication in the OSPAR maritime area, with the ultimate aim to achieve and maintain a healthy marine environment where anthropogenic eutrophication does not occur. | Strategic goal: A Baltic Sea unaffected by eutrophication | EQ5: Human-induced eutrophication is prevented, especially adverse effects thereof, such as losses in biodiversity, ecosystem degradation, harmful algal blooms, and oxygen deficiency in bottom waters | BSSAP 2009-EcoQO 3: Reduce eutrophication 5 |  |
| Criteria1 | Ecological Quality Objectives (EcoQO)2 | Ecological objectives3 | Proposed operational objectives 4 |  | Key policy questions6 |
| 5.1 Nutrients level | Winter DIN and/or DIP should remain below elevated levels, defined as concentration > 50% above salinity related and/or region-specific natural background concentrations; | Concentrations of nutrients close to natural levels | 5.1 Human introduction of nutrients in the marine environment is not conducive to eutrophication |  | Are nutrient concentrations in our surface waters decreasing? |
| 5.2 Direct effects of nutrient enrichment | Maximum and mean region-specific chlorophyll a concentrations during the growing season should remain below region-specific elevated levels, defined as concentrations > 50% above the spatial (offshore) and/or historical background concentration; |  | 5.2 Direct effects of nutrient over-enrichment are prevented |  | Is eutrophication in European surface waters decreasing? |
|  | Region/area-specific phytoplankton eutrophication indicator species should remain below respective nuisance and/or toxic elevated levels (and increased bloom duration); | Natural level of algal blooms |  |  |  |
|  |  | Clear water |  |  |  |
| 5.3 Indirect effects of nutrient enrichment | Oxygen concentration, decreased as an indirect effect of nutrient enrichment, should remain above regionspecific oxygen deficiency levels, ranging from 4-6 mg oxygen per litre; | Natural oxygen levels |  |  |  |
|  | There should be no kills in benthic animal species as a result of oxygen deficiency and/or nuisance/toxic phytoplankton indicator species for eutrophication. | Natural distribution and occurrence of plants and animals |  |  |  |

1Commission Decision 2010/477/EU

2OSPAR, 2009a

3HELCOM, 2007

4UNEP/MAP, 2014a

5BSSAP, 2009

http://www.eea.europa.eu/data-and-maps/indicators/

Table 2.2	Eutrophication indicators set by the MSFD, the RCSs and EEA.

| ΜSFD | OSPAR | HELCOM |  | UNEP/MAP | BSC | EEA |
| --- | --- | --- | --- | --- | --- | --- |
| Indicators1 | Common indicarors2 | Core indicators3 | Pre-core indicators3 | Proposed common indicators4 | Indicators in Diagnostic Report, 2010 5 | Indicators6 |
|  | Waterborne nutrient inputs |  | Core indicators of nutrient inputs under development |  | Inputs of nutrients from direct (point) sources |  |
|  | Atmospheric nutrient inputs |  |  |  |  |  |
| 5.1.1 Nutrients concentration in the water column | Winter nutrient concentrations | Concentration of dissolved inorganic nitrogen |  | Concentration of key nutrients in the water column | NO3+NO2 | CSI021-Nutrients in transitional, coastal and marine waters |
|  |  | Concentration of dissolved inorganic phosphorus |  |  | PO4 |  |
| 5.1.2 Nutrient ratios (silica, nitrogen and phosphorus), where appropriate |  |  |  |  |  |  |
| 5.2.1 Chlorophyll concentration in the water column | Chlorophyll concentrations | Concentration of chlorophyll a |  | Chlorophyll-a concentration in the water column | Chlorophyll-a | CSI023 -Chlorophyll in transitional, coastal and marine waters |
| 5.2.2 Water transparency related to increase in suspended algae, where relevant |  | Water transparency (Secchi depth) |  |  |  |  |
| 5.2.3 Abundance of opportunistic macroalgae |  |  |  |  | Considered and adopted by the CBD AG (Advisory Group on Conservation of Biodiversity) in 20137 |  |
| 5.2.4 Species shift in floristic composition such as diatom to flagellate ratio, benthic to pelagic shifts, as well as bloom events of nuisance/toxic algal blooms (e.g. cyanobacteria) caused by human activities | Species shift/indicator species: Nuisance species Phaeocystis |  |  |  | Considered and adopted by the CBD AG (Advisory Group on Conservation of Biodiversity) in 20137 |  |
| 5.3.1 Abundance of perennial seaweeds and seagrasses (e.g. fucoids, eelgrass and Neptune grass) adversely impacted by decrease in water transparency |  |  | Lower depth distribution limit of macrophytes8 |  | Considered and adopted by the CBD AG (Advisory Group on Conservation of Biodiversity) in 20137 |  |
|  |  |  | State of soft-bottom macrozoobenthos8 |  |  |  |
| 5.3.2 Dissolved oxygen, i.e. changes due to increased organic matter decomposition and size of the area concerned | Oxygen | Oxygen concentration |  |  | Hypoxic situations, expansion of zones of hypoxia |  |

1 Commission Decision2010/477/EU

2 ANNEX 6 (Ref. § 4.18) OSPAR Convention for the Protection of the Marine Environment of the North-East Atlantic Meeting of the Coordination Group (CoG) London (Secretariat): 21-22 November 2013

3HELCOM ,2013a

4 UNEP/MAP, 2014b

5BSC, 2010

6 http://www.eea.europa.eu/data-and-maps/indicators/

7Relevant indicators are proposed but further work is needed. Personal communication to BSC Secretariat

8HELCOM Biodiversity core indicator under development in HELCOM CORESET II project

## RSC methodologies for eutrophication indicators and assessment tools

HELCOM

The HELCOM Eutrophication Assessment Tool (HEAT)

The multi-metric indicator-based HELCOM Eutrophication Assessment Tool (HEAT) is used to assess the eutrophication status of the Baltic Sea (HELCOM 2009, HELCOM, 2014). HEAT builds on the OSPAR Common Procedure developed for assessment and identification of ‘eutrophication problem areas’ in the OSPAR convention (see next section on OSPAR). HEAT is based on existing indicators that are combined into groups. HEAT allows weighting between indicators within groups depending on the ecological significance of an indicator for a specific site and the confidence level of the indicator. Hence, indicators thought to be very good can be given a higher weight than an indicator with a low quality and vice versa. For each individual indicator, an interim classification is made. The classification system has five classes: high and good correspond to ‘areas not affected by eutrophication’ and moderate, poor and bad, which correspond to ‘areas affected by eutrophication’. Indicators are combined within groups and ultimately combined into an assessment of ‘overall eutrophication status’. The final step makes use of the ‘One out – All out’ principles sensu the Water Framework Directive which means that the overall classification of an assessed area is based on the most sensitive quality element. In addition, HEAT produces a provisional ‘accuracy assessment’ of the final classification results in order to assess the reliability of the final classification. It arrives at a primary classification of ‘areas affected by eutrophication’ and a secondary assessment of the confidence of the primary assessment.

The first version of the HEAT tool (HEAT 1.0) was based on the indicators dissolved inorganic nitrogen (DIN), dissolved inorganic phosphorus (DIP), chlorophyll-a, Secchi depth, and benthic invertebrate fauna, grouped under the quality elements 1)physical-chemical features, 2)phytoplankton, and 3)benthic invertebrate communities (HELCOM 2009). The assessment principle of HEAT 1.0 was based on reference values (RefCon) and acceptable deviation (AcDev) which, when combined, produced preliminary targets. The boundary between good and moderate was defined through the RefCon and AcDev (Andersen et al., 2011). In most Baltic sub-regions, the acceptable deviation was set arbitrarily at 50 % (25 % for Secchi depth). A detailed description of the HEAT 1.0 assessment tool methodology can be found in Andersen et al., (2011). HEAT 1.0 was applied in the HELCOM thematic assessment of eutrophication in the Baltic Sea (HELCOM 2009), in the HELCOM Initial Holistic Assessment (HELCOM 2010a) and in the demonstration set of core eutrophication indicators (HELCOM 2010b).

In the latest assessment of the eutrophication status of the open sea areas of the Baltic Sea version HEAT 3.0 was applied (HELCOM, 2014). The assessment of the open sea sub-basins was based on an integration of status data from core set indicators on inorganic nitrogen (DIN), inorganic phosphorus (DIP) , chlorophyll a, water transparency (Secchi depth) and oxygen conditions (oxygen debt, for six sub-basins). In HEAT 3.0 the indicators were grouped in a new way into three “Criteria” as described in the Commission Decision (EC 2010): 1) Nutrient levels, 2) Direct Effects and 3) Indirect Effects. At the time of applying HEAT 3.0, HELCOM had strengthened the scientific background of target setting (HELCOM 2013b) and agreed on a set of targets. The new HEAT Tool 3.0 compares an agreed eutrophication target for the selected indicator with the current status value derived from monitoring data. For each of these indicators an Eutrophication Ratio (ER) is calculated. If the eutrophication ratio is below 1.00, it reflects good environmental status (GES) and if it is at or above 1.00, it reflects indicator status where GES has not been reached (sub-GES). HEAT 3.0 integration is applied for each open sea basin assessment unit.

For each criterion the status was determined as the weighted average of the ERs of the individual indicators (e.g. ER for C1, nutrient levels, is given by average of DIP-ER and DIN-ER). The status for the criterion was then assessed as GES where ER≤1 and sub-GES where ER>1.In the final step, the one-out-all-out principle was used between the criteria status classifications to determine the overall eutrophication status for each basin; i.e. the worst of the three results on criterion level determined the final status classification.

Spatial coverage and assessment units

The Baltic Sea is divided into several sub-basins separated by sills, including a transition zone to the North Sea consisting of the Kattegat and the Belt Sea. A number of ‘assessment units’ are defined according to these sub-basins: 172 units in coastal waters marked with open circles and 17 open basins shown with numbered circles (Figure 2. 1). The previous Thematic Eutrophication assessment (HELCOM 2009) used 189 assessment units consisting of 17 open sea sub-basins and 172 coastal assessments. The Updated Baltic Sea open sea area Assessment (HELCOM 2014) was based on 17 open sea sub-basins and status of coastal waters is based on WFD classification status, where such information had been made available by HELCOM Contracting Parties that are also EU Member States.

Figure 2.1	Map of the Baltic Sea with location of ‘assessment units’ in coastal waters (172 units marked with open circles) and open basins (17 units shown with numbered circles). Source: Andersen et al. (2011).

Temporal coverage of the assessments

The HELCOM eutrophication assessments used data from the following time periods

2001-2006: Thematic Eutrophication assessment 2009 (HELCOM, 2009) and Initial Holistic Assessment (HOLAS) 2010

2003-2007: Update of the 2010 assessment for the time period 2003-2007 (published on HELCOM website)

2007-2011: Updated Baltic Sea open sea area assessment (HELCOM, 2014)

Indicators and targets for open sea sub-basins

The following indicators grouped into three criteria were used in the latest eutrophication assessment of the open sea sub-basins:

- Nutrient levels: winter (December-February) dissolved inorganic nitrogen (DIN) and dissolved inorganic phosphorus (DIP) concentrations in the surface layer (0 - 10 m depth).

- Direct effects: summer (June-September) chlorophyll a concentration in the surface layer (0 - 10 m depth) and summer (June-September) Secchi depth.

- Indirect effects: annual oxygen debt below the halocline (for the Bornholm Basin, Western Gotland Basin, Eastern Gotland Basin, Northern Baltic Proper and Gulf of Finland).

The indicators within the criteria were weighted according to their relevance for eutrophication in each basin.

The indicator targets were based on the results obtained from the HELCOM TARGREV project (HELCOM 2013b), taking also advantage of the work carried out during the HELCOM EUTRO PRO process (HELCOM 2009) and national work for WFD. The final targets were set through an expert evaluation process (HELCOM CORE EUTRO) and the targets were adopted by the HELCOM Heads of Delegations 39/2012 (see below Table from HELCOM 2014). Development work for new eutrophication indicators is expected to continue in the coming years and list of candidate indicators suitable for development work identified.

SOURCE: HELCOM 2014

Data aggregation in the open sea area Assessment

Data sources: Primary data source for the assessment was HELCOM data host ICES COMBINE (Cooperative Monitoring in the Baltic Marine Environment) data base. Another important data source was Baltic Nest Institute through their BED distributed database system that accesses Baltic Sea area institutional and national data bases containing both restricted access and open data sources. These two major sources were complemented by data received from national experts in Denmark, Finland, Germany, Poland and Sweden.

The aggregated average 2007-2011 indicator values were estimated as

an inter-annual winter (December-February) average for inorganic nutrients,

an inter-annual summer (June-September) average for chlorophyll a and Secchi depth

an inter-annual average for oxygen debt

For the DIN, DIP and oxygen debt indicators, data were aggregated using a combined spatial and seasonal model (as applied in the TARGREV project, HELCOM 2013b) to compensate for uneven distribution of stations and inadequate seasonal coverage of measurements in order to overcome shortcomings of the monitoring station networks of the Contracting Parties (HELCOM 2013b).

The confidence of indicator status (EUT_S-score) was rated for each indicator within an assessment unit, according to the availability and distribution of data during the assessment period.

Figure 2.2	shows the steps of assessing eutrophication status of the open sea sub-basins of the Baltic Sea from monitoring data to core indicator –based integrated assessment results for each sub-basin (HELCOM, 2014).

SOURCE: HELCOM, 2014

Figure 2.2 Steps of assessing eutrophication status of the open sea sub-basins of the Baltic Sea from monitoring data to core indicator –based integrated assessment results for each sub-basin

OSPAR

The Common Procedure

The definition of marine eutrophication given by the Eutrophication Strategy in a generalised and qualitative way is implemented and made operational through the Common Procedure for the Identification of the Eutrophication Status of the OSPAR maritime area (OSPAR, 2005; Ref no: 2005-3). The Common Procedure is based on a holistic scheme of qualitative assessment criteria which combines aspects of nutrient enrichment with aspects of direct, indirect and other possible effects of excessive nutrient enrichment on water quality and ecosystem components. The Common Procedure is supported under the eutrophication related part of the OSPAR Joint Assessment and Monitoring Programme (JAMP) by collective OSPAR monitoring. The Eutrophication Monitoring Programme (OSPAR, 2005) is supplemented by monitoring guidelines, as part of the OSPAR Co-ordinated Environmental Monitoring Programme (CEMP). Under the JAMP, monitoring and periodic assessments of temporal trends of waterborne and atmospheric inputs of nutrients to the OSPAR maritime area under the OSPAR Comprehensive Study of Riverine Inputs and Direct Discharges (RID) and the OSPAR Comprehensive Atmospheric Monitoring Programme (CAMP) also inform the assessment of the eutrophication status.

The Common Procedure comprises two procedural phases:

•	The first phase, the one-off “Screening Procedure”, was completed in 2001 and identified those areas of the OSPAR maritime area which are likely to be areas where eutrophication is not a problem. Those areas were classified as “non-problem areas” without further detailed assessment. The screening resulted in the areas shown in Fig. 2.3a which could not be set aside as obvious non-problem areas.

•	In the second phase, the “Comprehensive Procedure” of the Common Procedure, a comprehensive assessment of the eutrophication status of the areas in Fig 2.3b was carried out. The Comprehensive Procedure is a reiterative process which was first applied by Contracting Parties in 2002 (OSPAR, 2003). In the second application of the Comprehensive Procedure in 2007(OSPAR 2008) the areas assessed included those that had been identified as problem areas or potential problem areas in the first application with addition of over 59 areas had been which had not been previously assessed under the Comprehensive Procedure (Fig 2.3b).

Figure 2.3	Areas assesed in the first (a) and second application (b) of the Comprehensive Procedure in 2002 and 2007 respectively. Assesement parametres used by Contracting Parties in 2007 are also shown. Source: OSPAR, 2008.

Ten assessment parameters (indicators), grouped in 4 categories have been selected from a list of assessment criteria, for the harmonised application of the eutrophication assessment by Contracting Parties as shown in the following Table.

OSPAR harmonized assessment parameters (Source: OSPAR, 2008)

In previous OSPAR common procedure the Contracting Parties provided the national assesments that are summarized in the integrated report.

The assessment process used by Contracting Parties has generally followed the guidance of the Common Procedure which entails:

- the assignment of a score corresponding to the level of each assessment parameter which has been monitored;

- an initial assessment based on a combination of these scores according to an agreed

framework, and;

- an overall final assessment of all relevant information relating to harmonised assessment parameters, their corresponding assessment levels and supporting environmental factors.

For each parameter, area-specific assessment levels (thresholds) are derived in relation to the relevant background conditions. The assessment level may deviate from background conditions to reflect natural variability. For concentrations, the assessment level is generally defined as a justified area-specific % deviation from background conditions not exceeding 50%.An overview of deriving assesement levels (thresholds) of the assesement parameters (indicators) under the Common Procedure is shown in the following Table from OSPAR (2012).

Source: OSPAR, 2012

For an initial classification of an area, the observed levels for each assessment parameter are scored and evaluated in relation to each other. The scores for each of the parameters are reported in a common format, the departing point for the second step in the classification process. The initial classification is as follows:

  - areas showing an increased degree of nutrient enrichment accompanied by direct and/or indirect/ other possible effects are regarded as ‘problem areas’;

  - areas may show direct effects and/or indirect or other possible effects, when there is no evident increased nutrient enrichment, for example, as a result of transboundary transport of (toxic) algae and/or organic matter arising from adjacent/remote areas. These areas could be classified as ‘problem areas’;

  - areas with an increased degree of nutrient enrichment where:

    - either there is firm, scientifically based evidence of the absence of (direct, indirect, or other possible) eutrophication effects – these are classified initially as ‘non-problem areas’, although the increased degree of nutrient enrichment in these areas may contribute to eutrophication problems elsewhere;

    - or there is not enough data to perform an assessment or where the data available is not fit for the purpose – these are classified initially as ‘potential problem areas’;

  - areas without nutrient enrichment and related (in)direct/ other possible effects are considered to be ‘non-problem areas’.

Spatial coverage and assessment areas in the second application of the Comprehensive Procedure

The areas assesed in the second application of the Comprehensive Procedure in 2007 (OSPAR 2008) are shown in Fig 2.3b. Contracting Parties have used different geographical scales for identifying individual assessment areas ranging from small individual fjords to large coastal strips.The size of the assessment areas is decreasing from offshore to inshore waters (estuaries, bights, fjords).

Temporal coverage of the assesements

The first application of the Comprehensive Procedure roughly covered the years 1990-2000 and the second application assesed the period 2001 – 2005.

Assesement parameters (indicators) and assesement levels (thresholds) in the second application of the Comprehensive Procedure

The agreed harmonised assessment parameters have not been applied by all Contracting Parties (see Fig 2.3b). Inorganic winter nutrients, chlorophyll and oxygen concentrations are the main parameters that have been considered in estuaries, including fjords, and in coastal waters. Offshore, mainly winter nutrients and chlorophyll have been used in the assessment. Overall, chlorophyll is the most applied effect parameter, followed by oxygen. The more complicated analyses of phytoplankton indicator species, macrophytes and zoobenthos were less often performed. The following parameters used in the second application of the Comprehensive Procedure are included in OSPAR’s common Indicators list .

Winter nutrient concentrations. In areas affected by a salinity gradient, both winter nutrient (DIN, DIP) concentrations and corresponding salinities are reported, in order to normalize nutrient concentrations for salinity. In the North Sea, the assessment levels used by Contracting Parties for winter nutrient concentrations are set in relation to salinity gradients.

Assessment levels for winter DIN and winter DIP Source: OSPAR, 2008

Chlorophyll a. A number of countries used the 90 percentile instead of, or in addition to mean and maximum concentrations of chlorophyll. Maximum concentrations proved to be difficult to work with because of the high frequency of measurements needed to detect the maxima. Moroever, using 90 percentiles enhances compliance with the Water Framework Directive (2000/60/EC).

Assessment levels for chlorophyll a. Source: OSPAR, 2008

Phytoplankton indicator species. A number of Contracting Parties measured nuisance phytoplankton species like the foam-forming species Phaeocystis or the dense surface algal blooms of Noctiluca as eutrophication indicators. To assess the duration of bloom of nuisance phytoplankton indicator species, the next step would be to assess the % number of months in the year for which blooms are above assessment levels of 106 cells per litre, taking into account the corresponding developments under the Water Framework Directive

(2000/60/EC).

Assesement levels for area-specific phytoplankton indicator species.Source: OSPAR, 2008

Oxygen. The current assessment level for oxygen deficiency defined by OSPAR is 4-6 mg/l and marks the “threshold (range)” between problem and non-problem area. A further

harmonisation of the assessment of oxygen should include the duration and spatial extent (area and depth) of oxygen deficiency.

Assessment levels for oxygen. Source: OSPAR, 2008

UNEP/MAP

The Contracting parties to the Barcelona Convention in the process of implementing the Ecosystem Approach have adopted Mediterranean Ecological Objectives (EOs) including EO5 Eutrophication, associated with Operational Objectives and indicators (UNEP/MAP, 2012a). Following RSCs practices, the establishment of common indicators is the next step to put forward. For EO 5, two common indicators are currently proposed, concentration of key nutrients and chlorophyll-a concentration (Table 2.3, UNEP/MAP, 2014a).The process of identifying targets and corresponding GES for EO5 has initiated in 2012 (UNEP/MAP 2012b). Thresholds values are not yet available but MED POL has made some preparatory work to provide initial background information on methodologies for the establishment of threshold values for eutrophication (UNEP/MAP, 2011a). This work would need to be further discussed during national expert meetings organized by MAP MED POL during 2014-2015.

Table 2.3	Operational objectives, common indicators, GES and targets for EO5 eutrophication (adapted from UNEP/MAP, 2014a)

| Common Indicator | GES | Targets |
| --- | --- | --- |
| 5.1.1 Concentration of key nutrients in the water column geographic and climate conditions | Concentrations of nutrients in the euphotic layer are in line with prevailing physiographic characteristics of the un-impacted marine region 1 | State Reference nutrients concentrations according to the local hydrological, chemical and morphological Decreasing trend of nutrients concentrations in water column of human impacted areas, statistically defined Pressure 1. Reduction of BOD emissions from land based sources 2. Reduction of nutrients emissions from land based sources |
| 5.2.1 Chlorophyll-a concentration in the water column | Natural levels of algal biomass in line with prevailing physiographic, geographic and weather conditions2 | State 1. Chl-a concentrations in high-risk areas below thresholds3 2. Decreasing trend in chl-a concentrations in high risk areas affected by human activities |

1 Thresholds to be set, subject to decision of Contracting Parties by COP19.

2 Thresholds to be determined by COP19.

3Thresholds to be set in the future, feasibility to be addressed, subject to decision of Contracting Parties by COP19

Common Indicator 7: Concentration of key nutrients in the water column. Data for this indicator have been generated by a number of Contracting Parties for the purpose of the MED POL Phase IV eutrophication monitoring programme. The parameters monitored include NO3, NO2, NH4, PO4 (or Total N, Total P) and SiO4 (occasionally). Additional data generation for MED POL in terms of additional parameters to address this indicator would not be required. However the existing geographical coverage in terms of contribution of monitoring data by Contracting Parties for this indicator would need to be improved.

Common Indicator 8: Chlorophyll-a concentration in the water column. Data for this indicator have been generated by a number of Contracting Parties for the purpose of the MED POL Phase IV eutrophication monitoring programme. Methodologies considered include chlorophyll-a concentration measured in winter and in early spring (November–March) when phytoplankton bloom occurs following the deep winter mixing (<0.1 μg/l), data from a 25 meters top layer and /or surface data and satellite imagery to identify hotspots (different productivity areas). The existing geographical coverage in terms of contribution of monitoring data by Contracting Parties for this indicator would need to be improved. For a wider sub-regional and regional scale, the possibility to assess the actual condition for chl-a concentrations using satellite images is considered. These values could then be used as reference conditions for any subsequent GES monitoring based on trends.

NO3, NO2, NH4, PO4 and chlorophyll a are mandatory parameters set by the Eutrophication Marine Strategy of MED POL and guidance on relevant sampling and analysis techniques to support the Eutrophication Monitoring Strategy of MED POL are available (UNEP/MAP, 2005). As regards quality assurance, an intercalibration exercise for eutrophication parameters was completed in 2010 using the services of QUASIMEME. MED POL supported the participation of 19 Mediterranean laboratories in the exercise for the determination of nutrients and chlorophyll-a in seawater (UNEP/MAP, 2011b).

No standard methodologies and procedures have been developed for the regional assessment of eutrophication in the Mediterranean Sea. However, in a number of countries national eutrophication assessment methods are performed. Among the indices used for eutrophication assessment, the Trophic Index (TRIX) (Vollenweider, 1998) is the most widely used in Mediterranean waters over the last 10 years.

The TRIX index uses the following parameters:

ChA = Chlorophyll a concentration as µg/L

aD%O = Oxygen as absolute % deviation from saturation

DIN = Dissolved Inorganic Nitrogen, N-(NO3+NO2+NH4) as µg/L

TP = Total Phosphorus as µg/L

A description of the TRIX Index can be found in UNEP/MAP (2011a).

Black Sea Commission

The BSIMAP (Black Sea Integrated Monitoring and Assessment Program) is based on National monitoring programs financed by the Black Sea countries. Each country decides which and how many stations will use, the frequency of sampling and which stations will be used for the reporting for the BSIMAP. The BSIMAP provides recommendations on parameters, frequency and methodologies to be used. Manuals on sampling and analysis have been developed for phytoplankton (Moncheva& Parr, 2005Moncheva, 2010,).

The BSIMAP follows the DPSIR (Drivers, Pressures, State, Impact, Response) model allowing detection of negative impacts as well as the effects of measures taken in a timely manner, thereby enabling the necessary corrective actions to be further taken. The choice of parameters to monitor is related to the main environmental problems recognized in the Black Sea region and re-evaluated every 10 years based on BSC reports: Black Sea Transboundary Diagnostic Analysis (BS TDA) and State of the Environment of the Black Sea (SoE) (e.g. BSC SoE Report 2002 and 2008, www.blacksea-commission.org). Deficiencies in the provisions for indicator-based reporting in line with the DPSIRR model, hence, with the MSFD requirements, are presented in detail for Bulgaria, Romania and Turkey in the Diagnostic report II prepared in the framework of the MISIS project (Velikova et al, 2013).

The BSIMAP parameters are optional or compulsory, depending on the priorities. Nutrients and phytoplankton are compulsory parameters. Nutrients are regularly reported. Chlorophyll is occasionally reported and not by all countries. Data on expansion of hypoxic zone indicator is limited. Loads (Inputs of nutrients from direct (point) sources) are well reported and there is sufficient data on rivers and hot spots.

The Trophic Index (TRIX index) is proposed for assessment of eutrophication status but its implementation at regional scale is not currently possible as chlorophyll data are not provided by all countries.

The following overview provided in BSC (2010) shows the status of nutrient and chlorophyll monitoring in the Black Sea countries as well as the improvements needed.

Eutrophication related assessment methodologies in the WFD

One of the parameters reflecting the direct effects of nutrient enrichment is Phytoplankton which is also one of the three biological quality elements (BQE) of the WFD. According to Annex V of WFD (EC, 2003) the parameters defining the quality status of this element in coastal and transitional waters are:

- The composition and abundance of the phytoplanktonic taxa

- The average phytoplankton biomass

- Planktonic blooms

Among these, the phytoplancton biomass expressed as Chlorophyll a concentrations is the most region-wide intercallibrated parameter for the BQE phytoplankton under the  Geographical Intercalibration Groups of the four MSFD regions.

Nutrient status is included in the physicochemical quality elements which together with the hydro morphological and primarily the biological quality elements determine the ecological status.

EEA eutrophication indicators methodologies

Indicator CSIO21-Nutrients in transitional, coastal and marine waters. The indicator shows 1) annual winter concentrations; 2) classification of concentration levels (i.e. low, moderate, high) and 3) trends in winter oxidized nitrogen (nitrate + nitrite) and phosphate concentration in the regional seas of Europe. Levels and trends of winter concentrations of dissolved inorganic nutrients are used for this indicator, as it is assumed that winter concentrations are not significantly reduced due to uptake by primary producers. The winter period is January, February and March for stations east of longitude 15 degrees (Bornholm) in the Baltic Sea and January and February for all other stations. Nutrient concentrations are expressed in micromol/l. Annual concentrations for Nitrogen (i.e. Nitrates, Nitrites and Total Oxidised Nitrogen) and Phosphate (i.e. Orthophosphates) are calculated per station. For each (sub)regional sea, the observed concentrations are classified as Low, Moderate or High. Concentrations are classified as Low when they are lower than the 20-percentile value of concentrations within a (sub)region and as High when they are higher than the 80-percentile value of concentrations within a (sub)region. For trend analysis of nitrogen and phosphate concentrations, stations must have at least data in the last four years of the current assessment (2007 or later), and 5 or more years in the period since 1985. Details on methodology for the calculation of this indicator can be found at

http://www.eea.europa.eu/data-and-maps/indicators/nutrients-in-transitional-coastal-and/nutrients-in-transitional-coastal-and-4

Indicator CSIO23- Chlorophyll in transitional, coastal and marine waters. The indicator shows 1) annual mean summer surface concentrations 2) classification of concentration levels (i.e. low, moderate, high) and 3) trends in mean summer surface concentrations of chlorophyll-a in the regional seas of Europe. The concentration of chlorophyll-a is expressed as microgram /l in the uppermost 10 m of the water column during summer. Summer period is June to September for stations north of latitude 59 degrees in the Baltic Sea (Gulf of Bothnia and Gulf of Finland) and May to September for all other stations. Annual mean summer surface concentrations of Chl-a, are calculated per station. For each (sub)regional sea, the observed concentrations are classified as Low, Moderate or High. Concentrations are classified as Low when they are lower than the 20-percentile value of concentrations within a (sub)region and as High when they are higher than the 80-percentile value of concentrations within a (sub)region. For trend analysis stations must have at least data in the last four years of the current assessment (2007 or later), and 5 or more years in the period since 1985. Details on methodology for the calculation of this indicator can be found at

http://www.eea.europa.eu/data-and-maps/indicators/chlorophyll-in-transitional-coastal-and/chlorophyll-in-transitional-coastal-and-3

The data used for the EEA eutrophication indicators is part of the WISE - State of the Environment (SoE) data, available in Waterbase - TCM (Transitional, Coastal and Marine) waters.

## RSCs data streams analysis for identifying eutrophication indicators

ICES is the responsible data centre for the marine environmental monitoring data of HELCOM and OSPAR. Contracting Parties are obliged to report COMBINE (HELCOM Cooperative Monitoring in the Baltic Marine Environment) and CEMP (Comprehensive Environmental Monitoring Programme) monitoring data to ICES on an annual basis by 15 September every year using the agreed reporting formats. The submission is done by email. Data are organized in relational data base and the metadata are ISO19115 compliant. Both data and their metadata are available on-line through Web Services.

HELCOM

HELCOM pollution load compilation database is hosted and maintained by HELCOM data consultant SYKE with direct email submissions on riverborne, coastal and point source loads by 1th of November every year and more detailed reporting including sources on land approximately every sixth year.

Information on the parameters relevant to eutrophication indicators that the HELCOM data streams include can be found below:

- COMBINE database holding CP data on Temperature, Salinity, Oxygen, Phosphate, Total Phosphorus, Silicate, Nitrate, Nitrite, Ammonium, Total Nitrogen, Hydrogen Sulphide, pH, Alkalinity, Chlorophyll a, Secchi depth

- HELCOM Pollution Load Compilation 5.5 (PLC 5.5) dataset containing all waterborne nutrient and hazardous substances loads gathered by HELCOM contracting parties within pollution load monitoring1

- HELCOM Pollution Load Compilation 6 (PLC 6) ongoing comprehensive assessment of water- and airborne inputs and their sources to the Baltic Sea during the period 1994-2014 with more detailed assessment for 20141

- Ship-of opportunity (Ferry-box) DIN, DIP & chl-a2

- Ship-of opportunity (Ferry-box) in situ fluorescence-based chl-a (validated) 2

- Earth observation-based chl-a (validated)2

1CP submissions to HELCOM data consultant (SYKE) by email. Currently there is no online access point. Need for developing SQL database and interface to modernize reporting, QA and maximize the use of and access to data. Development work is ongoing within HELCOM PLUS project.

2 Planned to be included into HELCOM operational eutrophication indicator database (EUTRO-OPER project) hosted by ICES.

OSPAR

Information on the parameters relevant to eutrophication indicators that the OSPAR data streams include can be found below:

- “Comprehensive Atmospheric Monitoring Programme (CAMP)” includes data and model results on wet and dry deposition of nitrogen compounds (oxidized and reduced).

- “Comprehensive Study of Riverine Inputs and direct Discharges (RID)” includes data compilations of yearly loads from nitrogen (NH4, NO3, TotN) and phosphorus (PO4, TP)  compounds, based on measurements (large rivers) and estimates (smaller tributaries).

- “Models used for quantification and reporting of nutrient discharges and losses”. This datastream is currently under development. It is anticipated that model results will provide information on source appointments and trans-boundary nutrient transport of nutrients, that can be used in eutrophication assessments and plans of measures.

- “Comprehensive Environmental Monitoring Programme (CEMP)” contains data on NH4, NO3, NO2, PO4, SiO4, TotN, TotP, Dissolved Oxygen and Chlorophyll-a, as well as a limited amount of data on phytoplankton species counts.

UNEP/MAP

MED POL monitoring data reported by the Contracting Parties to UNEP/MAP-Barcelona Convention are held in the MED POL database (http://195.97.36.231/medpol/). The data are reported by the end of the year by email. However not all Contracting Parties report data on a regular basis and there are gaps both in terms of temporal and spatial coverage. The data are held in excel format. Metadata are standardized using internal conventions and rules. The MED POL Info System is a networked information system intended to provide the Contracting Parties and MED POL Unit with the tools to manage, share, preserve and analyse MED POL data to MED POL users. The Info System is not yet operational but it will soon be available.

Information on the parameters relevant to eutrophication indicators that the UNEP/MAP data streams include can be found below:

- MED POL data base holding data on NO3, NO2, NH4, PO4 (or Total N, Total P) and SiO4 (occasionally) and general oceanographic parameters (temperature, salinity, dissolved oxygen)

- Land-based pollution sources database holding national data on pollutants industrial and municipal discharges collected by national surveys

BSC

The Black Sea Integrated Monitoring and Assessment Programme (BSIMAP) provides common data/information reporting formats and the contracting parties have the obligation to report to the BSC on an annual basis using these agreed formats. Data are annually collected, though with a different frequency per year.

The BSC Regional Database on Pollution is a component of the Black Sea Information System (BSIS) held by the Regional Activity Center for Pollution Monitoring and Assessment (RAC PMA). Nutrients data constitute the major part of the database.The Black Sea Regional Database is available through the web site:http://rdbp.sea.gov.ua. Password is required for external users to access it. The database is built using a RDBMS system. Metadata descriptions and their standards follow the SeaDataNet approach. Detailed description of the BSC database can be found in the report “The development of a new version of the Regional Data base on Pollution of BSIS” prepared by the RAC PMA under the BSC-Baltic2Black Service contract (rdbp.sea.gov.ua/docs/FINAL_REPORT_RDB.doc

Information on the parameters relevant to eutrophication indicators that the BSC data streams include can be found below:

- Regional Data base on Pollution holding data on NO2, NO3, NH4, Total N, PO4, Total P, SiO4, Chl a, Т, Salinity, O2, O2 % , phytoplankton (total biomass)

- Data on input of nutrients from direct sources. The data are stored in the BSC information system as excel files and are not accessible online

EEA

EEA has organized its data flows with the European environment information and observation network (Eionet), a partnership network of the EEA and 33 members and 6 cooperating countries. Eionet consists of the EEA itself, a number of European Topic Centres (ETCs) – ETC/ICM (Inland, Coastal and Marine Waters) being the water thematic center - and a network of National Focal Points (NFPs) and National Reference Centres (NRCs).

In the context of the implementation of the Water Framework Directive (WFD), EEA´s Eionet-Water annual data flow for waters was transferred into the WISE ‘State of the Environment’ (SoE) data flow in 2008 and gained full integration into the reporting under WISE complementary with data reported under the WFD (i.e. information reported already under the WFD obligatory reporting would not be required again, but used as available in WISE).

The WISE SoE TCM (Transitional, Coastal and Marine Waters) dataset results from data collected annually both from EEA member countries and from the RSCs through the WISE-SoE TCM data collection process.

WISE-SoE data collection process is based on the existing monitoring networks in the EEA member countries, where a representative sub-sample of national monitoring sites in rivers, lakes, groundwater and transitional, coastal and marine waters were selected for the European network with no additional demands for new data gathering. WISE-SoE data collection runs annually and traditionally starts at the end of July/beginning of August.

WISE SoE data on transitional, coastal and marine waters (WISE SoE TCM) is reported annually to the EEA by the Eionet countries using the Reportnet tools based on agreed set of specifications, schemas, templates and common dictionaries available to countries for structuring and formatting their deliveries. Countries are responsible for the quality control and assurance of their national datasets. WISE SoE datasets are then further handled by the European Topic Centre on Inland, Coastal and Marine waters (ETC/ICM), where a series of QA and validation routines are performed in order to ensure that the data delivered to the EEA are comparable at the European level.

To manage the duplicate submissions within the WISE SoE TCM data flow, an agreement was reached whereby countries were to provide only data that had not already been submitted to the RSC. Data submitted to the RSC is obtained from ICES directly.

Data and information obtained through the above processes are used to produce indicators upon which EEA assessment reports are based. Collected data are also published in Waterbase, a series of water topic-specific databases and web pages, publicly accessible via the EEA Data Service's web site

The most recent WISE SoE TCM data can be viewed, analysed and downloaded from the Waterbase – TCM, an EEA data service available at the following website : http://www.eea.europa.eu/data-and-maps/data/waterbase-transitional-coastal-and-marine-waters-9

The WISE SoE TCM dataset contains data on physical characteristics of the transitional, coastal and marine water monitoring and flux stations, proxy pressures on the upstream catchment, basin and River Basin District associated with transitional and coastal waters, chemical quality data on nutrients in seawater and hazardous substances in biota, sediment and seawater, as well as data on direct discharges and riverine input loads.

ANALYSIS

It must be noted that data are not regularly reported to the RSCs for all parameters and by all contracting parties and for some parameters monitoring is currently developing. Thus an analysis was performed on data relevant to MSFD eutrophication indicators that are actually reported to the RSCs by the contracting parties. The parameters/methods used by the MS for reporting under articles 8, 9 and 10 of the MSFD, summarized and evaluated by JRC (Palialexis et al. 2014) were the ones considered for the analysis of existing data flows. Data reported to the RSCs in 2012 by the contracting parties that are EU MS was used in this analysis. Data reported to HELCOM and OSPAR was obtained from ICES, UNEP/MAP data was obtained from the MED POL data base and BSC data was provided by the IRIS SES project. Details on data management aspects such as stewardship, quality control, updates of the data sets, sampling frequencies etc. can be found in Annex 1. Data reported through Eionet (WISE SoE) within 2008-2012 was provided by ICES.

SUMMARY

An overview of data relevant to MSFD eutrophication indicators that are actually reported to the RSCs by the contracting parties and at what frequencies across RSCs is shown in the following tables (Tables 2.4-2.9).The frequency of reporting for each RSC (% of CPs that are EU MS reporting to each RSC) for each parameter is categorized in classes represented by different colors. The frequency of use of the reported parameters by MS for MSFD reporting in each Region assessed by JRC (Palialexis et al. 2014) is shown. Relevant EEA and WFD parameters are also shown.

Nutrients, chlorophyl a and oxygen are reported by most contracting parties across RSCs. The percentage of CPs reporting on eutrophication to UNEP/MAP was low and oxygen was not reported.  Nutrients, chlorophyl a and oxygen present high frequency of use by MS reporting for the implementation of Art 8, 9 and 10 of the MSFD and are also used in the WFD.  EEA also uses nutrients and chlorophyl data for the two eutrophication indicators CSI021 and CSI023.

Nutrients, chlorophyl a and oxygen data are reported though Eionet although reporting is not identical with RSC reporting (i.e. number of stations is different or number of parameters is different). Some countries report more data through Eionet but mostly more data is reported through ICES. When comparing OSPAR and HELCOM data flows through Eionet and through ICES in terms of number of countries reporting, more countries report nutrients, chlorophyl a and oxygen data through ICES. It must be noted that data flows through ICES in fact refers to RSC data flows that are recycled as Eionet data flows, so although agreements are made with RSC or the Eionet, it is in fact possible to use the same data flow for multiple purposes.

Information on data flows for nutrient inputs were available per country for HELCOM, OSPAR and BSC. Data reported per country as regards nutrient inputs from rivers and point sources as well as emissions can be found in Annex 1.  All HELCOM CPs reported on nutrient input parameters in 2012 with the exception of one CP as regard inputs from rivers and point sources. OSPAR CPs apart from three reported on water borne nutrient input parameters in 2011, and all CPs apart from one reported on atmospheric nutrient inputs in 2011. BSC data used were reported in the period 2001-2008 (BSC, 2010). The two CPs reported nutrient inputs from municipal, industrial and riverine sources during that period. Nutrient inputs in the UNEP/MAP region are reported in the National Budget (NBB) assessing the current trends of the pollutants emissions and releases (2003-2008) including data from all CPs apart from two (UNEP/MAP, 2012c).

Table 2.4	Parameters relevant to MSFD indicator 5.1.1: Frequency of reporting to the RSCs by CP and frequency of use by MS for MSFD reporting

| Indicator 5.1.1 | PO4 | NO3 | NO2 | NH4 | DIN | DIP | TN | TP | SI | nitrogen composition | nutrient concentration | phosphorous composition | ToxN | DON | POC |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| OSPAR | X | x | X | X | x** | x | X | x | x |  |  |  |  |  |  |
| MSFD art 8,9,10 * | X |  |  |  | x | x | x | x |  | x | x | x | x |  |  |
| HELCOM | X | x | X | X | x** | x | x | x | x |  |  |  |  |  |  |
| MSFD art 8,9,10* | X |  |  |  | x | x | x | x | x | x | x | x |  |  |  |
| UNEP/MAP | X | x | X | X | x ** | x | x | x |  |  |  |  |  |  |  |
| MSFD art 8,9,10* | X | x |  |  | x |  |  | x |  | x | x | x |  | x | x |
| BSC | X | x | X | X | x ** | x | x | x | x |  |  |  |  |  |  |
| MSFD art 8,9,10* | X | x | X | X | x | x |  |  |  |  |  |  |  |  |  |
| WFD* |  | x |  | X |  |  |  |  |  |  | x |  |  |  |  |
| EEA | X | x | X |  |  |  |  |  |  |  |  |  |  |  |  |

* Palialexis et al. 2014

** DIN can be calculated

| Frequency | 100-81% | 80-61% | 60-41% | 40-21% | 20-0% |
| --- | --- | --- | --- | --- | --- |

Table 2.5	Parameters relevant to MSFD indicator 5.2.1: Frequency of reporting to the RSCs by CP and frequency of use by MS for MSFD reporting

| Indicator 5.2.1 | Chlorophyl a | Phytoplankton biomass | Phytoplankton biovolume |
| --- | --- | --- | --- |
| OSPAR | x |  |  |
| MSFD art 8,9, 10 * | x |  | x |
| HELCOM | x |  |  |
| MSFD art 8,9, 10 * | x | x | x |
| UNEP/MAP | x |  |  |
| MSFD art 8,9, 10 * | x |  |  |
| BSC | x |  |  |
| MSFD art 8,9, 10 * | x |  |  |
| WFD* | x | x |  |
| EEA | x |  |  |

* Palialexis et al. 2014

| Frequency | 100-81% | 80-61% | 60-41% | 40-21% | 20-0% |
| --- | --- | --- | --- | --- | --- |

Table 2.6	Parameters relevant to MSFD indicator 5.2.2: Frequency of reporting to the RSCs by CP and frequency of use by MS for MSFD reporting

| Indicator 5.2.2 | Water Transparency | Total suspended solids |
| --- | --- | --- |
| OSPAR |  |  |
| MSFD art 8,9, 10 * | x | x |
| HELCOM | x |  |
| MSFD art 8,9, 10 * | x |  |
| UNEP/MAP |  |  |
| MSFD art 8,9, 10 * | x | x |
| BSC | x |  |
| MSFD art 8,9, 10 * | x |  |
| WFD | x |  |
| EEA |  |  |

* Palialexis et al. 2014

| Frequency | 100-81% | 80-61% | 60-41% | 40-21% | 20-0% |
| --- | --- | --- | --- | --- | --- |

Table 2.7	Parameters relevant to MSFD indicator 5.2.4: Frequency of reporting to the RSCs by CP and frequency of use by MS for MSFD reporting

| Indicator 5.2.4 | Pelagic shift | Phytoplankton blooms | Toxic algae | % of Dinoflagellates | Phytoplankton quality | Phycocyanin | Eutro_ plankt_index | Benthic shift | Cyanobact blooms |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| OSPAR |  |  |  |  |  |  |  |  |  |
| MSFD art 8,9, 10 * | X | X | x |  | x |  | x | x |  |
| HELCOM |  |  |  |  |  |  |  |  |  |
| MSFD art 8,9, 10 * | X |  | x | x |  | x |  | x | x |
| UNEP/MAp |  |  |  |  |  |  |  |  |  |
| MSFD art 8,9, 10 * | X | X | x |  |  |  | x |  |  |
| BSC |  | X |  |  |  |  |  |  |  |
| MSFD art 8,9, 10 * | X | X |  | x |  |  |  |  |  |
| WFD |  | X |  |  |  |  |  |  |  |
| EEA |  |  |  |  |  |  |  |  |  |

* Palialexis et al. 2014

| Frequency | 100-81% | 80-61% | 60-41% | 40-21% | 20-0% |
| --- | --- | --- | --- | --- | --- |

Table 2.8	Parameters relevant to MSFD indicator 5.3.1: Frequency of reporting to the RSCs by CP and frequency of use by MS for MSFD reporting

| Indicator 5.3.1 | Perennial seaweeds | Phytobenthos distribution | Seagrasses distribution | Macroalgae abundance | MM Skew index | Macroalgae condition |
| --- | --- | --- | --- | --- | --- | --- |
| OSPAR |  |  |  |  |  |  |
| MSFD art 8,9, 10 * | X |  | x | x |  |  |
| HELCOM |  |  |  |  |  |  |
| MSFD art 8,9, 10 * | X | x | x |  |  | x |
| UNEP/Map |  |  |  |  |  |  |
| MSFD art 8,9, 10 * | X |  | x | x | x |  |
| BSC |  | x |  |  |  |  |
| MSFD art 8,9, 10 * | X |  |  |  |  |  |
| WFD* |  |  |  | x |  |  |
| EEA |  |  |  |  |  |  |

* Palialexis et al. 2014

| Frequency | 100-81% | 80-61% | 60-41% | 40-21% | 20-0% |
| --- | --- | --- | --- | --- | --- |

Table 2.9	Parameters relevant to MSFD indicator 5.3.2: Frequency of reporting to the RSCs by CP and frequency of use by MS for MSFD reporting

| Indicator 5.3.2 | DO | Organic content | BOD5 | BQI | M-AMBI | Anoxia | COD | Benthic mortality |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| OSPAR | X |  |  |  |  |  |  |  |
| MSFD art 8,9, 10 * | X |  | x | x | x |  |  | x |
| HELCOM | X |  |  |  |  |  |  |  |
| MSFD art 8,9, 10 * | X | x |  | x |  |  |  |  |
| UNEP/MAP |  |  |  |  |  |  |  |  |
| MSFD art 8,9, 10 * | X | x |  |  | x | x | x |  |
| BSC | X |  |  |  |  |  |  |  |
| MSFD art 8,9, 10 * | X | x | x |  |  |  |  |  |
| WFD* | X |  |  |  |  |  |  |  |
| EEA |  |  |  |  |  |  |  |  |

* Palialexis et al. 2014

| Frequency | 100-81% | 80-61% | 60-41% | 40-21% | 20-0% |
| --- | --- | --- | --- | --- | --- |

## Categorization of RSCs eutrophication indicators according to the DPSIR approach

The Table 2.10 below is an initial effort to associate the RSCs eutrophication indicators with driver, pressure, state or impact in order to address the DPSIR approach. The type of indicators (P, S, I) are according to SEC (2011).  All RSCs cover pressure state and impact indicators. OSPAR and BSC apply indicators on nutrient inputs providing information on drivers. HELCOM Core pressure indicator of nutrient loads is under development in an advanced development phase. UNEP/MAP does not include relevant common indicators although collects data on nutrient inputs and produces assessments of pollution loads (see MEDPOL; Releases, emissions and sources of pollutants in the Mediterranean region. An assessment of 2003-2008 trends; 2012).  A UNEP/MAP indicator on “Release of toxic  substances and nutrients from industrial sector’ representing the emissions from industrial sources from individual facilities within the Mediterranean coastal zone with regard to nutrients and oxygen depleting substances, halogenated hydrocarbons, hydrocarbons and heavy metals is under development.

Table 2.10	RSCs indicators type, D: Driver P: Pressure, S: State. I: Impact.

|  | OSPAR | HELCOM | UNEP/MAP | BSC |
| --- | --- | --- | --- | --- |
| Type | Common indicators | Core indicators | Proposed common indicators | Indicators in Diagnostic Report, 2010 |
| D/P | Waterborne nutrient inputs | Core pressure indicator including water- and airborne nutrient loads is under development in an advanced development phase |  | Inputs of nutrients and HSs from direct (point) sources |
| D/P | Atmospheric nutrient inputs |  |  |  |
| S/P | Winter nutrient concentrations | Concentration of dissolved inorganic nitrogen | 5.1.1 Concentration of key nutrients in the water column | NO3+NO2 |
| S/P |  | Concentration of dissolved inorganic phosphorus |  | PO4 |
| I | Chlorophyll concentrations | Concentration of chlorophyll a | 5.2.1 Chlorophyll-a concentration in the water column | Chlorophyll-a |
| I |  | Water transparency (Secchi depth) |  |  |
| I | Species shift/indicator species: Nuisance species Phaeocystis |  |  |  |
| I |  | Lower depth distribution limit of macrophytes* |  |  |
| I |  | State of soft-bottom macrozoobenthos* |  |  |
| I | Oxygen | Oxygen concentration |  | Hypoxic situations, expansion of zones of hypoxia |

*HELCOM Biodiversity core indicator under development in HELCOM CORESET II project

# RSC and EEA data holdings and data flows related to hazardous substances

## RSCs hazardous substances indicators in relation to MSFD and EEA indicators

RSCs have adopted (HELCOM, OSPAR) or are in the process of adopting (UNEP/MAP, BSC) indicators for hazardous substances covering objectives set by their strategies. Table 3.1 presents the objectives of the four RSCs in parallel to the corresponding criteria of MSFD for the GES Descriptors D8 Contaminants and pollution effects and D9 contaminants in seafood. RSCs indicators for hazardous substances as well as relevant MSFD and EEA indicators are shown in Table 3.2.

Commission Decision 2010/477/EU sets out four criteria for GES Descriptors D8 and D9 dealing with contaminant concentrations in the marine environment and their effects and contaminant concentrations in seafood and five indicators to describe these criteria. The occurrence, origin and extent of significant pollution events from oil spills are addressed by indicator 8.2.2. Other hazardous substances inputs are not included in the list of indicators for D8 and D9 in the COM Decision 210/477/EU but article 8 on initial assessments and annex III to the MSFD Directive lists hazardous substances introduction in the marine environment among the elements to be addressed in the assessment of marine waters.

The four HELCOM ecological objectives for hazardous substances under Baltic Sea Action Plan cover concentrations of hazardous substances, their effects, radioactivity and safe seafood (HELCOM, 2007). HELCOM’s CORESET expert group for hazardous substances developed and proposed 13 core indicators that address all four ecological objectives (HELCOM, 2013a). The core indicators were selected on the basis of their policy relevance, adverse effects to the environment, cost-efficient analyses and available targets. Among these indicators eight were recommended by HELCOM MONAS to the HELCOM Heads of Delegation for approval in 2013 and five were considered as pre-core indicators, which are to be further developed and resubmitted by 2015 for further evaluation. Among the thirteen proposed core indicators, nine measure concentrations of hazardous substances and four measure their effects. The substance indicators include organic contaminants as well as metals and a radioactive isotope. TBT and PAH substance indicators also include effect aspects, imposex and PAH metabolites. The HELCOM core indicators cover the MSFD criteria sufficiently. Oil pollution addressed by the MSFD indicator 8.2.2 was not included in the set of core indicators from the hazardous substances expert group but the biodiversity expert group proposed an indicator for oiled waterbirds – addressing the effects of oil in the water, that was considered as pre-core indicator to be further developed. The information of the number of oil spills is included in the oiled bird indicator as supplementary information, which is also published annually as a Baltic Sea Environment Fact Sheet. The indicators for food safety are the same as for the environment, with the exception that not all substances currently have a GES boundary and therefore they cannot be used in quantitative assessments. Core indicators to assess concentrations in seafood against specific limit levels include the following substances: Cadmium, Lead, Mercury, dl-PCBs, dioxins, Benzo[a]Pyrene and Cesium-137.

The OSPAR Hazardous Substances Strategy sets the objective of preventing pollution of the maritime area by continuously reducing discharges, emissions and losses of hazardous substances, with the ultimate aim of achieving concentrations in the marine environment near background values for naturally occurring substances and close to zero for man-made synthetic substances (OSPAR web page). Tasks considering hazardous substances is implemented through the HASEC committee. Data streams that potentially are used for these assessments include atmospheric inputs, riverine inputs, and assessment of concentrations in the water. The latest assessment includes concentrations of a selection of hazardous substances in marine sediment and in fish and shellfish tissues. The ICOn project (2008 – 2012) aimed to demonstrate the application of methodology developed through OSPAR/ICES on larger scales. The ICON assessment is considered to be suitable for the determination of GES for Descriptor 8 under the MSFD. The analysis led by OSPAR on trends in hazardous substances were used in the part dealing with marine waters of the EEA technical report on hazardous substances in European waters.

UNEP/MAP is in the process of adopting common indicators for hazardous substances addressed by Ecological Objective 9 of the Ecosystem Approach (UNEP/MAP, 2012a). The common indicators have been proposed by the Secretariat taking into account practices of other RSCs and on the basis of experience already gained by the Contracting Parties through their regular MED POL monitoring activities as well as the experience gained by EU Mediterranean countries through their implementation of EU Directives such as the Marine Strategy Framework Directive and the Water Framework Directive. In the latest Integrated Correspondence Groups of GES and Targets meeting (UNEP/MAP, 2014) three operational objectives and six indicators were proposed by UNEP/MAP Secretariat that cover the three MSFD criteria for GES Descriptors8 and 9.

The hazardous substances theme is addressed in the Strategic Action Plan for the Rehabilitation and Protection of the Black Sea (BSSAP) 2009 under Ecosystem Quality Objective 4 (EcoQO 4): Ensure Good Water Quality for Human Health, Recreational Use and Aquatic Biota. EcoQO 4 is subdivided into Eco QO 4a - Reduce pollutants originating from land based sources, including atmospheric emissions and EcoQO 4b - Reduce pollutants originating from shipping activities and offshore installations. BSC in the implementation of BSIMAP (Black Sea Integrated Monitoring and Assessment Program) used four indicators for hazardous substances concerning the two MSFD criteria for D8 (BSC, 2010). Indicators on shipping density, illegal discharges of oil at sea, accidental oil spills from shipping and loads (Inputs of nutrients and HSs from direct (point) sources) were also applied.

The EEA marine thematic indicator on hazardous substances in marine organisms (MAR001) addresses the key policy question: Are the concentrations and trends of hazardous substances in marine organisms acceptable? This indicator describes the levels and trends in European seas of hazardous substances concentrations in marine biota. The indicator is based on the assessment of seven substances (sub indicators): cadmium, lead, mercury, DDT, lindane, HCB and PCBs.

Table 3.1	Hazardous substances objectives of the four RSCs, key policy question of the EEA and environmental objective of the WFD  in parallel to the corresponding criteria of MSFD for the GES Descriptors D8 D9

| ΜSFD | OSPAR | HELCOM | UNEP/MAP | BSC | EEA | WFD |
| --- | --- | --- | --- | --- | --- | --- |
| D8: Concentrations of contaminants are at levels not giving rise to pollution effects D9: Contaminants in fish and other seafood for human consumption do not exceed levels established by Community legislation or other relevant standards. | Strategic objective: To prevent pollution of the OSPAR maritime area by continuously reducing discharges, emissions and losses of hazardous substances (as defined in Annex 1), with the ultimate aim to achieve concentrations in the marine environment near background values for naturally occurring substances and close to zero for man-made synthetic substances | Strategic goal: The Baltic Sea life undisturbed by hazardous substances | EQ: Contaminants cause no significant impact on coastal and marine ecosystems and human health. | 1. LBSA Protocol 20095 2. BSSAP 2009 – EcoQO4 - Ensure Good Water Quality for Human Health, Recreational Use and Aquatic Biota6 |  |  |
| Criteria1 | Ecological Quality Objectives 2 | Ecological objectives3 | Proposed operational objectives 4 |  | Key policy question7 | Environmental Objective8 |
| 8.1 Concentration of contaminants | Concentrations of mercury in the eggs of Common Tern (Sterna hirundo) and Eurasian Oystercatcher (Haematopusostralegus) breeding adjacent to the eight industrialised estuaries, should not exceed concentrations in eggs of the same species breeding in similar habitats in south-western Norway and in the Moray Firth | Concentrations of hazardous substances close to natural levels | 9.1 Concentration of priority contaminants is kept within acceptable limits and does not increase |  | Are the concentrations and trends of hazardous substances in marine organisms acceptable? | Good chemical status |
|  | Concentrations of organochlorines in the eggs of Common Tern (Sterna hirundo) and Eurasian Oystercatcher (Haematopusostralegus) breeding adjacent to the eight industrialised estuaries, should not exceed set values. |  |  |  |  |  |
|  |  | Radioactivity at pre-Chernobyl level |  |  |  |  |
| 8.2 Effects of contaminants | The proportion of oiled common guillemots should be 10% or less of the total found dead or dying in all areas of the North Sea. | Viable Populations of species | 9.2 Effects of released contaminants are minimized |  |  |  |
|  | The average level of imposex(development of male characteristics by females) in female dog whelks should be consistent with specified levels. | Healthy wildlife |  |  |  |  |
| 9.1 Levels, number and frequency of contaminants | . | All fish safe to eat | 9.4 Levels of known harmful contaminants in major types of seafood do not exceed established standards |  |  |  |

1Commission Decision 2010/477/EU

2OSPAR, 2009a

3HELCOM, 2007

4UNEP/MAP, 2014

5LBSA2009

6BSSAP, 2009

7http://www.eea.europa.eu/data-and-maps/indicators/

8WFD 2000/60/EC

Table 3.2	Hazardous substances indicators set by the MSFD, the RCSs, and EEA.

| ΜSFD | OSPAR | OSPAR | HELCOM | HELCOM | UNEP/MAP | BSC | EEA |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Indicators1 | Common indicators2 | Candidate indicators2 | Core indicators3 | Pre-core indicators3 | Proposed indicators4 | Indicators in Diagnostic Report, 2010 5 | Indicators6 |
|  | Inputs of Hg, Cd and Pb via water and air |  |  |  |  | Loads: Inputs of nutrients and HSs from direct (point) sources |  |
| 8.1.1 Concentration of the contaminants mentioned above, measured in the relevant matrix (such as biota, sediment and water) in a way that ensures comparability with the assessments under Directive 2000/60/EC | Metal (Hg, Cd, Pb) concentrations in biota |  | Metals (lead, cadmium and mercury) |  | Concentration of key harmful contaminants* in biota, sediment or water | Hazardous substances in biota | MAR001Hazardous substances in biota Sub indicators: Cadmium, Mercury, Lead, HCB, Lindane, PCB, DDT |
|  | Metal (Hg, Cd, Pb) concentrations in sediment |  |  |  |  | Hazardous substances in sediments |  |
|  | PCB concentrations in biota |  | Polychlorinated biphenyls (PCB) and dioxins and furans: CB-28, 52, 101, 118, 138, 153and 180; WHO-TEQ of dioxins, furans +dl-PCBs |  |  |  |  |
|  | PCB concentrations in sediments |  |  |  |  |  |  |
|  |  | PAHs concentrations in biota | Polyaromatic hydrocarbons and their metabolites: US EPA 16 PAHs / selected metabolites. |  |  |  |  |
|  | PAHs concentrations in sediments |  |  |  |  |  |  |
|  |  | Organotin concentrations in biota | Tributyltin (TBT) and imposex |  |  |  |  |
|  | Organotin concentrations in sediments |  |  |  |  |  |  |
|  | PBDE concentrations in biota |  | Polybrominated biphenyl ethers (PBDE): BDE-28, 47, 99, 100, 153 and 154 |  |  |  |  |
|  | PBDE concentrations in sediments |  |  |  |  |  |  |
|  |  | HCB (hexachlorobenzene) concentrations in biota |  |  |  |  |  |
|  |  | HCB (hexachlorobenzene) concentrations in sediments |  |  |  |  |  |
|  |  | HCBD (hexachlorobutadiene) concentrations in biota |  |  |  |  |  |
|  |  | HCBD (hexachlorobutadiene) concentrations in sediments |  |  |  |  |  |
|  |  |  | Hexabromocyclododacene (HBCD) |  |  |  |  |
|  |  |  | Perfluorooctane sulphonate (PFOS) |  |  |  |  |
|  |  |  |  | Pharmaceuticals: Diclofenac, EE2 (+E1, E2, E3 + in vitro yeast essay |  |  |  |
|  |  |  | Radioactive substances: Caesium-137 in fish and surface waters |  |  |  |  |
| 8.2.1 Levels of pollution effects on the ecosystem components concerned, having regard to the selected biological processes and taxonomic groups where a cause/effect relationship has been established and needs to be monitored | Imposex/intersex |  | Tributyltin (TBT) and imposex |  | Level of pollution effects of key contaminants where a cause and effect relationship has been established |  |  |
|  |  | Externally visible fish diseases |  | Fish diseases – a fish stress indicator |  |  |  |
|  |  | Lysosomal stability (LMS) |  | Lysosomal Membrane Stability – a toxic stress indicator |  |  |  |
|  |  | Bile metabolites (of PAHs) | Polyaromatic hydrocarbons and their metabolites: US EPA 16 PAHs / selected metabolites. |  |  |  |  |
|  |  | Micronuclei (MN) |  | Micronuclei test – a genotoxicity indicator |  |  |  |
|  |  | EROD |  |  |  |  |  |
|  |  |  |  | Eelpout and amphipod embryo malformations |  |  |  |
| 8.2.2 Occurrence, origin (where possible), extent of significant acute pollution events (e.g. slicks from oil and oil products) and their impact on biota physically affected by this pollution |  | Oiled birds |  | Number of waterbirds being oiled annually (biodiversity core indicator) | Occurrence, origin (where possible), extent of significant acute pollution events (e.g. slicks from oil, oil products and hazardous substances) and their impact on biota affected by this pollution | Illegal discharges of oil at sea Accidental oil spills from shipping Shipping density | EN14 Discharge of oil from refineries and offshore installations7 EN15 Accidental oil spills from marine shipping7 |
| 9.1.1 Actual levels of contaminants that have been detected and number of contaminants which have exceeded maximum regulatory levels |  |  | HELCOM core indicators to assess concentrations against specific limit levels; substances are Cadmium, Lead, Mercury, dl-PCBs, dioxins, Benzo[a]Pyrene and Cesium-137 |  | Actual levels of contaminants that have been detected and number of contaminants which have exceeded maximum regulatory levels in commonly consumed seafood |  |  |
| 9.1.2 Frequency of regulatory levels being exceeded |  |  |  |  | Actual levels of contaminants that have been detected and number of contaminants which have exceeded maximum regulatory levels in commonly consumed seafood |  |  |
|  |  |  |  |  | Percentage of intestinal enterococci concentration measurements within established standards |  |  |

*Priority contaminants as listed under the Barcelona Convention and LBS Protocol

1 Commission Decision2010/477/EU

2 ANNEX 6 (Ref. § 4.18) OSPAR Convention for the Protection of the Marine Environment of the North-East Atlantic Meeting of the Coordination Group (CoG) London (Secretariat): 21-22 November 2013

3HELCOM 2013a

4 UNEP/MAP, 2014b

5BSC (2010)

6http://www.eea.europa.eu/data-and-maps/indicators/

7not active any more

## RSCs methodologies for hazardous substances indicators and assessment tools

HELCOM

The core indicators will be included as mandatory parameters to the coordinated monitoring programme. Guidelines for monitoring of contaminants and their effects are available in the Manual for Marine Monitoring in the COMBINE Programme of HELCOM at http://helcom.fi/action-areas/monitoring-and-assessment/manuals-andguidelines/combine-manual. A short overview of the revisions needed in the coordinated monitoring programme presented in HELCOM (2013a, see table below) showed that six of the 13 indicators are at least somehow included in the manual and seven substances or effect indicators need to be included in the manual. In addition, there is a need to consider adding and removing species from the manual, changing sampled tissue, sharpening the details of the sample and agreeing on an effective placement of the monitoring stations. Guidelines for revised HELCOM monitoring programmes addressing all core indicators are being developed in the HELCOM MORE project by the end of year 2014.

SOURCE: HELCOM, 2013a

For all the proposed core indicators, targets showing the boundary for good environmental status (GES) have been determined (HELCOM, 2013a). The status of the hazardous substances core indicators is presented in three status classes: good, moderate and bad. ‘Good’ environmental status and ‘moderate’ status are determined by the GES boundary or GES threshold, while ‘bad’ represents a condition of particularly high concentration or adverse impacts. The GES boundaries were primary selected using EU Environmental Quality Standards (EQSs) and also the OSPAR Environmental Assessment Criteria (EAC).

HELCOM guidelines on hazardous substances indicators methodologies can be found at http://www.helcom.fi/baltic-sea-trends/hazardous-substances/indicators/

An overview of the HELCOM guidelines on hazardous substances indicators methodologies is presented in Table 3.3.

Table 3.3. Overview of the HELCOM guidelines on hazardous substances indicators methodologies

| Core indicator | Preferred matrix | Assessment criteria | Reference |
| --- | --- | --- | --- |
| Metals (lead, cadmium and mercury) | Shellfish or sediment for local surveys Fish muscle (Hg) and liver (Cd, Pb) for regional surveys | OSPAR Background Assessment Criteria (BAC) as the GES boundary EU food safety limits for the boundary of ‘bad’ status in mussels and fish ERL threshold in sediment | Nyberg, et al, 2013a |
| Polyaromatic hydrocarbons (PAH) and their metabolites - US EPA 16 PAHs / selected metabolites | Bivalves and sediment for PAHs Fish for PAH metabolites | Environmental Assessment Criteria of OSPAR (EAC) or Environmental Quality Standards (EQS) of the EU Background Assessment Criteria for PAHS for which EAC or EQS are not available | Nyberg, et al, 2013b |
| Caesium-137 (Radioactive substances) in fish and surface waters | Herring flounder plaice surface seawater (samples 0–10 m). | Average concentrations of 137Cs prior the Chernobyl accident have been used as target values. These are for herring (2.5 Bq/kg), flounder and plaice (2.9 Bq/kg) and seawater (15 Bq/kg). | Herrmann et al., 2013 |
| Tributyltin (TBT) and imposex | Biota (mussels or gastropoda snails). Fish (liver) can be used as alternative if molluscs are not present, Sediments – Secondary approach Water – Optional | The GES boundary follows the OSPAR Environmental Assessment Criteria (EAC) and the Environmnetal Quality Standards (EQS) of the EU. Biological effects assessment classes for ECOQO on imposex/intersex are established for six gastropod species. | Nyberg et al., 2013c |
| Hexabromocyclododecane (HBCD) | Biota Sediment | The GES boundary is the Environmental Quality Standard (EQS) of EU. The EQS is 167 μg kg-1 fish ww. An alternative approach is to use the Quality Standard for sediment (170 μg kg-1 dw) (WFD WG E Dossier 19.1.2012). | Nyberg et al., 2013d |
| Perfluorooctane sulphonate (PFOS) | Fish Sediment | The GES boundary is the Environmental Quality Standard (EQS), of EU, The EQS is 9.1 μg kg-1 fish ww. An alternative approach is to use the EU Quality Standard for water (0.23 μg l-1) There is no QS for benthic organisms (sediment). | Nyberg et al., 2013e |
| PolybrominatedDiphenyl Ethers (PBDE) | Biota e.g. fish and mussels (primary matrix) Sediment (secondary matrix.) | GES boundary is the EU Environmental Quality Standard for the sum of polybrominateddiphenyl ethers (congeners 28, 47, 99, 100, 153 and 154): 0.0085 μg kg-1 ww fish. The GES boundary for sediment (a secondary approach) is proposed to be 4.5 μg kg-1 dw.. | Nyberg et al., 2013f |
| Polychlorinated biphenyls (PCB) and dioxins and furans | Sediments Biota | The seven PCBs should be monitored but the core indicator assesses primarily two congeners only: CB-118 (dioxin like) and CB-153 (non-dioxin like). Tentatively the OSPAR EACs for these two congeners are suggested to be used. For dioxins, the GES boundary of 4.0 ng kg-1 ww WHO-TEQ for dioxins and 8.0 ng kg-1 ww WHO-TEQ for dioxins and dl-PCBs. | Boalt et al., 2013 |

The HELCOM Hazardous Substances Status Assessment Tool (CHASE)

The HELCOM Hazardous Substances Status Assessment Tool (CHASE) is a multimetric indicator-based tool developed for the HELCOM integrated thematic assessment of hazardous substances in the Baltic Sea (HELCOM, 2010c).

The quantification of the “hazardous substances status” is based on a Contamination Ratio (CR), which is the ratio of the current status (measurement of the concentration of a substance or biological effect) and a threshold level or quality criterion, which is used as an approximation for an environmental target for that particular substance or biological effect. For each of the four HELCOM ecological objectives dealing with hazardous substances, the CRs of all substances or indicators are integrated to yield a status classification (“high”, “good”, “moderate”, “poor” or “bad”) of that particular ecological objective. CR values are summed within an element and then divided by the square root of the number of indicators. As a result, the status is not totally dependent on the number of indicators in the element, it does not give much weight to several low-CR indicators and it does not “mask” individual indicators with high CR values. The primary classification is based on the threshold level, which defines a moderate or good status for each element. This boundary is ecotoxicologically justifiable (if the threshold is based on ecotoxicology). The further classification is not intended to be understood strictly as an ecotoxicological status of the environment, but rather as a deviation from the boundary condition. Moderate status is defined as a CR sum value >1.0. Poor status is assigned for a CR sum value >5.0 and bad status is reached at a value of 10.0. At the opposite end, a high status is a CR sum value <0.5.

Subsequently, the CHASE tool makes use of a ‘one out, all out’ principle among the four ecological objectives. The ecological objective receiving the lowest status classification serves as the overall classification of the assessed site or area, giving the classification of the “hazardous substances status” of that site or area according to one of five classes. “High” and “good” classes indicate that areas are not disturbed by hazardous substances, while “moderate”, “poor” and “bad” indicate different degrees of disturbance by hazardous substances. The tool also carries out an estimate of the quality of the assessment results.

The selection of indicators for CHASE was based on two primary criteria: (1) an indicator must have a threshold level which is preferably ecotoxicologically or statistically justified, and (2) an indicator must reliably describe the status of hazardous substances in the Baltic Sea. In addition, the following criteria were set: (3) a substance cannot be entered into an element more than once, and (4) the indicators under elements 1 and 4 are to be based on measurements primarily from bivalves, secondarily from fish, and thirdly from sediment.

OSPAR

The OSPAR assessment of levels and trends in marine hazardous substances is prepared using the methods for data screening, treatment of quality assurance information, temporal trend assessment and assessment against assessment criteria which have been used in previous CEMP assessments and are described in the CEMP Assessment Manual .   The assessment criteria used to assess environmental concentrations of hazardous substances are set out  in OSPAR agreement on CEMP Assessment Criteria for the QSR 2010 (OSPAR agreement 2009-2). The  derivation of these assessment criteria for hazardous substances is discussed in a Background Document  on CEMP Assessment Criteria for the QSR 2010 (OSPAR, 2009b). The assessment criteria reflect a two  stage process in which data are compared to concentrations that are unlikely to give rise to unacceptable  biological effects (c.f. Environmental Assessment Criteria, EACs) and then against Background  Concentrations (BCs) or zero, expressed as Background Assessment Concentrations (BACs). The latter reflects the objective of the OSPAR Hazardous Substances Strategy that concentrations should be at or  close to background levels for naturally occurring substances or to zero for man-made substances.

UNEP/MAP

For EO 9, five common indicators are currently proposed (Table 3.4, UNEP/MAP, 2014a).The process of identifying targets and corresponding GES for EO5 has initiated in 2012 (UNEP/MAP 2012b). Thresholds values are not yet available but MED POL has made some preparatory work to provide initial background information on the methodology to be followed for the definition of the assessment criteria for hazardous substances in the Mediterranean (UNEP/MAP, 2011c). The proposed methodology is the one developed by OSPAR. Using Mediterranean data from the MED POL database and applying the OSPAR methodology, a first estimate of the background concentrations (BCs) and the background assessment concentrations (BACs) of trace meals (mercury, cadmium and lead) and organic contaminants (chlorinated hydrocarbons and PAHs) in sediments and biota in the Mediterranean basin was provided (UNEP/MAP, 2011c).This work would need to be further discussed during national expert meetings organized by MAP MED POL during 2014-2015.

Table 3.4	Common indicators, GES and targets for EO9 contaminants Adapted from UNEP/MAP (2014a)

| Common Indicator | GES | Targets |
| --- | --- | --- |
| 9.1.1 Concentration of key harmful contaminants1in biota, sediment or water | Level of pollution is below a determined threshold defined for the area | State Concentrations of specific contaminants below EACs or below reference concentrations2 No deterioration trend in contaminants concentrations in sediment and biota from human impacted areas, statistically define Pressure Reduction of contaminants emissions from land based sources3 |
| 9.2.1 Level of pollution effects of key contaminants where a cause and effect relationship has been established | Concentrations of contaminants are not giving rise to pollution effects. | State Contaminants effects below threshold4 |
| 9.3.1 Occurrence, origin (where possible), extent of significant acute pollution events (e.g. slicks from oil, oil products and hazardous substances) and their impact on biota affected by this pollution | Occurrence of acute pollution events are reduced to the minimum | State Decreasing trend in the occurrences of acute pollution events. Pressure Decreasing trend in the operational releases of oil and other contaminants from coastal, maritime and off-shore activities. |
| 9.4.1 Actual levels of contaminants that have been detected and number of contaminants which have exceeded maximum regulatory levels in commonly consumed seafood5 | Concentrations of contaminants are within the regulatory limits for consumption by humans | State Concentrations of contaminants are within the regulatory limits set by legislation6 |
| 9.5.1 Percentage of intestinal enterococci concentration measurements within established standards | Concentrations of intestinal enterococci are within established standards | State Increasing trend in the percentage of intestinal enterococci concentration measurements within established standards |

1Priority contaminants as listed under the Barcelona Convention and LBS Protocol found at http://www.unep.ch/regionalseas/main/med/mlbsprot.html

2Use for further work on reference conditions ERL for sediments taking into account specifics of the Mediterranean.

3Thresholds to be set by COP19.

4Reduction programmes are already in place through the Protocols of the Barcelona Convention and the Marine Litter Regional Strategy.

6Thresholds to be set by COP19.

Common Indicator 11: Concentration of key harmful contaminants measured in the relevant matrix (biota, sediment, seawater)

Data and relevant methodologies for common indicator 11 are provided in the framework of the MED POL monitoring programme (UNEP/MAP, 2011b). The information reported by countries for the assessment of this indicator ranges from heavy metals and petroleum hydrocarbons only, to 3 groups of contaminants in water, sediment and biota: synthetic contaminants (PAHs, PCBs, DDTs, aldrin, endrin and dieldrin), non –synthetic contaminants (Cd, Pb, Cu, Zn, Hg) and petroleum hydrocarbons, and radionuclides (137 Cs). However, many countries do not provide monitoring data on a regular basis and there are gaps in spatial and temporal coverage of the Mediterranean coastline. An overview of MED POL Phase III and IV data provided by the countries is shown in Table 3.5.

For the Thematic Assessment Report on Hazardous Substances in the Mediterranean coastal environment prepared using MED POL data (UNEP/MAP, 2011d), only trace metals were found suitable for assessment at the regional level in sediments and trace metals and organochlorinated compounds in marine biota. As regards species applied, the bivalve Mytilus galloprovincialis and the benthic fish Mullus barbatus, were the more common and widely analyzed species in the region.

Table 3.5	MED POL Phase III and IVdata provision by each country until 2010. SOURCE: UNEP/MAP 2011

Common Indicator 12: Level of pollution effects of key contaminants where a cause and effect relationship has been established.

Biological effects of contaminants are less measured in the Mediterranean coastal environment than contaminant concentrations. Data on biological effects of contaminants have been generated for the MED POL Phase IV contaminants monitoring programme, involving the biomarker lysosomal membrane stability. Outside of the MED POL biological effects monitoring activities, biomarkers including EROD activity, lysosomal membrane stability, stress on stress, acetylcholinesterase activity, metallothionein content and frequency of micronuclei occurrence measured by a small number of Mediterranean countries. Sentinel organisms include Mullus barbatus and Mytilus edulis.

Since the most widely used specific technique where a cause and effect relationship has been established is the measurement of TBT effects (imposex) on gastropods, the possibility to use available information for TBT thresholds from other regions in order to propose similar effects thresholds for the Mediterranean is considered. However it has been mentioned that the “imposex" indicator for the biological effect of TBT is not suitable for the French Mediterranean Sea, and is operational only in coastal areas. For the time being, it is not considered possible to define thresholds in relation to effects, using a quantitative approach, for other contaminants. The need to develop and test more contaminant-specific techniques is pointed out and will be the objective of expert group meetings organized by MAP during 2014-2015.

Common Indicator 13: Occurrence, origin (where possible) extent of acute pollution events (e.g. slicks from oil, oil products and hazardous substances) and their impact on biota affected by this pollution)

Acute pollution events (oil spills) are followed and recorded in the framework of the Prevention and Emergency Protocol, 2002 by the MAP Regional Marine Pollution Emergency Centre (REMPEC), which is also reviewing the maritime traffic in the Mediterranean providing information on routine operations. REMPEC has data on shipping accidents that caused oil or other hazardous and noxious substances (HNS) pollution in the Mediterranean or were likely to cause it.

Moreover, in the field of scientific assistance for oil spill drift forecast, during the last few years, REMPEC developed a strong relationship with the Mediterranean Operational Oceanographic Network (MOON) with regard to operational use of forecasting and backtracking system for oil spills based on meteo oceanographic observations and models. REMPEC and MOON have also signed a co-operation agreement to formalise their working relationship and define the type of common activities to be implemented.

Common Indicator 14: Actual levels of contaminants that have been detected and number of contaminants which have exceeded maximum regulatory levels in commonly consumed seafood

The information reported by countries for the assessment of this indicator includes concentrations of heavy metals (Pb, Cd and Hg) in fish tissues, heavy metals and different persistent organic pollutants according to EU regulations with partial availability of data for regulated substances, contaminants data on a wide variety of marine commercial species, and trends of bioaccumulation in the biota and functional groups used as bio-indicators (Mytilus galloprovincialis and Mullus barbatus/ Boops boops, respectively). Some monitoring programmes still need improvements to provide an appropriate assessment. No corresponding data has been collected under MED POL Phase IV.

BSC

The BSIMAP is based on National monitoring programs financed by the Black Sea countries. Each country decides which and how many stations will use, the frequency of sampling and which stations will be used for the reporting for the BSIMAP. The BSIMAP provides recommendations on parameters, frequency and methodologies to be used.

Mandatory parameters under the PMA (Pollution Monitoring and Assessment) Advisory Group, include heavy metals (Hg, Cu, Cd, Pb), and Total Petroleum Hydrocarbons (TPHs) in water; heavy metals (Hg, Cu, Cd, Pb), pesticides (DDT, DDD, DDE, α-HCH, β-HCH, γ-HCH ), PCBs total, TPHs and phenols in sediments; and heavy metals (Hg, Cu, Cd, Pb), pesticides (DDT, DDD, DDE, γ-HCH ) and PCBs in biota.  Proposed media for contaminants in biota are bivalves, anchovies, sprat, turbot, horse mackerel. Pressures from Hot Spots (rivers, municipal and industrial sources) are mandatory parameters under the LBS (Control of Pollution from Land Based Sources) Advisory Group.

## Hazardous substances assessment in the WFD

Article 16 of the Water Framework Directive 2000/60/EC requires the establishment of a list of priority substances, to be selected amongst those presenting a significant risk to or via the aquatic environment at EU level. Risk to or via the aquatic environment is identified by: (a) risk assessment carried out under Council Regulation (EEC) No 793/93 (1), Council Directive 91/414/EEC (2), and Directive 98/8/EC of the European Parliament and of the Council (3), or (b) targeted risk-based assessment (following the methodology of Regulation (EEC) No 793/93) focusing solely on aquatic ecotoxicity and on human toxicity via the aquatic environment. When necessary in order to meet the timetable set by WFD, risk to, or via the aquatic environment, can be identified by a simplified risk-based assessment procedure based on scientific principles taking particular account of evidence regarding the intrinsic hazard of the substance concerned and in particular its aquatic ecotoxicity and human toxicity via aquatic exposure routes, and evidence from monitoring of widespread environmental contamination and other proven factors which may indicate the possibility of widespread environmental contamination,

Decision 2455/2001/EC established the First list, and Directive 2008/105/EC (the Environmental Quality Standards Directive – EQSD) set environmental quality standards (EQS) for 33 priority substances and certain other pollutants in surface waters (river, lake, transitional and coastal). According to Annex V, point 1.4.3 of the WFD and Article 1 of the EQSD, good chemical status is reached for a water body when it complies with the EQS for all the priority substances and other pollutants listed in Annex I of the EQSD. Directive 2013/39/EU amended Directives 2000/60/EC and 2008/105/EC as regards priority substances, adding 8 substances in the list.

EEA indicator Hazardous substances in marine organisms

The EEA indicator Hazardous substances in marine organisms (MAR 001) describes the levels and trends in European seas of hazardous substances concentrations in marine biota, based on the individual assessment of monitoring data for the following substances: Cadmium, Lead, Mercury, DDE, p,p', Gamma-HCH (Lindane), Hexachlorobenzene (HCB), PCB101 (2,2',4,5,5'-pentachlorobiphenyl), PCB118, PCB138 (2,2',3,4,4',5'-hexachlorobiphenyl), PCB153 (2,2',4,4',5,5'-hexachlorobiphenyl), PCB180 (2,2',3,4,4',5,5'-heptachlorobiphenyl), PCB28 (2,4,4'-trichlorobiphenyl), PCB52 (2,2',5,5'-tetrachlorobiphenyl). The indicator is based on data for substances measured in Atlantic herring (Clupea harengus)

in the Baltic Sea, blue mussel (Mytilus edulis), Atlantic cod (Gadus morhua) and flounder (Platichtys flesus) in the  North-east Atlantic Ocean,  the Mediterranean mussel (Mytilus galloprovinicialis) in the Mediterranean Sea and the Black Sea.

The data used in this indicator is part of the WISE - State of the Environment (SoE) data, available in Waterbase - TCM (Transitional, Coastal and Marine) waters. There is generally good data coverage for concentrations in the north-east Atlantic, except for Portugal, and in the Baltic Sea.The Mediterranean Sea was only represented by data from Croatia, France and Italy and for the Black Sea, the only available data were Romanian mussel data.

Details on methodologies for calculation of this indicator can be found at http://www.eea.europa.eu/data-and-maps/indicators/hazardous-substances-in-marine-organisms/hazardous-substances-in-marine-organisms-3

## RSCs data streams analysis for identifying hazardous substances indicators

ICES is the responsible data centre for the marine environmental monitoring data of HELCOM and OSPAR. Contracting Parties are obliged to report CEMP and COMBINE monitoring data to ICES on an annual basis by 15 September every year using the agreed reporting formats. The submission is done by email. Data are organized in relational data base and the metadata are ISO19115 compliant. Both data and their metadata are available on-line through Web Services. HELCOM pollution load compilation database is hosted and maintained by HELCOM data consultant SYKE with direct email submissions on riverborne, coastal and point source loads by 1th of November every year and more detailed reporting including sources on land approximately every sixth year. MORS Radionuclide discharge database (Including discharges of Cesium-137) from nuclear installations is hosted by Finnish radiation safety authority STUK and MORS environmental database containing information on radionuclide concentrations in water and biota by HELCOM secretariat. Radionuclide data is reported by email once a year by 1stof September.

HELCOM

Information on the parameters on hazardous substances that the HELCOM data streams include can be found below:

- HELCOM COMBINE database holding data on hazardous substances in sediment, seawater and biota

- HELCOM Pollution Load Compilation 5.5 (PLC 5.5) dataset containing all waterborne nutrient and hazardous substances loads gathered by HELCOM contracting parties within pollution load monitoring1

- HELCOM Pollution Load Compilation 6 (PLC 6) ongoing comprehensive assessment of water- and airborne inputs and their sources to the Baltic Sea during the period 1994-2014 with more detailed assessment for 20141

- HELCOM Monitoring of Radioactive Substances Expert Group (HELCOM MORS EG) reporting to HELCOM MORS environmental database that holds data on radioactive substance concentrations in Baltic Sea seawater, sediment and biota2

- HELCOM Monitoring of Radioactive Substances Expert Group (HELCOM MORS EG) reporting to HELCOM MORS discharge database3that holds data on discharges of radionuclides to the Baltic Sea3

1CP submissions to HELCOM data consultant (SYKE) by email. Currently there is no online access point. Need for developing SQL database and interface to modernize reporting, QA and maximize the use of and access to data. The development work is underway in HELCOM PLUS project.

2 CP submission to HELCOM secretariat

3CP submission to HELCOM data consultant (STUK, Finnish Radiation and Nuclear Safety Authority)

OSPAR

Information on the parameters on hazardous substances that the OSPAR data streams include can be found below:

- “Comprehensive Atmospheric Monitoring Programme (CAMP)” includes data and model results on wet and dry deposition of hazardous substances.

- “Comprehensive Study of Riverine Inputs and direct Discharges (RID)” includes data compilations of yearly loads of hazardous substances (Cd, Hg, Cu, Pb, Zn, g-HCH, PCBs) based on measurements (large rivers) and estimateds (smaller tributaries).

- “Comprehensive Environmental Monitoring Programme (CEMP)” contains data on Metals and metalloids, Organobromines, Chlorobiphenyls, Organochlorines (general), Dichloro-diphenyl-trichloroethane (DDTs), Cyclodienes, Dioxins, Hexachlorocyclohexanes, Organic esters, Organofluorines, Pesticides (general), Major organic constituents, Organo-metallic compounds, Polycyclic aromatic hydrocarbons (PAHs).

UNEP/MAP

MED POL monitoring data reported by the Contracting Parties to UNEP/MAP-Barcelona Convention are held in the MED POL database (http://195.97.36.231/medpol/). The data are reported by the end of the year by email. However not all contracting parties report data on a regular basis and there are gaps both in terms of temporal and spatial coverage. The data are held in excel format. Metadata are standardized using internal conventions and rules. The MED POL Info System is a networked information system intended to provide the Contracting Parties and MED POL Unit with the tools to manage, share, preserve and analyse MED POL data to MED POL users. The Info System is not yet operational but it will soon be available.

Information on the parameters that the UNEP/MAP data streams include can be found below:

- MED POL data base holding data on contaminants in sediments (Total mercury, total cadmium (mandatory), chromium, copper, lead, zinc, halogenated hydrocarbons, PAHs) Contaminants in biota (Total mercury, total cadmium (mandatory), halogenated hydrocarbons, PAHs, arsenic, chromium, copper, lead, zinc,) and biological effects (micronuclei frequency, DNA damage, EROD activity, lysosomal membrane stability and metallothionein content).

- Land-based pollution sources database holding national data on pollutants industrial and municipal discharges

- Regional Marine Pollution Emergency Response Centre (REMPEC )database on alerts and accidents in the Mediterranean Sea holding data on oil-related accidents reported since 1977 and  other hazardous and noxious substances (HNS) related accidents reported since 1988.

BSC

The Black Sea Integrated Monitoring and Assessment Programme (BSIMAP) provides common data/information reporting formats and the contracting parties have the obligation to report to the BSC on an annual basis using these agreed formats.  Data are annually collected, though with a different frequency per year.

The BSC Regional Database on Pollution is a component of the Black Sea Information System (BSIS) held by the Regional Activity Center for Pollution Monitoring and Assessment (RAC PMA). The Black Sea Regional Database on Pollution is available through the web site:http://rdbp.sea.gov.ua.  Passwd is required for external users to access it. The database is built using a RDBMS system. Metadata descriptions and their standards follow the SeaDataNet approach. Detailed description of the BSC database can be found  rdbp.sea.gov.ua/docs/FINAL_REPORT_RDB.doc in the report “The development of a new version of the “Regional Data base on Pollution” of BSIS” prepared by the RAC PMA under the BSC-Baltic2Black Service contract (rdbp.sea.gov.ua/docs/FINAL_REPORT_RDB.doc).

Information on the parameters that the BSC data streams include can be found below:

- Regional Data base on Pollution holding data on contaminants in water (PAHs, PCBs, pesticides, petroleum hydrocarbons, trace (heavy) metals, bacteria, phenols, detergents, radionuclides), sediment (PAHs, PCBs, pesticides, petroleum hydrocarbons, radionuclides, trace (heavy) metals, phenols)and biota (PAHs, pesticides, trace (heavy) metals). Data reported to BSC by the Advisory Group on Pollution Monitoring and Assessment (AG PMA)

- Data on pressures from Hot Spots (rivers, municipal and industrial sources) reported to the BSC by the Advisory Group on Control of Pollution from Land Based Sources (AG LBS). The data are stored in the BSC information system as excel files and are not accessible online.

- Data on number of ships calling at ports, accidental oil spills from shipping and illegal discharges as number and imposed fines reported to the BSC by the Advisory Group on Environmental Safety Aspects of Shipping (AG ESAS). The data are stored in the BSC information system as excel files and are not accessible online.

EEA

EEA data flows are described in section 2.3.

The WISE SoE TCM dataset contains among other data, chemical quality data on hazardous substances in biota, sediment and seawater, as well as data on direct discharges and riverine input loads.

ANALYSIS

It must be noted that data are not regularly reported to the RSCs for all parameters and by all contracting parties and for some parameters monitoring is currently developing. Thus an analysis was performed on hazardous substances data relevant to MSFD indicators that are actually reported to the RSCs by the contracting parties. The priority substances covered by the WFD directive (2008/105/EC, 2013/39/EC) were the parameters considered for the analysis of data flows on hazardous substances concentrations. Data reported to the RSCs in 2012 by the contracting parties that are EU MS was used in this analysis.  Data reported to HELCOM and OSPAR was obtained from ICES, UNEP/MAP data was obtained from the MED POL data base and BSC data was provided by the IRIS SES project1. Details on data management aspects such as stewardship, quality control, updates of the data sets, sampling frequencies etc. can be found in Annex 2. Data reported through Eionet within 2008-2012 was provided by ICES.

SUMMARY

An overview of data on hazardous substances relevant to MSFD indicators that are actually reported to the RSCs by the contracting parties and at what frequencies across RSCs, is shown in the following tables (Tables 3.6-3.8). The frequency of reporting for each RSC (% of CPs that are EU MS reporting to each RSC) for each parameter is categorized in classes represented by different colors. The chemical contaminants used by the MS for reporting under articles 8, 9 and 10 of the MSFD, summarized and evaluated by JRC (named as in the MS reports) (Palialexis et al. 2014) are also shown.

Tables 3.9-3.11 presents the data on hazardous substances reported by the RSCs grouped in chemical categories used by the ICES. The percentage of counties reporting to each RSC is also shown.

The WFD sets out a list of priority substances that do not exactly much the priority substances of the RSCs and in some cases the grouping of substances. Furthermore the CPs in each RSC show inconsistency in the substances they cover. Although the data reported to OSPAR and HELCOM overall cover a good number of the WFD priority substances, the percentage of countries reporting for each substance is low particularly for water and sediment data (less than 40%) and only for 11 substances in biota ranges between 40 and 80%. As regards UNEP/MAP and BSC, the reported data cover few of the WFD priority substances. The percentage of countries reporting to UNEP/MAP is low, in most cases lower than 20%, whereas percentage of counties reporting to BSC is 50% since one of the two countries included in the analysis is reporting data on hazardous substances. All the hazardous substances reported to the RSCs were used by MS in reporting for the implementation of Art 8, 9 and 10 of the MSFD. When considering chemical groups used by the ICES, a high consistency of reporting between OSPAR and HELCOM is shown.

Hazardous substances are reported though Eionet although reporting is different than RSC reporting (i.e. individual substances reported and number of stations are different). Overall more data is reported through ICES. It must be noted that data flows through ICES in fact refers to RSC data flows that are recycled as Eionet data flows, so although agreements are made with RSC or the Eionet, it is in fact possible to use the same data flow for multiple purposes.

Information on data reporting relevant to MSFD descriptor 8.2.1 on pollution effects in 2012 can be found in Annex 2. Three of the OSPAR CPs and two of the HELCOM CPs reported biological effects parameters used by the MS for reporting under articles 8, 9 and 10 of the MSFD, summarized and evaluated by JRC (Palialexis et al. 2014). These included imposex in gastropods, fish liver pathologies, EROD and levels of bile metabolite in fish. Additional biological effect parameters used in MSFD reporting were reported to OSPAR and HELCOM in previous years (see Annex 2). Relevant parameters were not reported by the UNEP/MAP and BSC CPs.

Over 50% of the OSPAR and 70% of the HELCOM contracting parties reported on riverine inputs and direct inputs (point sources) of contaminants. The reported parameters included direct inputs and riverine inputs of Cd, Hg, Cu, Pb, Zn, g-HCH, and PCBs. Details on parameters reported by each country can be found in Annex 2. The BSC data used in the analysis were reported in the period 2001-2008 (BSC, 2010). The two CPs reported metal and TP inputs from municipal, industrial and riverine sources during that period. Pollutant inputs in the UNEP/MAP region are reported in the National Budget (NBB) assessing the current trends of the pollutants emissions and releases (2003-2008) including data from all CPs apart from two (UNEP/MAP, 2012c). The parameters most reported were  Chromium (liq) , Arsenic (gas), Chromium (g), Lead (gas), Mercury (gas), PAH (gas) and Copper (gas).

Table 3.6	Parameters relevant to MSFD indicator 5.1.1-Contaminants in water: Frequency of reporting to the RSCs by the CPs. Chemical contaminants used for MSFD reporting by MS are also shown.

| MSFD reporting* | WFD Priority Substances and certain other pollutants (2008/105/EC), WFD Priority Substances Amendment (2013/39/EC) | OSPAR | HELCOM | UNEP/ MAP | BSC |
| --- | --- | --- | --- | --- | --- |
| Alachlor | Alachlor |  |  |  |  |
| Anthracene | Anthracene |  |  |  |  |
| Atrazine | Atrazine |  |  |  |  |
| Benzene | Benzene |  |  |  |  |
| Brominated diphenylethers (BDE) | Brominated diphenylethers |  |  |  |  |
| Cd | Cadmium and its compounds |  |  |  |  |
| Chloroalkanes C10-13 | Chloroalkanes, C 10-13 |  |  |  |  |
| Chlorfenvinphos | Chlorfenvinphos |  |  |  |  |
| Chlorpyrifos | Chlorpyrifos (Chlorpyrifos-ethyl) |  |  |  |  |
| 1,2-dichloroethane (DCE) | 1,2-dichloroethane |  |  |  |  |
| Dichloromethane | Dichloromethane |  |  |  |  |
| Di(2-ethylhexyl)phthalate (DEHP) | Di(2-ethylhexyl)phthalate (DEHP) |  |  |  |  |
| Diuron | Diuron |  |  |  |  |
| Endosulfan | Endosulfan |  |  |  |  |
| Fluoranthene | Fluoranthene |  |  |  |  |
| Hexachlorobenzene (HCB) | Hexachlorobenzene |  |  |  |  |
| Hexachlorobutadiene (HCBD) | Hexachlorobutadiene |  |  |  |  |
| Χ-Hexachlorocyclohexane (lindane), a-Hexachlorocyclohexane | Hexachlorocyclohexane |  |  |  |  |
| Isoproturon | Isoproturon |  |  |  |  |
| Pb | Lead and its compounds |  |  |  |  |
| Hg | Mercury and its compounds |  |  |  |  |
| Naphthalene | Naphthalene |  |  |  |  |
| Ni | Nickel and its compounds |  |  |  |  |
| Nonylphenols | Nonylphenols |  |  |  |  |
| Octylphenols | Octylphenols ( 6 ) |  |  |  |  |
| Pentachlorobenzene | Pentachlorobenzene |  |  |  |  |
| Pentachlorophenol | Pentachlorophenol |  |  |  |  |
| Polycyclic Aromatic Hydrocarbons (PAH), Benzo(a)pyrene, Indeno[1,2,3-cd]pyrene, Benzo[b]fluoranthen, Benzo[g,h,i]perylene, Benzo[k]fluoranthene | Polyaromatic hydrocarbons (PAH) ( 7 ) |  |  |  |  |
| Simazine | Simazine |  |  |  |  |
| Tributylin compounds (TBT) | Tributyltin compounds |  |  |  |  |
| Trichlorobenzenes | Trichlorobenzenes |  |  |  |  |
| Trichloromethane | Trichloromethane (chloroform) |  |  |  |  |
|  | Trifluralin |  |  |  |  |
| Carbon-tetrachloride | Carbon-tetrachloride |  |  |  |  |
| DDT | DDT total |  |  |  |  |
| pp'-DDT | para-para-DDT |  |  |  |  |
| Aldrin | Aldrin |  |  |  |  |
| Dieldrin | Dieldrin |  |  |  |  |
| Endrin | Endrin |  |  |  |  |
|  | Isodrin |  |  |  |  |
| Tetrachloroethylene | Tetrachloro-ethylene |  |  |  |  |
| Trichloroethylene | Trichloro-ethylene |  |  |  |  |
|  | Dicofol** |  |  |  |  |
| Perfluorooctane sulfonic acid (PFO) | Perfluorooctane sulfonic acid and its derivatives (PFOS) |  |  |  |  |
|  | Quinoxyfen |  |  |  |  |
| PCB156, PCB105, PCB118 | Dioxins and dioxin-like compounds |  |  |  |  |
|  | Aclonifen |  |  |  |  |
|  | Bifenox |  |  |  |  |
|  | Cybutryne |  |  |  |  |
|  | Cypermethrin ( 10 ) |  |  |  |  |
|  | Dichlorvos |  |  |  |  |
| Hexabromocyclododecanes | Hexabromocyclododecanes (HBCDD) |  |  |  |  |
| Heptachlor epoxide | Heptachlor and heptachlor epoxide |  |  |  |  |
| Terbutryn | Terbutryn |  |  |  |  |
| Polychlorinated biphenyl (PCB) |  |  |  |  |  |
| Cu |  |  |  |  |  |
| pp'-DDE |  |  |  |  |  |
| Zn |  |  |  |  |  |
| pp'-DDD |  |  |  |  |  |
| Cr |  |  |  |  |  |
| PCB52 |  |  |  |  |  |
| PCB138 |  |  |  |  |  |
| PCB153 |  |  |  |  |  |
| PCB101 |  |  |  |  |  |
| Benzo[a]anthracene |  |  |  |  |  |
| PCB28 |  |  |  |  |  |
| PCB180 |  |  |  |  |  |
| Chrysene |  |  |  |  |  |
| β-Hexachlorocyclohexane |  |  |  |  |  |
| As |  |  |  |  |  |
| Heavy metals |  |  |  |  |  |
| Fluorene |  |  |  |  |  |
| Drins |  |  |  |  |  |
| Petroleum hydrocarbons |  |  |  |  |  |
| PCB31 |  |  |  |  |  |
| Chlordane |  |  |  |  |  |
| Fe |  |  |  |  |  |
| Li |  |  |  |  |  |
| Organochlorine compounds |  |  |  |  |  |
| Organochlorine pesticides |  |  |  |  |  |
| Paraffins |  |  |  |  |  |
| Co |  |  |  |  |  |
| Ethylbenzene |  |  |  |  |  |
| Xylene |  |  |  |  |  |

*Palialexis et al. 2014

| Frequency | 100-81% | 80-61% | 60-41% | 40-21% | 20-0% |
| --- | --- | --- | --- | --- | --- |
| For groups of substances, the highest frequency of individual substance is shown |  |  |  |  |  |

Table 3.7	Parameters relevant to MSFD indicator 5.1.1-Contaminants in sediment: Frequency of reporting to the RSCs by the CPs. Chemical contaminants used for MSFD reporting by MS are also shown.

| MSFD reporting* | WFD Priority Substances and certain other pollutants (2008/105/EC) WFD Priority Substances Amendment (2013/39/EC) | OSPAR | HELCOM | UNEP/MAP | BSC |
| --- | --- | --- | --- | --- | --- |
| Alachlor | Alachlor |  |  |  |  |
| Anthracene | Anthracene |  |  |  |  |
| Atrazine | Atrazine |  |  |  |  |
| Benzene | Benzene |  |  |  |  |
| Brominated diphenylethers (BDE) | Brominated diphenylethers |  |  |  |  |
| Cd | Cadmium and its compounds |  |  |  |  |
|  | Chloroalkanes, C 10-13 |  |  |  |  |
| Chlorfenvinphos | Chlorfenvinphos |  |  |  |  |
| Chlorpyrifos | Chlorpyrifos (Chlorpyrifos-ethyl) |  |  |  |  |
| 1,2-dichloroethane (DCE) | 1,2-dichloroethane |  |  |  |  |
| Dichloromethane* | Dichloromethane |  |  |  |  |
|  | Di(2-ethylhexyl)phthalate (DEHP) |  |  |  |  |
| Diuron | Diuron |  |  |  |  |
| Endosulfan | Endosulfan |  |  |  |  |
| Fluoranthene | Fluoranthene |  |  |  |  |
| Hexachlorobenzene (HCB) | Hexachlorobenzene |  |  |  |  |
|  | Hexachlorobutadiene |  |  |  |  |
| Χ-Hexachlorocyclohexane (lindane) a-Hexachlorocyclohexane | Hexachlorocyclohexane |  |  |  |  |
| Isoproturon | Isoproturon |  |  |  |  |
| Pb | Lead and its compounds |  |  |  |  |
| Hg | Mercury and its compounds |  |  |  |  |
| Naphthalene | Naphthalene |  |  |  |  |
| Ni | Nickel and its compounds |  |  |  |  |
| Nonylphenols | Nonylphenols |  |  |  |  |
| Octylphenols | Octylphenols ( 6 ) |  |  |  |  |
| Pentachlorobenzene | Pentachlorobenzene |  |  |  |  |
|  | Pentachlorophenol |  |  |  |  |
| Polycyclic Aromatic Hydrocarbons (PAH), Benzo(a)pyrene, Indeno[1,2,3-cd]pyrene, Benzo[b]fluoranthene Benzo[g,h,i]perylene Benzo[k]fluoranthene | Polyaromatic hydrocarbons (PAH) ( 7 ) |  |  |  |  |
| Simazine | Simazine |  |  |  |  |
| Tributylin compounds (TBT) | Tributyltin compounds |  |  |  |  |
|  | Trichlorobenzenes |  |  |  |  |
|  | Trichloromethane (chloroform) |  |  |  |  |
|  | Trifluralin |  |  |  |  |
|  | Dicofol** |  |  |  |  |
| Perfluorooctane sulfonic acid (PFO) | Perfluorooctane sulfonic acid and its derivatives (PFOS) |  |  |  |  |
|  | Quinoxyfen |  |  |  |  |
| Dioxins and dioxin-like PCBs, PCB156, PCB105, PCB118, Furans | Dioxins and dioxin-like compounds |  |  |  |  |
|  | Aclonifen |  |  |  |  |
|  | Bifenox |  |  |  |  |
|  | Cybutryne |  |  |  |  |
|  | Cypermethrin ( 10 ) |  |  |  |  |
|  | Dichlorvos |  |  |  |  |
| Hexabromocyclododecanes | Hexabromocyclododecanes (HBCDD) |  |  |  |  |
| Heptachlor epoxide | Heptachlor and heptachlor epoxide |  |  |  |  |
| Terbutryn | Terbutryn |  |  |  |  |
|  | Carbon-tetrachloride |  |  |  |  |
| DDT | DDT total |  |  |  |  |
| pp'-DDT | para-para-DDT |  |  |  |  |
| Aldrin | Aldrin |  |  |  |  |
| Dieldrin | Dieldrin |  |  |  |  |
| Endrin | Endrin |  |  |  |  |
|  | Isodrin |  |  |  |  |
|  | Tetrachloro-ethylene |  |  |  |  |
|  | Trichloro-ethylene |  |  |  |  |
| Polychlorinated biphenyl (PCB) |  |  |  |  |  |
| Cu |  |  |  |  |  |
| pp'-DDE |  |  |  |  |  |
| Zn |  |  |  |  |  |
| pp'-DDD |  |  |  |  |  |
| Cr |  |  |  |  |  |
| PCB52 |  |  |  |  |  |
| PCB138 |  |  |  |  |  |
| PCB153 |  |  |  |  |  |
| PCB101 |  |  |  |  |  |
| Benzo[a]anthracene |  |  |  |  |  |
| PCB28 |  |  |  |  |  |
| PCB180 |  |  |  |  |  |
| Chrysene |  |  |  |  |  |
| β-Hexachlorocyclohexane 3 2 3 |  |  |  |  |  |
| As |  |  |  |  |  |
| Phenanthrene - 4 2 |  |  |  |  |  |
| Pyrene |  |  |  |  |  |
| Fluorene |  |  |  |  |  |
| Drins |  |  |  |  |  |
| Petroleum hydrocarbons |  |  |  |  |  |
| PCB31 |  |  |  |  |  |
| Chlordane |  |  |  |  |  |
| Fe |  |  |  |  |  |
| Li |  |  |  |  |  |
| Nonachlor |  |  |  |  |  |
| Al |  |  |  |  |  |
| Mn |  |  |  |  |  |
| Acenaphthylene |  |  |  |  |  |
| Dibezo(a,h)anthracene |  |  |  |  |  |
| Benzo[e]perylene |  |  |  |  |  |
| Bisphenol A |  |  |  |  |  |
| Naphthalene/Pyrene |  |  |  |  |  |
| PCB128 |  |  |  |  |  |

*Palialexis et al. 2014

| Frequency | 100-81% | 80-61% | 60-41% | 40-21% | 20-0% |
| --- | --- | --- | --- | --- | --- |
| For groups of substances, the highest frequency of individual substance is shown |  |  |  |  |  |

Table 3.8	Parameters relevant to MSFD indicator 5.1.1-Contaminants in biota: Frequency of reporting to the RSCs by the CPs. Chemical contaminants used for MSFD reporting by MS are also shown.

| MSFD reporting* | WFD Priority Substances and certain other pollutants (2008/105/EC) WFD Priority Substances Amendment (2013/39/EC) | OSPAR | HELCOM | UNEP/MAP | BSC |
| --- | --- | --- | --- | --- | --- |
|  | Alachlor |  |  |  |  |
| Anthracene | Anthracene |  |  |  |  |
|  | Atrazine |  |  |  |  |
|  | Benzene |  |  |  |  |
| Brominated diphenylethers (BDE) | Brominated diphenylethers |  |  |  |  |
| Cd | Cadmium and its compounds |  |  |  |  |
|  | Chloroalkanes, C 10-13 |  |  |  |  |
|  | Chlorfenvinphos |  |  |  |  |
|  | Chlorpyrifos (Chlorpyrifos-ethyl) |  |  |  |  |
|  | 1,2-dichloroethane |  |  |  |  |
|  | Dichloromethane |  |  |  |  |
| Di(2-ethylhexyl)phthalate (DEHP) | Di(2-ethylhexyl)phthalate (DEHP) |  |  |  |  |
|  | Diuron |  |  |  |  |
| Endosulfan | Endosulfan |  |  |  |  |
| Fluoranthene | Fluoranthene |  |  |  |  |
| Hexachlorobenzene (HCB) | Hexachlorobenzene |  |  |  |  |
|  | Hexachlorobutadiene |  |  |  |  |
| Χ-Hexachlorocyclohexane (lindane) a-Hexachlorocyclohexane | Hexachlorocyclohexane |  |  |  |  |
|  | Isoproturon |  |  |  |  |
| Pb | Lead and its compounds |  |  |  |  |
| Hg, Methyl-Hg | Mercury and its compounds |  |  |  |  |
| Naphthalene | Naphthalene |  |  |  |  |
| Ni | Nickel and its compounds |  |  |  |  |
| Nonylphenols | Nonylphenols |  |  |  |  |
| Octylphenols | Octylphenols ( 6 ) |  |  |  |  |
|  | Pentachlorobenzene |  |  |  |  |
|  | Pentachlorophenol |  |  |  |  |
| Polycyclic Aromatic Hydrocarbons (PAH), Benzo(a)pyrene, Indeno[1,2,3-cd]pyrene, Benzo[b]fluoranthene,Benzo[g,h,i]perylene, Benzo[k]fluoranthene | Polyaromatic hydrocarbons (PAH) ( 7 ) |  |  |  |  |
|  | Simazine |  |  |  |  |
| Tributylin compounds (TBT) | Tributyltin compounds |  |  |  |  |
|  | Trichlorobenzenes |  |  |  |  |
| Trichloromethane | Trichloromethane (chloroform) |  |  |  |  |
|  | Trifluralin |  |  |  |  |
|  | Dicofol |  |  |  |  |
| Perfluorooctane sulfonic acid (PFO) | Perfluorooctane sulfonic acid and its derivatives (PFOS) |  |  |  |  |
|  | Quinoxyfen |  |  |  |  |
| Dioxins and dioxin-like PCBs, Furans, PCB169, PCB189, PCB77, PCB156, PCB105, PCB118 | Dioxins and dioxin-like compounds |  |  |  |  |
|  | Aclonifen |  |  |  |  |
|  | Bifenox |  |  |  |  |
|  | Cybutryne |  |  |  |  |
|  | Cypermethrin ( 10 ) |  |  |  |  |
|  | Dichlorvos |  |  |  |  |
| Hexabromocyclododecanes | Hexabromocyclododecanes (HBCDD) |  |  |  |  |
| Heptachlor epoxide | Heptachlor and heptachlor epoxide |  |  |  |  |
|  | Terbutryn |  |  |  |  |
|  | Carbon-tetrachloride |  |  |  |  |
| DDT | DDT total |  |  |  |  |
| pp'-DDT | para-para-DDT |  |  |  |  |
| Aldrin | Aldrin |  |  |  |  |
| Dieldrin | Dieldrin |  |  |  |  |
| Endrin | Endrin |  |  |  |  |
| Isodrin | Isodrin |  |  |  |  |
|  | Tetrachloro-ethylene |  |  |  |  |
|  | Trichloro-ethylene |  |  |  |  |
| Polychlorinated biphenyl (PCB) |  |  |  |  |  |
| Cu |  |  |  |  |  |
| pp'-DDE |  |  |  |  |  |
| Zn |  |  |  |  |  |
| pp'-DDD |  |  |  |  |  |
| Cr |  |  |  |  |  |
| PCB52 |  |  |  |  |  |
| PCB138 |  |  |  |  |  |
| PCB153 |  |  |  |  |  |
| PCB101 |  |  |  |  |  |
| Benzo[a]anthracene |  |  |  |  |  |
| PCB28 |  |  |  |  |  |
| PCB180 |  |  |  |  |  |
| Chrysene |  |  |  |  |  |
| β-Hexachlorocyclohexane 3 2 3 |  |  |  |  |  |
| As |  |  |  |  |  |
| Phenanthrene - 4 2 |  |  |  |  |  |
| Pyrene |  |  |  |  |  |
| Heavy metals |  |  |  |  |  |
| Fluorene |  |  |  |  |  |
| Drins |  |  |  |  |  |
| PCB31 |  |  |  |  |  |
| Chlordane |  |  |  |  |  |
| Fe |  |  |  |  |  |
| Nonachlor |  |  |  |  |  |
| Mn |  |  |  |  |  |
| Organochlorine pesticides |  |  |  |  |  |
| Paraffins |  |  |  |  |  |
| PCB128 |  |  |  |  |  |

*Palialexis et al. 2014

| Frequency | 100-81% | 80-61% | 60-41% | 40-21% | 20-0% |
| --- | --- | --- | --- | --- | --- |
| For groups of substances, the highest frequency of individual substance is shown |  |  |  |  |  |

Table 3.9.	Parameters relevant to MSFD indicator 5.1.1-Contaminants in water, grouped in chemical groups used by ICES: Frequency of reporting to the RSCs by the CPs

| ICES chemical groups | OSPAR | HELCOM | UNEP/MAP | BSC |
| --- | --- | --- | --- | --- |
| Chlorobiphenyls |  | x |  | x |
| Cyclodienes | x | x |  | x |
| Dichloro-diphenyl-trichloroethetane DDTs | x | x |  | x |
| Hexachlorocyclohexanes | x | x |  | x |
| Metals and metalloids | x | x |  | x |
| Monocyclic aromatic hydrocarbons |  | x |  |  |
| Organic esters |  | x |  |  |
| Organobromines | x | x |  | x |
| Organochlorines (general) | x | x |  |  |
| Organo-metallic compounds | x | x |  |  |
| Organophosphorus pesticids |  | x |  |  |
| Pesticides (general) |  | x |  |  |
| Polycyclic aromatic hydrocarbons (PAHs) | x | x |  | x |
| Triazine pesticides | x |  |  |  |

| Frequency | 100-81% | 80-61% | 60-41% | 40-21% | 20-0% |
| --- | --- | --- | --- | --- | --- |
| For groups of substances, the highest frequency of individual substance is shown |  |  |  |  |  |

Table 3.10.	Parameters relevant to MSFD indicator 5.1.1-Contaminants in sediment, grouped in chemical groups used by ICES: Frequency of reporting to the RSCs by the CPs

| ICES chemical groups | OSPAR | HELCOM | UNEP/MAP | BSC |
| --- | --- | --- | --- | --- |
| Chlorobiphenyls | x | x |  | x |
| Cyclodienes | x | x |  | x |
| Dichloro-diphenyl-trichloroethetane DDTs | x | x |  | x |
| Dioxins | x | x |  |  |
| Hexachlorocyclohexanes | x | x |  | x |
| Metals and metalloids | x | x | x | x |
| Organic esters | x | x |  |  |
| Organobromines | x | x |  |  |
| Organochlorines (general) | x | x |  | x |
| Organofluorines | x | x |  |  |
| Organo-metallic compounds | x | x |  |  |
| Pesticides (general) |  | x |  |  |
| Polycyclic aromatic hydrocarbons (PAHs) | x | x | x | x |

| Frequency | 100-81% | 80-61% | 60-41% | 40-21% | 20-0% |
| --- | --- | --- | --- | --- | --- |
| For groups of substances, the highest frequency of individual substance is shown |  |  |  |  |  |

Table 3.11.	Parameters relevant to MSFD indicator 5.1.1-Contaminants in biota, grouped in chemical groups used by ICES: Frequency of reporting to the RSCs by the CPs

| ICES chemical groups | OSPAR | HELCOM | UNEP/MAP | BSC |
| --- | --- | --- | --- | --- |
| Chlorinated paraffins |  | x |  |  |
| Chlorobiphenyls | x | x | x | x |
| Cyclodienes | x | x |  | x |
| Dichloro-diphenyl-trichloroethetane DDTs | x | x |  | x |
| Dioxins |  | x |  |  |
| Hexachlorocyclohexanes | x | x | x | x |
| Metals and metalloids | x | x | x | x |
| Organobromines | x | x |  |  |
| Organochlorines (general) | x | x | x | x |
| Organofluorines | x | x |  | x |
| Organo-metallic compounds | x | x |  |  |
| Pesticides (general) |  |  |  |  |
| Polycyclic aromatic hydrocarbons (PAHs) | x | x | x | x |

| Frequency | 100-81% | 80-61% | 60-41% | 40-21% | 20-0% |
| --- | --- | --- | --- | --- | --- |
| For groups of substances, the highest frequency of individual substance is shown |  |  |  |  |  |

## Categorization of indicators according to the DPSIR approach

The Table 3.12 below is an initial effort to associate the RSCs hazardous substances indicators with driver, pressure, state or impact in order to address the DPSIR approach. The type of indicators (P, S, I) are according to SEC (2011).  All RSCs cover pressure and impact indicators. OSPAR and BSC apply indicators on hazardous substances loads providing information on drivers. HELCOM collects data on inputs and produce assessments of pollution loads (see Review of the Fifth Baltic Sea Pollution Load Compilation for the 2013 HELCOM Ministerial Meeting http://www.helcom.fi/helcom-at-work/projects/plc-5-5/ MEDPOL; Releases, emissions and sources of pollutants in the Mediterranean region. An assessment of 2003-2008 trends; 2012) but has not developed a relevant core indicator. UNEP/MAP is currently developing an indicator on “Release of toxic  substances and nutrients from industrial sector’ representing the emissions from industrial sources from individual facilities within the Mediterranean coastal zone with regard to nutrients and oxygen depleting substances, halogenated hydrocarbons, hydrocarbons and heavy metals.

Table 3.12	RSCs indicators type, D: Driver P: Pressure, S: State. I: Impact.

| Type | OSPAR | OSPAR | HELCOM | HELCOM | UNEP/MAP | BSC |
| --- | --- | --- | --- | --- | --- | --- |
|  | Common indicators | Candidate indicators | Core indicators | Pre-core indicators | Proposed indicators | Indicators in Diagnostic Report, 2010 |
| D/P | Inputs of Hg, Cd and Pb via water and air |  |  |  |  | Loads: Inputs of nutrients and HSs from direct (point) sources |
| I | Metal (Hg, Cd, Pb) concentrations in biota |  | Metals (lead, cadmium and mercury) |  | 9.1.1 Concentration of key harmful contaminants in biota, sediment or water | Hazardous substances in biota |
| I | Metal (Hg, Cd, Pb) concentrations in sediment |  |  |  |  | Hazardous substances in sediments |
| I | PCB concentrations in biota |  | Polychlorinated biphenyls (PCB) and dioxins and furans: CB-28, 52, 101, 118, 138, 153and 180; WHO-TEQ of dioxins, furans +dl-PCBs |  |  |  |
| I | PCB concentrations in sediments |  |  |  |  |  |
| I | PAHs concentrations in biota* |  | Polyaromatic hydrocarbons and their metabolites: US EPA 16 PAHs / selected metabolites. |  |  |  |
| I | PAHs concentrations in sediments |  |  |  |  |  |
| I |  | Organotin concentrations in biota | Tributyltin (TBT) and imposex |  |  |  |
| I | Organotin concentrations in sediments |  |  |  |  |  |
| I | PBDE concentrations in biota |  | Polybrominated biphenyl ethers (PBDE): BDE-28, 47, 99, 100, 153 and 154 |  |  |  |
| I | PBDE concentrations in sediments |  |  |  |  |  |
| I |  | HCB (hexachlorobenzene) concentrations in biota |  |  |  |  |
| I |  | HCB (hexachlorobenzene) concentrations in sediments |  |  |  |  |
| I |  | HCBD (hexachlorobutadiene) concentrations in biota |  |  |  |  |
| I |  | HCBD (hexachlorobutadiene) concentrations in sediments |  |  |  |  |
| I |  |  | Hexabromocyclododacene (HBCD) |  |  |  |
| I |  |  | Perfluorooctane sulphonate (PFOS) |  |  |  |
| I |  |  |  | Pharmaceuticals: Diclofenac, EE2 (+E1, E2, E3 + in vitro yeast essay |  |  |
| I |  |  | Radioactive substances: Caesium-137 in fi sh and surface waters |  |  |  |
| I | Imposex/intersex |  | Tributyltin (TBT) and imposex |  | 9.2.1 Level of pollution effects of key contaminants where a cause and effect relationship has been established |  |
| I |  | Externally visible fish diseases |  | Fish diseases – a fish stress indicator |  |  |
| I |  | Lysosomal stability (LMS) |  | Lysosomal Membrane Stability – a toxic stress indicator |  |  |
| I |  | Bile metabolites (of PAHs) | Polyaromatic hydrocarbons and their metabolites: US EPA 16 PAHs / selected metabolites. |  |  |  |
| I |  | Micronuclei (MN) |  | Micronuclei test – a genotoxicity indicator |  |  |
| I |  | EROD |  |  |  |  |
| I |  |  |  | Eelpout and amphipod embryo malformations |  |  |
| P/I |  | Oiled birds |  | Number of waterbirds being oiled annually (biodiversity core indicator) | 9.3.1 Occurrence, origin (where possible), extent of significant acute pollution events (e.g. slicks from oil, oil products and hazardous substances) and their impact on biota affected by this pollution | Illegal discharges of oil at sea Accidental oil spills from shipping Shipping density |
| P/I |  |  | HELCOM core indicators to assess concentrations against specific limit levels; substances are Cadmium, Lead, Mercury, dl-PCBs, dioxins, Benzo[a]Pyrene and Cesium-137 |  | 9.4.1 Actual levels of contaminants that have been detected and number of contaminants which have exceeded maximum regulatory levels in commonly consumed seafood |  |
| P/I |  |  |  |  | 9.4.1 Actual levels of contaminants that have been detected and number of contaminants which have exceeded maximum regulatory levels in commonly consumed seafood |  |
| I |  |  |  |  | 9.5.1 Percentage of intestinal enterococci concentration measurements within established standards |  |

# Summary

All RSCs have adopted (HELCOM, OSPAR) or are in the process of adopting (UNEP/MAP, BSC) key indicators for eutrophication and hazardous substances that cover sufficiently the MSFD descriptors of eutrophication and contaminants.

The eutrophication and hazardous substances indicators and relevant methodologies for OSPAR and HELCOM are well defined. Guidelines exist for monitoring and assessment for most of the indicators including assessment criteria and for those that are not covered by existing guidelines (for example some of the HELCOM hazardous substances core indicators) guidelines are currently being developed and will soon be available. Tools for assessments of eutrophication and hazardous substances at regional scale are also available for HELCOM and OSPAR.  UNEP/MAP and BSC indicators for eutrophication and hazardous substances are currently under development. Although guidelines for monitoring and analytical methodologies on eutrophication and hazardous substances parameters are available, the indicator methodologies need further development. UNEP/MAP is in the process of developing assessment criteria and threshold levels for the proposed indicators of GES.

An analysis of data on eutrophication and hazardous substances relevant to MSFD indicators that are actually reported to the RSCs by the Contracting Parties and at what frequencies across RSCs was performed. Existing data flows of the RSCs relevant to eutrophication and hazardous substances were analysed using data reported in 2012 by the RSCs CPs that are EU MS.

Among the data flows relevant to eutrophication, nutrients, chlorophyl a and oxygen data were reported by most contracting parties across RSCs. These parameters present high frequency of use by MS reporting for the implementation of Art 8, 9 and 10 of the MSFD and are also used in the WFD.  EEA uses nutrients and chlorophyl data for the two eutrophication indicators CSI021 and CSI023. The percentage reporting on eutrophication was low for UNEP/MAP CPs in 2012. Nutrients, chlorophyl a and oxygen data are reported through EIONET although reporting is not identical with RSC reporting in terms of number of parameters or number of stations reported.

All HELCOM CPs reported on nutrient input parameters in 2012 with the exception of one CP as regard inputs from rivers and point sources. Most OSPAR CPs reported on water borne nutrient input parameters and atmospheric nutrient inputs in 2011. Nutrient inputs from municipal, industrial and riverine sources were reported to BSC by both CPs during the period 2001-2008. Nutrient inputs in the UNEP/MAP region were reported in the National Budget (NBB) assessing the current trends of the pollutants emissions and releases (2003-2008).

Data flows on hazardous substances show inconsistencies among CPs in each RSC as regards the substances covered. The data reported to OSPAR and HELCOM overall cover a good number of the WFD priority substances but the percentage of countries reporting for each substance is low particularly in water and sediment data (less than 40%) and only for 11 substances in biota ranges between 40 and 80%. As regards UNEP/MAP and BSC, the reported data cover few of the WFD priority substances. The percentage of countries reporting to UNEP/MAP is low, whereas percentage of countries reporting to BSC is 50% since one of the two countries included in the analysis is reporting data on hazardous substances. All the hazardous substances reported to the RSCs were used by MS in reporting for the implementation of Art 8, 9 and 10 of the MSFD. Hazardous substances are reported though Eionet although reporting is different than RSC reporting (i.e. individual substances reported and number of stations are different).

Biological effects parameters used by the MS for reporting under articles 8, 9 and 10 of the MSFD were reported by three of the OSPAR CPs and two of the HELCOM CPs. These include imposex in gastropods, fish liver pathologies, EROD and levels of bile metabolite in fish. Biological effect parameters were not reported to UNEP/MAP and BSC,

High percentages of the OSPAR and HELCOM contracting parties reported on riverine inputs and direct inputs (point sources) of contaminants. Metal and TP inputs from municipal, industrial and riverine sources were reported to BSC by both CPs during the period 2001-2008. Pollutant inputs in the UNEP/MAP region were reported in the National Budget (NBB) assessing the current trends of the pollutants emissions and releases (2003-2008).

# Following steps to complete the inventory

The project Tasks 1 and 2 are to include an inventory all substances of relevance for the MSFD reporting. The final version of D2 will take into account the response and comments to this draft report from the TG-Data meeting and it will be extended to include the complete inventory.

Work and required activities to complete the inventory:

Collection of data/information of relevance to all MSFD descriptors

Updating the inventory based on the comments and needs raised in the TG-Data meeting

# References

Andersen JH, Axe P, Backer H, Carstensen J, Claussen U et al. 2011. Getting the measure of eutrophication in the Baltic Sea: towards improved assessment principles and methods. Biogeochemistry 106:137-156.

Boalt, E., Nyberg, E., Bignert, A., Hedman, J., Danielson, S., CORESET, (2013).Polychlorinated biphenyls (PCB) and dioxins and furans - CB-28, 52, 101, 118, 138, 153 and 180: WHO-TEQ of dioxins, furans –dl-PCBs. HELCOM Core Indicator Report. Online.[4Apr.2014],[http://www.helcom.fi/Core%20Indicators/HELCOMCoreIndicator_Polychlorinated_biphenyls_and_dioxins_and_furans.pdf].

BSC 2010. ‘Diagnostic Report’ to guide improvements to the regular reporting process on the state of the Black Sea environment, Publications of the Commissions on the Protection of the Black Sea Against Pollution (BSC) 2010. http://www.blacksea-commission.org/_publ-BSDiagnosticReport2010.asp

BSC SoE 2008.State of the Environment of the Black Sea (2001-2006/7).Edited by TernelOguz. Publications of the Commissions on the Protection of the Black Sea Against Pollution (BSC) 2008-3, Istanbul, Turkey, 448 pp.

BSSAP 2009. Strategic Action Plan for the Environmental Protection and Rehabilitation of the Black Sea, Sofia, Bulgaria, 17 April 2009. 61p

HELCOM 2007.Baltic Sea Action Plan. Available at: http://www.helcom.fi/baltic-sea-action-plan/

HELCOM 2009.Eutrophication in the Baltic Sea.An integrated thematic assessment of the effects of nutrient enrichment in the Baltic Sea region.Baltic Sea Environment Proceedings 115B.148 pp.

HELCOM 2010a.Ecosystem Health of the Baltic Sea - HELCOM Initial Holistic Assessment.Baltic Sea Environment Proceedings 122.63 pp.

HELCOM 2010b. What was the eutrophication status of the Baltic Sea in 2003- 2007? Demonstration set of HELCOM core eutrophication indicators. Published in May 2010. http://www.helcom.fi/BSAP_assessment/eutro/HEAT/en_GB/ status/.

HELCOM 2010c.Hazardous substances in the Baltic Sea – An integrated thematic assessment of hazardous substances in the Baltic Sea.Balt. Sea Environ. Proc. No. 120B

HELCOM, 2011. The Fifth Baltic Sea Pollution Load Compilation (PLC-5)Balt. Sea Environ. Proc. No. 128 http://www.helcom.fi/Lists/Publications/BSEP128.pdf

HELCOM, 2013a. HELCOM core indicators: Final report of the HELCOM CORESET project. Balt. Sea Environ. Proc. No. 136

HELCOM 2013b. Approaches and methods for eutrophication target setting in the Baltic Sea region. Baltic Sea Environment Proceedings 133.134 pp.

HELCOM, 2014. Eutrophication status of the Baltic Sea 2007-2011 - A concise thematic assessment.Baltic Sea Environment Proceedings No. 143Moncheva S. & Parr B. 2010.Manual for Phytoplankton Sampling and Analysis in the Black Sea. 68pp.

Herrmann, J., Kanisch, G., Heinrich, J., 2013. Radioactive substances - Caesium-137 in fish and surface waters.HELCOM Core Indicator Report.Online.[4Apr.2014], [http://www.helcom.fi/Core%20Indicators/HELCOM_CoreIndicator_Radioactive_substances.pdf].

LBSA 2009. Protocol on the Protection of the Marine Environment of the Black Sea from Land-Based Sources and Activities

Moncheva S. 2010.Guidelines for quality control of biological data phytoplankton. 18pp.

Moncheva, S. and Par, B. 2005 (updated-2010). Manual for Phytoplankton Sampling and Analysis in the Black Sea, GEF/UNDP Black Sea Ecosystem Recovery Project (BSERP)-RER/01/G33/A/1G/31, EC, FP7, Upgrade Black Sea Scene Project, Istanbul, 67 pp.

MRAG/UNEP-WCMC/URS 2013 Final Report for activities in 2013 to European Commission for contract on Development of WISE-Marine for the Marine Strategy Framework Directive 2008/56/EC (070307/2011/610594/ENV.D.2/SER).

Nyberg, E., Larsen, M.M.,  Bignert, A., Boalt, E., Danielson S. and the CORESET expert group for  hazardous substances indicators. 2013a. Metals (lead, cadmium and mercury). HELCOM Core Indicator Report.Online. 23.3.14, [http://www.helcom.fi/Core%20Indicators/HELCOM-CoreIndicator-Metals.pdf]

Nyberg, E.,  Kammann, U., Garnaga, G., Bignert, A., Schneider, R., Danielsson S., and the CORESET expert group for hazardous substances indicators. 2013b. Core Indicator Report.  Online.23.3.14http://www.helcom.fi/Core%20Indicators/HELCOM_CoreIndicator_Polyaromatic_hydrocarbons_and_their_metabolites.pdf

Nyberg, E., Poikane, R., Strand, J., Larsen, M.M., Danielsson, S., Bignert, A., CORESET, 2013c. Tributyltin (TBT) and imposex.HELCOM Core Indicator Report.Online.[4Apr.2014], [http://www.helcom.fi/Core%20Indicators/HELCOM_CoreIndicator_Tributyltin_and_imposex.pdf].

Nyberg, E., Bignert, A., Danielsson, S., Mannio, J., CORESET, 2013d.Hexabromocyclododecane.HELCOM Core Indicator Report. Online.[4Apr.2014], [http://www.helcom.fi/Core%20Indicators/HELCOMCoreIndicator_Hexabromocyclododecane.pdf]

Nyberg, E., Bignert, A., Danielsson, S., CORESET, 2013e.Perfluorooctane sulphonate (PFOS).HELCOM Core Indicator Report. Online.[4Apr.2014], [http://www.helcom.fi/Core%20Indicators/HELCOM CoreIndicator_Perfluorooctane_sulphonate.pdf]

Nyberg, E., Bignert, A., Danielsson, S., Mannio, J., HELCOM CORESET, 2013f.PolybrominatedDiphenyl Ethers (PBDE) - BDE-28, 47, 99, 100, 153 and 154.HELCOM Core Indicator Report.Online.[4Apr.2014], [http://www.helcom.fi/Core%20Indicators/HELCOM-CoreIndicator_Polybrominated_Diphenyl_Ethers.pdf]

OSPAR, 2003. OSPAR Commission, 2003. OSPAR integrated report 2003 on the eutrophication status of the OSPAR maritime area based upon the first application of the Comprehensive Procedure. OSPAR Eutrophication Series, publication 189/2003. OSPAR Commission, London

OSPAR 2005.Common Procedure for the Identification of the Eutrophication Status of the OSPAR Maritime Area , Ref no: 2005-3

OSPAR 2008. OSPAR Commission 2008.Second Integrated Report on the Eutrophication Status of the OSPAR Maritime AreaRef no: 372/2008

OSPAR 2009a. Evaluation of the OSPAR system of Ecological Quality Objectives for the North Sea(update 2010),publication number 406/2009

OSPAR 2009b.Background Document on CEMP assessment criteria for the QSR 2010. Ref no: 461/2009

OSPAR, 2012. MSFD Advice Manual and Background document on Good environmental status - Descriptor 5: Eutrophication. A living document - Version 5 January 2012

OSPAR 2013. Common Procedure for the Identification of the Eutrophication Status of the OSPAR Maritime Area(Reference number: 2013-8)

Palialexis, A. V Tornero,V.  Barbone, E. Gonzalez, D. Hanke, G. Cardoso, A. C. Hoepffner, N.  Katsanevakis, S. Somma, F. s Zampoukas N. 2014. In-Depth Assessment of the EU Member States’ Submissions for the Marine Strategy Framework Directive under articles 8, 9 and 10. 149pp,  EUR – Scientific and Technical Research series – ISSN 1831-9424

SEC 2011. Commission Staff Working Paper, Relationship between the initial assessment of marine waters and the criteria for good environmental status, 1255 final, Brussels, 14.10.2011

UNEP/MAP 2005 Sampling and analysis Techniques for the Eutrophication Monitoring Strategy of MED POL MAP Technical Reports Series No. 163

UNEP/MAP, 2011a. Review of the methods, criteria and limit values for the assessment of Eutrophication (Biological Quality Element “Phytoplankton”) as developed in the framework of the Intercalibration Exercise of the MED GIG (Mediterranean Eco-region) Water Framework Directive 2000/60 EC UNEP(DEPI)/MED WG.365/Inf.7

UNEP/MAP, 2011b. Reviewing MED POL marine monitoring activities and planning for the new integrated monitoring system Consultation Meeting to Review MED POL Monitoring Activities, UNEP(DEPI)/MED WG.365/3. Athens, 22-23 November 2011.

UNEP/MAP, 2011c. Development of Assessment Criteria for Hazardous substances in the Mediterranean.Consultation Meeting to Review MED POL Monitoring Activities, UNEP(DEPI)/MED WG. 365/Inf.8Athens, 22-23 November 2011

UNEP/MAP, 2011d. Hazardous substances in the Mediterranean:a spatial and temporal assessment. Consultation Meeting to Review MED POL Monitoring Activities, UNEP(DEPI)/MED WG.365/3, Athens, 22-23 November 2011

UNEP/MAP, 2012a. Support to the Barcelona Convention for the implementation of the Ecosystem Approach, including the establishment of MPAs in open seas, including deep sea. Contribution Agreement N°21.0401/2008/519114/SUB/D2, Final Report April 2012.

UNEP/MAP 2012b. Report on Meeting of the correspondence group on Good Environmental Status (GES) and targets: Pollution and Litter cluster, UNEP(DEPI)/MED WG.372/4,  Sarajevo (Bosnia & Herzegovina), 29-30 October 2012

UNEP/MAP 2012c MEDPOL; Releases, emissions and sources of pollutants in the Mediterranean region. An assessment of 2003-2008 trends; 2012

UNEP/MAP 2014a.Secretariat  Analysis of Common Indicators. Integrated Correspondence Groups of GES and Targets Meeting Athens (Greece), 17-19 February 2014, UNEP(DEPI)/MED WG.390/Inf.3

UNEP/MAP, 2014b.Working document on Common Indicators for the Mediterranean.Integrated Correspondence Groups of GES and Targets Meeting.UNEP(DEPI)/MED WG.390/3, Athens (Greece), 17-19 February 2014.

Velikova V., Boicenco L., Beken-Polat C., Moncheva S., Levent B., Sezgin M., Begun T., Oros A. 2013. Diagnostic Report II guiding improvements in the Black Sea monitoring system. EC DG Env. MISI Project Deliverables. http://sharepoint.misisproject.eu/sharepoint/

Vollenweider, R.A., Giovanardi, F., Montanari, G. &Rinaldi, A. (1998). Characterization of the trophic conditions of marine coastal waters, with special reference to the NW Adriatic Sea: Proposal for a trophic scale, turbidity and generalized water quality index. Environmetrics, 9: 329-357.

List of abbreviations

AG LBS: Advisory Group on Control of Pollution from Land Based Sources

AG PMA: Advisory Group on Pollution Monitoring and Assessment

BAC: Background Assessment Criteria

BSIMAP: Black Sea Integrated Monitoring and Assessment Program

BSIS: Black Sea Information System

BSSAP: Strategic Action Plan for the Rehabilitation and Protection of the Black Sea

CAMP: OSPAR Comprehensive Atmospheric Monitoring Programme

CBD AG: Advisory Group on Conservation of Biodiversity

CEMP: OSPAR Co-ordinated Environmental Monitoring Programme

CHASE: The HELCOM Hazardous Substances Status Assessment Tool

COMBINE: Cooperative Monitoring in the Baltic Marine Environment

COMBINE: HELCOM Cooperative Monitoring in the Baltic Marine Environment

CP: Contracting party

DPSIR: Drivers, Pressures, State, Impact, Response

EEA: European Environmental Agency

Eionet: European environment information and observation network

EQS: Environmental Quality Standards

ETC/ICM:  European Topic Centre on  Inland, Coastal and Marine Waters

ETCs: European Topic Centres

GES: Good Environmental Status

HASEC: Hazardous Substances and Eutrophication Committee

HEAT: The HELCOM Eutrophication Assessment Tool

HELCOM MORS EG: HELCOM Monitoring of Radioactive Substances Expert Group

JAMP: OSPAR Joint Assessment and Monitoring Programme

LBSA Protocol: Protocol on the Protection of the Marine Environment of the Black Sea from Land-Based Sources and Activities

MAP: Mediterranean Action Plan

MED POL: Programme for the Assessment and Control of Marine Pollution in the Mediterranean Region

MONAS: Monitoring and assessment group

MS: Member State

MSFD: Marine Strategy Framework Directive

NFPs:  National Focal Points

NRCs: National Reference Centres

PLC 5.5: HELCOM Pollution Load Compilation 5.5

PLC 6: HELCOM Pollution Load Compilation 6

REMPEC: Regional Marine Pollution Emergency Centre

RID: OSPAR Comprehensive Study of Riverine Inputs and Direct Discharges

RSCs: Regional Sea Conventions

WFD: Water Framework Directive

WG DIKE: Working Group on Data, Information, and Knowledge Exchange

WISE SoE TCM: WISE SoE data on transitional, coastal and marine waters

###### A1    Overview and inventory of RSCs data streams on eutrophication

A2 Overview and inventory of RSCs data streams on hazardous substances

Common Procedure for the Identification of the Eutrophication Status of the OSPAR Maritime Area(Reference number: 2013-8)

MRAG/UNEP-WCMC/URS to European Commission for contract on Development of WISE-Marine for the Marine Strategy Framework Directive 2008/56/EC (070307/2011/610594/ENV.D.2/SER).

Integrated Regional monitoring Implementation Strategy in the South European Seas IRIS-SES

OSPAR, 2009: CEMP 2008/2009 Assessment of trends and concentrations of selected hazardous

substances in sediments and biota. OSPAR Publication Number: 390/2009. ISBN 978-1-906840-30-3.

Hazardous Substances in European waters - Analysis of the data on hazardous substances in groundwater, rivers, transitional, coastal and marine waters reported to the European Environment Agency from 1998 - 2010

. Co-ordinated Environmental Monitoring Programme Assessment Manual for contaminants in sediment and biota. Monitoring and Assessment Series.OSPAR 2008, London.
http://www.ospar.org/documents/dbase/publications/p00379/p00379_cemp_assessment_manual.pdf

1 Integrated Regional monitoring Implementation Strategy in the South European Seas IRIS-SES